# BI rad-tol optical-links (BRATOL) meeting 02/12/2022

Attendees

SY-BI: Christos Zanamtzas, Thibaut Lefevre, Manoel Barros Marin

EP-ESE : Sophie Baron, Jan Troska, Francois Vasey

Excused

SY-BI: Andrea Boccardi

IPT-PE: Joshua Luke Davison

Indico: https://indico.cern.ch/event/1197467/

## Minutes and actions

- SPS BLM modules (Christos, Jan):

  - the re-work of the ~300 existing SM-VTRx is now finished. A yield of 85-90% has been observed during insertion of the modules into the metal housing, mostly due to the degradation of the TOSA flexes. Christos thanked ESE for this work. BLM team has already got 150 and the other ones are ready to be picked up. The remaining ~15% faulty modules could potentially be repaired but if the available quantity is enough, ESE would prefer to save the TOSAs for some other use.

  - ACTION: Christos to give the minimum needed quantity to Jan

- Irradiation of COTS transceivers (Manoel):

  - Manoel gave a very preliminary report of the irradiation tests which took place on 26 and 27th of November

    - Two families of components were tested: SFP+ and SFP28 (10Gbps), and QSFP28 and SFP+ (25Gbps)

    - All were tested individually under a flux of about 10^9 (tbc) and with proton energy of about 200MeV (tbc)

    - The results are very preliminary as the tests only happened a few days ago, but a first interpretation shows that:

      - 25Gbps family:

        - The QSFP28 modules were all out of order after a few seconds

        - The Skylane QSFP+ seemed to resist a bit better but the memory was unreadable.

      - 10Gbps family:

        - Most of the modules seem to behave better than expected (loss of locks or memory being unreadable were always recoverable). In particular the Broadex module seems promising.

  - Thibaut mentioned that the VTRX-based solution remains the baseline except if the GBLD yield at 10Gbps is too low, in which case the COTS could become the fallback solution (hence the radiation tests); Even if the COTS are able to recover after a latch-up, redundancy would be required and it would be quite problematic to lose BPM links continuously. Therefore a VTRx-based solution has to be favored.

  - ACTION: Manoel will prepare a report with the details

  - ACTION: Manoel will open the modules to understand better their weak points

- TOSAs: Gamma irradiation of the 4 TOSA categories (Jan)

  - All the modules were irradiated at the CERN CC60 irradiation facility at a total dose of 30kGy

    - All modules are performing well after radiation (no significant changes were observed)

  - The CBO and the Allwave modules are 3dB less powerful (not linked to radiation) but Christos and Thibaut confirmed that this is not an issue.

    - ACTION: Prepare the Technical Spec for the TOSAs and price enquiry (Jan)

      - Andrea will accompany Jan on that process

- Muxes:

  - need to be irradiated in the GIF

    - No need to go to PSI

  - Maybe they could spend some time in Charm

  - ACTION: Irradiation of the WDM mux: a slot needs to be booked at GIF

- GLBD

  - Laser drivers are in the fab

  - Delivery of the first lot is expected in January

  - Packaging offer received

    - Novapack subcontracts this to another company

    - We might package one wafer, test them in the lab and send the rest for the packaging

    - Full tests will be handled (hopefully) at RoodMicrotek and followed up by the new Quest

- Quest for the Quest:

  - So far, only 4 applications, no obvious candidate

  - https://jobs.smartrecruiters.com/CERN/743999861609391-optical-link-development-engineer

  - ACTION: Contact HR to expand the range of publication (Jan?)

- Radiation levels for the BLMs (Christos):

  - The values given before are our best knowledge so far.

  - The locations of the systems are not fully fixed so far, so the radiation level may slightly change but the order of magnitude is fine.

- Next meeting: 3rd of February. To be expected then:

-Detailed report from the COTS QSFP and SFP tests at PSI (Manoel)

-news from the GBLDs, the TOSA spec, the mux irradiation at GIF, the ROSA