| PS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  | Matthew Fraser |  |  |  |  |  |  |  |
| Machine Coordinator this week |  |  | Denis Cotte |  |  |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |  |
| East Area | Yes | nTOF |  | Yes |  | AD | Yes |  | SPS | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |  |
| AD | 90.8% | EA N |  | 90.8% |  | EA T8 | 90.8% |  | EA T9 | 90.8% |
| nTOF | 90.8% | SPS |  | 90.8% |  |  |  |  |  |  |
| Facility Status |  |  |  |  |  |  |  |  |  |  |
| Summary | A very smooth week for the PS operation team: - First version of 1.4 GeV (injected kinetic energy) EAST cycle ready - Optimisation of fast extraction settings without TPS15: promising tests of AD and TOF cycles with 4 extraction bumpers (now possible with TPS15 retracted, instead of 2 BSWs) indicating that the KFA71/79 voltage can be significantly reduced - Commissioning with SY-BI of new East Area dBLM (in SS63) showed first fast acquisitions - Optimised BTP quadrupole settings (result of turn-by-turn SEM grid measurements) show little effect on FT emittance of LHC beam indicating that mismatch at injection is not a dominant source of emittance blow-up: detailed investigations needed to quantify effect |  |  |  |  |  |  |  |  |  |
| Issues | No major issues: - Loss of PSB-PS synchronisation on Monday morning caused by a settings issue on the barrier bucket front-end: power-cycle of the barrier bucket controller, along with reboot of most of the RF front-ends, fixed the problem: detailed follow-up by SY-RF underway - Network glitch on Tuesday morning: piquet needed to restart QSE - Cavity gaps not opening automatically when cavity is reset by OP from Working Set - KFA71/79 modules continue to trip intermittently - KFA71/79 settings issue allowed pulse length (2350 ns) > PFL length (~ 2150 ns) SW limits to be set correctly in KITS next week |  |  |  |  |  |  |  |  |  |
| Plans | - Work to continue to deploy M-TOF (2 bunches): timings PEX.WEJTOF-EAR1 and PEX.WEJTOF-EAR2 not triggering on second extraction - 4 extraction bumper settings to be rolled out on operational AD and TOF beams |  |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |  |
| No | Duration | - |  |  | Preferred date/time |  |  | - |  |  |
| Reason | - |  |  |  |  |  |  |  |  |  |
| Impact | - |  |  |  |  |  |  |  |  |  |