| PS Booster |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  | F. Asvesta |  |  |  |
| Machine Coordinator this week |  |  | C. Bracco |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |
| ISOLDE | Yes |  |  | PS |  | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |
| ISOLDE | 94% |  |  | PS | 93.5% |  |
| Facility Status |  |  |  |  |  |  |
| Summary | - All operational beams were delivered as requested, but an intense week nonetheless. Many thanks to the OP crew and all the experts that had to intervene. - Preparation of a special LHCINDIV user for the BSRT calibration in the LHC, available emittance ranges 1-5um. - Set up some monitoring tools to follow up the Quadrupoles situation from the CCC. In particular, the volume of water added in the system and the frequency of the opening of the valve. Indeed, the frequency of water refills in the PSB cooling system seems to have increased since mid-April, when the water leak was found. Any further pattern changes will be monitored by the operators and reported accordingly. - Several reference measurements on the operational users taken during the week. |  |  |  |  |  |
| Issues | - Electrical glitch on Tuesday morning caused issues in several systems including: POPS-B, Finemet cavities, Transverse Feedback and several elements on both injection and extraction lines. Beams were blocked for about an hour until all systems recovered. - Intervention on Wednesday morning to check the water leak situation on the main quadrupoles (as discussed on FOM). The situation was stable as no new leaks were found and the drainage fix on QFO11 is still in place. - The recombination septum, BT4.SMV10, tripped on Saturday night, with a WIC interlock. The expert had to intervene and an urgent access in the machine was prepared, as the electrovalve was faulty and the head had to be replaced. The beam was stopped for 2h30 while the intervention was ongoing. - Minor issues with the gain settings of the BPMs caused losses for some high intensity users, quickly identified and resolved. - Ring BPMs stopped publishing data during the weekend and rebooting the FEC didn’t resolve the situation. No impact on operational users – will be followed up on Tuesday. |  |  |  |  |  |
| Plans | - Deliver beams to downstream machines - Intervention for the main quadrupoles (as discussed on FOM) |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |
| Yes | Duration | 1h |  | Preferred date/time | Wednesday at 8h00 – beam stop 7h30 (already discussed on FOM) |  |
| Reason | Follow-up on the water leak situation in the PSB main quadrupoles |  |  |  |  |  |
| Impact |  |  |  |  |  |  |