Minutes of Mucol WP6 Task 6.3 meeting #3, May 25, 2023

(https://indico.cern.ch/event/1277734/)

Present : 	C. Marchand, G. Ferrand, D. Giove, S. Calatroni, A. Grudiev, C. Pira, M. R. Masullo,

O. Azzolini, T. Puig, J. Guttierez, G. Burt, L. Zhang, G. Keppel

1/ Dario Giove did present a proposal to perform DC based experimental tests of break-down in magnetic field of about 1T at LASA, with some small funding to start testing selected materials and surface treatment processes (see Dario-s slides). The experimental setup would be inspired by CERN pulsed DC Large Electrode Systems (slide #30 from Sergio Calatroni presentation), but with a gap of up to 1 mm, compared to CERN one limited to 100 mm. INFN counts very much on help of CERN for the design of the setup, and transfer of expertise for the measurements.

2/Sergio Calatroni did first present a very exhaustive picture of vacuum break-downs, and then showed recent results for BD measurements in DC fields preformed at CERN and Uppsala. Only one of these measurements did address the effect of magnetic field (0.54T) to BD rates, with no clear conclusion.

After some round table discussion, it is believed that some additional BD studies with DC and in magnetic fields as proposed by LASA are useful, merely to learn about influence of magnetic field at the field emission site as a function of electrical field, pulse length, rep. rate, material, etc. In this sense, it would be very useful to have plasma development simulations like done by R. Koitermaa and A. Kyritsakis (slide 15 of Sergio presentation) in the presence of magnetic field. However, dependance of BD rates as a function of B field in DC can not be extrapolated to RF, as there, it is believed that the beamlet model applies, which is governed by thermal issues.