Future Circular Collider

INTEGRATION

INTEGRATION MEETING

2024-08-07

| Document identifier: | FCC-INF-MIN-0065 ¦ EDMS 3134736 v.1 |
| --- | --- |
| Date: | 2024-08-07 |
| Work package/unit: | Technical Infrastructures / Integration |

Indico: https://indico.cern.ch/event/1429623/

Participants: Fani Valchkova-Georgieva(chair),

Liam Bromiley (SCE-SAM), Jean-Paul Burnet (ATS-DO), Federico Carra (EN-MME), Halil Deveci (TE-MSC), Klaus Hanke (ATS-DO), Barbara Humann (SY-STI), Damien Lafarge (EN-HE), Giacomo Lavezzi (HSE-RP), Charline Marcel (EN-EL), Dieudonne Ngo'O Ella (EN-ACE), Guillermo Peon (EN-CV), Antonio Perillo Marcone (SY-STI), Audrey Piccini (EN-MME), Roberto Rinaldesi (EN-HE), Ghislain Roy (BE-ABP), Pawel Skupien (EN-HE), Pavlina Trubacova (SY-ABT), Byamba Wicki (SY-EPC), Markus Widorski (HSE-RP),

Agenda

- General Information

- Comments

- Actions

# General Information

In power point document Fani presented the FCC-ee Underground infrastructure at point A (slide 3). She presentation the modification of bypass tunnels, which are connected classer to IP (97m) comparing of previous option where the bypass tunnels were connected to the machine tunnel at 600m from the IP. This new proposal came after discussion with Markus where he proposed the have ‘S’ type geometry of the bypass tunnel, as chicane, through that we can have no radioactive area in the bypass tunnel. The new geometry of the bypass tunnel has to be study in addition by Markus and Giacomo, and to be confirmed from their side the connection distance from the IP.

Also, Fani removed the connection between the evacuation tunnel and machine connection tunnel. Which was requested by Guillermo, Markus and Ghislain of the reason of better pressurization of the evacuation tunnel.

On slide 4, Fani showed the different cross sections of the enlarged tunnel which will be with different length and positions, this will be due the beams separation (electron, positron and booster).

Fani proposed to move the first alcove on the end of the LSS from 700m to 830m, which is with 130m, and inside of the alcove to be added the Laser Hutch. As that it won’t be needed to build a dedicated alcove for Laser Hutch and will not be needed to have a separate connection tunnel from Service cavern to Laser Hutch, as we can have two Laser Hutch on different points, and to provide coverage in case of mirror folds.

This new proposal of Fani will be confirmed after separate discussion with Byamba, Charline, Davide and Mario (EL team).

On slide Fani presented the new proposed organization of the Alcove where the Lasar Hutch on the machine level and the UPS systems, MV cubicles, 800kVA transformers and LV power centre are moved on balcony siting over the laser hutch.

During the meeting Andre Henriques gave presentation of Safety requirements for experimental zone.

He presented the MTR baseline where the Service shaft was elliptical with size 18m x 14m, which housed one lifts cage with staircases and in Experiment shaft we also have the lifts cage with staircases. He shows that in ATLAS we have two shafts (PM15 and PX15) for the evacuation of the people from machine tunnel and experiment area. And two shaft in CMS - PM54 and PM56, as PM56 is only for evacuation purpose. He said that during FCC Safety meeting in May (with Jean-Paul, Guillermo, Inigo, Oriol, Andre, Tomasz, Markus, Ghislain and Fani), Fani was proposed to change the geometry of the access shaft from elliptical to round, due that the shaft can house two lift’s cages with two separate safety areas in the service cavern.  Thanks, of this proposal won’t be necessary to build an additional shaft for evacuation purpose.

Andre also showed an evacuation procedure for evacuation of the people from the experimental cavern to service cavern using evacuation tunnels, the big connection tunnel with 10m in diameter and eventually EL gallery.

# Comments

Ghislain said that Fani’s proposal for eventual shifting of the first alcove is not bad but the request of Robert for alcoves with laser hutch on different experimental points is not optimal. Ghislain propose to have two alcoves with laser hutch at point A but on both sides of LSS.

Guillermo said that he has to check what will be impact for CV equipment and he has to make new estimations for cooling pipes and ventilation ducts.

Pavlina said that the booster injector which will be housed at LSS point A also will need an Alcove. She said that she will check with Wolfgang about the position of the Booster alcove. She also said that she will meet with Fani to discuss separately with her underground structure at point B, number of Alcoves and collider injection/extraction, booster dump optics.

Robert said that he will check with his colleagues the proposal of Ghislain for the laser hutch at point A.

# Actions

☐ Guillermo and Inigo have to make update scenario of CV systems at point A

☐ Fani has to check with EL team if it’s possible to change the position of the first Big Alcove on LSS.

# Next Integration Meeting

The next Integration meeting is foreseen on 21.08.2024.