An Improved Solar Declination Angle Formula for Accurate Determination of the Sun’s Position for Optimum Orientation of Solar Photovoltaic Modules.

Author: Mavyline Motari K.1

Co - authors: Geoffrey Okengo O.1; Robinson Ndegwa G.1, Rana Liaqat A.2

1 Department of Physics, Faculty of Science & Technology, University of Nairobi, Kenya

2 The Embedded systems lab, Department of Physics, COMSATS University Islamabad, Pakistan.

Corresponding Author: mavyline.motari@gmail.com

Accurate determination of the solar declination angle is essential for the calculation of other solar angles that give the Sun’s position in the sky and relative to a solar photovoltaic (PV) module surface. These angles give insight into the optimal orientation of the PV for maximised energy output throughout the year. This study analyses existing approaches for calculating the solar declination angle with varying levels of complexity and accuracy, ranging from trigonometric to Fourier series approximations. In addition, a simplified approach for calculating the solar declination angle that divides the earth's ecliptic orbit into two sections for better approximations is developed. The proposed approach demonstrates a maximum error of 0.230 around the equinoxes, outperforming conventional approaches that assumes the earth’s orbit to be a perfect circle yielding a maximum error of 1.130.  The solar irradiance levels on a solar photovoltaic module with a fixed tilt of 150 facing due north and on a surface whose tilt is adjusted daily in line with changes in the solar declination angle was simulated. The simulations were done for a normal and a leap year to ascertain the suitability of the existing solar declination angle approximations for use in a Sun Position Algorithm (SPA) to be applied in a sun tracker. The solar irradiance levels on the surface with a changing tilt were higher in both years with a minimum of 750.35 W/m2 compared to the fixed tilt whose minimum irradiance level was 533.77 W/m2.

Abstract category: Renewable energy & energy efficiency