| SPS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  |  |  | Stephane Cettour Cave, Panos Zisopoulos |  |  |  |  |  |
| Machine Coordinator this week |  |  |  |  | Stephane Cettour Cave, Panos Zisopoulos |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |  |
| LHC | No |  | NA |  | No |  | AWAKE | No | HiRadMat | No |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |  |
| LHC | % |  | NA |  | % |  | AWAKE | % | HiRadMat | % |
| Facility Status |  |  |  |  |  |  |  |  |  |  |
| Summary | Friday, 14 March: - Beam commissioning began with the injection & acceleration of 5×10¹¹ ppb SFTPRO core beam and 1.2x1011 ppb LHCINDIV beam in the SPS. Aperture and kick-response beams were set up and used for measurements on the same day. Saturday, 15 March: - Significant beam perturbations were observed. After investigation by SPS-OP team, the issue was traced to SMD13, which was removed from the configuration the following day. The perturbation disappeared after this adjustment. Monday, 17 March: - Beam-Based Alignment (BBA) measurements were successfully performed in the morning. The resulting quadrupole displacements were provided to the survey team, who completed the realignment by the afternoon. Post-alignment, both LHC and FT beam orbits matched expectations from BBA measurements. - RF voltage calibration studies were also carried out. - A multi-scraping demonstration of the LHC beam within a single cycle was successfully achieved on a 4-injection LHCINDIV cycle. Wednesday, 19 March: - Following orbit distortions observed on 14 March, OP identified the root cause as the non-closure of the BGI H+V 3C bump. The issue was due to saturation of the central bump, preventing proper closure. By adjusting the operating current from 80 A to 50 A, orbit distortions were effectively eliminated, while the integrated magnetic field remained within operational limits. - A scrubbing beam with a long flat-bottom cycle was successfully set up and integrated into the supercycle. During scrubbing: - The crab cavity was put IN BEAM to benefit from the scrubbing cycle. - The transverse damper was configured by expert. - The HiRadMat beam was also set up during the night. - Hysteresis compensation studies were conducted in the SPS from CSS team. - ALPS-related issues were identified, including oscillating BPM signals at flat top and data handling problems during long capture of 64k turns. Resolution of the problems is expected early next week. Thursday, 20 March: - No beam from PS almost all day due to RF issues and interventions. - At night, work on optimisation of scrubbing cycle with 4x72 bunches, at around 2x1011 ppb. Friday, 21 March: - Flat bottom scrubbing beam with 4 injections of 72 bunches (200 ns spacing, 1.9×10¹¹ ppb) was injected into the SPS - Discovered problem with LLRF – RF experts aware and working on the issue. - Patrol of BA81 & TCC8 done by SPS-OP - Subsequently, North Extraction setup was completed up to TT20 TED. - A 1×72b beam was accelerated to flat top at 450 GeV, with 1.4×10¹¹ ppb intensity. Saturday, 22 March: - Scrubbing continued with 4×72 bunches, 200 ns batch separation, and bunch intensities around 2.4×10¹¹ ppb, later increased to 2.5×10¹¹ ppb. Sunday, 23 March: - Scrubbing proceeded with increased intensity of around 2.6×10¹¹ ppb. - Around midday, a rising MKP temperature was observed, prompting a switch to a reduced scrubbing beam with 4×1 INDIV bunches, each at 1.3×10¹¹ ppb. - At 16:00, the SPS went on full ECO to allow RF expert to perform RF cavity conditioning. |  |  |  |  |  |  |  |  |  |
| Issues | - The LLRF issue is being followed up by RF experts. - On Thursday, there was no beam from the PS due to RF problems, and on Friday there were partial disruptions; this will be checked in the PS on Monday morning. - BPM ALPS issues are expected to be resolved early next week. - The BGI closed bump non-closure issue has been resolved. - The SMD13 was short-circuited to Earth inside the power converter, and the issue has been resolved. - Wire scanners issue discovered on Friday, resolved by experts on the same day. |  |  |  |  |  |  |  |  |  |
| Plans | - Scrubbing with beam at FT - Two dedicated MDs in the SPS - Fast extraction set-up for HiRadMat, LHC & AWAKE - Continue Hysteresis Compensation studies - Continue slow extraction set-up with ZS alignment and TEX&TECA commissioning. |  |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |  |
| Yes / No |  | Duration |  |  |  | Preferred date/time |  |  |  |  |
| Reason |  |  |  |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |  |  |  |