| AD - ELENA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Supervisor last week |  |  | Bruno Dupuy |  |  |  |  |
| Machine Supervisor this week |  |  | Laurette Ponce |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |
| AD |  |  |  | ELENA |  |  |  |
| Availability (AFT) |  |  |  |  |  |  |  |
| AD |  |  |  | ELENA |  |  |  |
| Facility Status |  |  |  |  |  |  |  |
| Summary | AD : - Performance remained normal, despite several random fluctuations in efficiency, which may point to underlying issues. ELANA : - Excellent performance, Irradiations resumed on TELMAX line for PAX team, using a 198 micro-bunch cycle. Monday : - Following the power cut and GPS timing issues, nominal beam conditions were restored in AD and ELENA. Specialists put the collector power supply of the AD electron cooler back into service. Wednesday : - Machine Development (MD) was carried out for acceptance, emittance, and dispersion measurements on the AD and the AD–ELENA transfer line. - Partial sublimation was performed on the kickers in sector 1A of the AD to contain a minor leak previously detected in sector 2B. |  |  |  |  |  |  |
| Issues | - Tuesday (14:00–17:00 and at 19:00): Poor injection into AD despite normal beam intensity and trajectory. No issues observed on C10 cavities, bunch rotation at injection. A suspected RF phase mismatch between the PS and AD is under investigation. - Reset on C10-25 and C10-26 cavities during long weekend. |  |  |  |  |  |  |
| Plans | - Several AD equipment are under monitoring for diagnostic purposes: the extraction kicker, the GFAS on TRIM51-52, and the C10 cavities due to phase jumps at injection. |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |
| No | Duration |  |  |  | Preferred date/time |  |  |
| Reason |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |