| LEIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Machine Supervisor last week |  |  | Theodoros Argyropoulos |  |  |
| Machine Supervisor this week |  |  | Oliver Hans |  |  |
| Statistics |  |  |  |  |  |
| Availability |  |  |  |  |  |
| Facility Status |  |  |  |  |  |
| Ion species | O4 |  |  |  |  |
| Summary | -The good performance of the NOMINAL beam was recovered with intensities above 4.5e10 charges. - However, bunch length/longitudinal emittance at LEIR extraction was quite large (300-350ns), resulting in a very large bunch at PS extraction (>5ns) as well as a strong satellite bunch with population ~10%. - Reducing the bunch intensity to ~3.75E10 charges by mistuning the RF frequency at LEIR capture improved the situation: ~270ns@LEIR extraction, ~4.4ns@PS extraction (still quite large), ~5% of satellite population. - EARLY beam was also optimised for intensities above 3E10. Adjusted also the RF voltages at capture to reduce the longitudinal emittance and thus the population of the satellite bunch at the PS extraction. -EARLY beam was provided for North Area physics. |  |  |  |  |
| Issues | -Issue with Cavity41 still remains. RF experts are working on it. -Cavity 43 trips over the week, resulted by the wrongly set type of PARTY in the beam generation. To be followed up in order to avoid similar issues in the future. -Problem with the control of various FECs on Friday. Solved after rebooting them. |  |  |  |  |
| Plans | -Provide beam to physics, EARLY (North Area) and NOMINAL (LHC). -Improve as much as possible the beam: intensity and longitudinal bunch size. |  |  |  |  |
| Intervention Request |  |  |  |  |  |
| Yes / No | Duration |  |  | Preferred date/time |  |
| Reason |  |  |  |  |  |
| Impact |  |  |  |  |  |