| PS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  | Alex Huschauer |  |  |  |  |  |  |  |
| Machine Coordinator this week |  |  | Alex Lasheen |  |  |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |  |
| East Area | Yes | nTOF |  | Yes |  | AD | Yes |  | SPS | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |  |
| AD | 95.2% | EA N |  | 95.2% |  | EA T8 | 95.2% |  | EA T9 | 95.2% |
| nTOF | 95.2% | SPS |  | 97.4% |  |  |  |  |  |  |
| Facility Status |  |  |  |  |  |  |  |  |  |  |
| Summary | Very good availability and successful operation with ions for the LHC Oxygen run - Beginning of the week dedicated to testing the different LEIR/PS cycle combinations to agree on beams for LHC together with the LEIR experts - Eventually created two main variants - High-intensity single bunch with about 2% satellite population (NOMINAL from LEIR, satellite population~1% measured in the LHC) - Lower-intensity single bunch without satellites (ANOMINAL from LEIR) - NOMINAL-variant taken for p-O run - ANOMINAL-variant taken for O-O run due to issues with LHC BPMs - Optimisation of the AD performance - AD realised slightly longer bunches at extraction from the PS with direct impact on their transmission - PS cycle adjusted subsequently (long. Blow-up, gamma jump amplitude, transition timing) and slight reduction of total intensity to previous operational value of 2050E10 p - Bunch length back to 32-33 ns (from 35 ns before) Ad transmission immediately improved - 8b4e @ LIU intensity being prepared for SPS MD on 17 July - Several different cycle variants tested in EAST T8 - 12 GeV and Oxygen ions at different energies - Clear reduction of noise on FTA BLMs observed after installation of filters by BI experts during TS |  |  |  |  |  |  |  |  |  |
| Issues | - 30 mins stop on Tuesday due to intervention of electrical network compensator - Injection kicker KFA45 communication error causing ~3h downtime Thursday morning - Other than that, only very short interruptions of operations |  |  |  |  |  |  |  |  |  |
| Plans | - LHC Ne preparation as of Monday afternoon |  |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |  |
| No | Duration |  |  |  | Preferred date/time |  |  |  |  |  |
| Reason |  |  |  |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |  |  |  |