| PS Booster |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  | Raul Murillo Garcia |  |  |  |
| Machine Coordinator this week |  |  | Jean-François Comblin |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |
| ISOLDE | Yes |  |  | PS |  | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |
| ISOLDE | 87.8 % |  |  | PS | 89.2% |  |
| Facility Status |  |  |  |  |  |  |
| Summary | A bumpy week dominated by single long issues that hindered the machine availability, especially R4. Beams: - Three versions of the VdM beam were prepared and tested early in the week so they could be sent to the LHC as requested: - Low transverse emittance single bunch with 1.2e11 p/b - 2.5 um transverse emittance, four bunches with 1.2e11 p/b - 1.2 um transverse emittance, single bunch with 1.2e11 p/b - Beam sent to ISOLDE HRS and GPS - PSB in degraded mode without R4 or with R4 intensity limited due to the issues encountered on Thursday and Friday (see below). On Thursday morning the R4.H0HM monitoring triggered false positive interlocks. Understanding and eventually fixing the cause required three machine accesses and extensive diagnostics by experts from EPC, BI, ABT in collaboration with OP and MPP. See detailed explanation below. |  |  |  |  |  |
| Issues | - Monday: BI2.SMV10 required reset of the FGC3 (17m). - Wednesday: BR.BLM.B cut the beam several times during the early morning although there were no hardware interlocks (4m, 6m). When the expert intervened, he realized the acquisition crate was randomly restarting. The issue was resolved by changing the 48V power supply (2h44m). - Wednesday: BTY.QFO184 tripped and was impossible to reset. Piquet called but they were busy due to a fire in ADE. The expert was called at 8:00, who went on site and replaced the electronics power supply (3h15m). - H0H- monitoring: since Thursday morning the BR4.H0HM monitor started exhibiting high signals which interlocked the ring even when beam was not present. OP verified the foil was ok. - The H0H- input is common for the 4 rings so PSB cannot work in degraded mode. - An access was given for SY-BI experts, who replace part of the equipment without any effect. They masked out R4 to allow PSB to run in degraded mode. The experts noticed the cables of some of the BI.BSWs moving and the ceiling lights flickering when the power converters were pulsing. - SY-EPC expert was called in since it was suspected that power converters were affecting the BI electronics either through EMI or grounding. Indeed, when some BSW converters were off the H0H- signal was ok. It was decided to organize an access on Friday morning at 9:00 for SY-EPC, EN-EL and SY-ABT experts. Beforehand, PSB run in degraded mode without R4. - Friday at 9:00 the experts accessed the machine. After extensive tests, they concluded that the BSW converters are not causing the H0H- misbehaviour. - Emergency meeting at 14:00 with EPC, ABT, BI, MPP and OP. A strategy to increase the threshold of the H0H- and relay on other systems would guarantee the same protection. RF bypass suspected to not work, leading to Eddy currents which could potentially affect other systems. - Third access at 16:00. The experts verified the RF bypass was in fact ok. But the insulation of the R4 chamber was not working as expected anymore. They gave the green light to pulse the BSWs whilst the operators modified the interlock settings to allow beam in R4. - The source of the problem is not yet understood. The R4 BSW magnet might need to be replaced due to the insulation problem detected. - Total downtime: 11h58m. - Friday: beam losses were detected after R4 is put back in due to the Transfer Feedback (TFB) BPM in R4 being saturated. Beam was lost at intensities of ~300e10 ppr. OP reset the FEC, but the situation remained. The beam in R4 is limited until the issue is fixed. On Saturday morning the experts went on site and after doing some diagnostics, rebooted the FEC cfv-361-atfbr4 again. This fixed the TFB and the intensity limitation was removed. - Saturday: BI2.SMV10 tripped (29m). Piquet called to power cycle the FGC3 |  |  |  |  |  |
| Plans | Continue providing beams to the downstream accelerators/facilities |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |
| Maybe | Duration | 1 day |  | Preferred date/time | TBD |  |
| Reason | An access might be needed for replacing/opening the R4 BSW magnet after detecting its insulation being not ok. |  |  |  |  |  |
| Impact | No beam |  |  |  |  |  |