| AD - ELENA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Supervisor last week |  |  | Laurette Ponce |  |  |  |  |
| Machine Supervisor this week |  |  | Lajos Bojtar |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |
| AD | Yes/No |  |  | ELENA |  | Yes/No |  |
| Availability (AFT) |  |  |  |  |  |  |  |
| AD | % |  |  | ELENA |  | % |  |
| Facility Status |  |  |  |  |  |  |  |
| Summary | * good production week: 8 hours in degraded mode Saturday due to ELENA e-cooler filament power supply fault and 1 hour without beam due to water level interlock on the AD e-cooler * H- source operation for GBAR everyday from 8:00 to 22:00 * in the shadow of LINAC4 and PS intervention, access in AD to replace 3 modules of the finemet cavity |  |  |  |  |  |  |
| Issues | * 3 modules of finemet cavity exchanged due to burnt resistors * BCCCA parameters adjustement by specialist * 2 GeV new opticla notch filter to be restarted after PS intervention due to switching to MD mode during the access to fix the timing issue of last week * trip of ELENA e-cooler filament, power cycle of FGC needed by First line * water level interlock on AD e-cooler, OP on site to refill the tank |  |  |  |  |  |  |
| Plans | * Physics with Pbars and Hminus |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |
| Yes / No | Duration |  |  |  | Preferred date/time |  |  |
| Reason |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |