| SPS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  |  |  | P. Arrutia |  |  |  |  |  |
| Machine Coordinator this week |  |  |  |  | T. Levens |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |  |
| LHC | Yes |  | NA |  | Yes |  | AWAKE | No | HiRadMat | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |  |
| LHC | 97% |  | NA |  | 88% |  | AWAKE | % | HiRadMat | 95% |
| Facility Status |  |  |  |  |  |  |  |  |  |  |
| Summary | Very dynamic week balancing HiRadMat run, SFT high-intensity physics, LHC filling and LHC MD preparation. HiRadMat run (HRMT-67): - Extensive pulse list, successfully completed by Thursday evening. SFTPRO: - Standard physics production. Availability somewhat compromised due to issues with splitter magnet (See Issues). LHC: - Some issues filling due to trips of 800MHz cavity 1 across the week. - On Friday, erratic injection orbit coming from misbehaviour of PS extraction kicker. LHC MD preparation (Friday): - Affected by challenging LHC fill, PS beam stop and RF power overdrive difficulties -> PS sent some bunches with 3.5e11 p/bunch instead of ~2.5e11 - Fully tested high-intensity 8b4e beam, but not enough time to optimise injection for LIU beam (2.3e11 p/bunch, 72 bunches, 4 batches). - Other MD beams also checked, but less challenging overall. |  |  |  |  |  |  |  |  |  |
| Issues | - MSSB.2117 power converter (~6h downtime for NA only): several trips of power converter for splitter magnet in NA transfer line (T6). After inspection and tests, power converter had to be exchanged. - Difficulties with 800MHz cavity 1 (degraded beams sporadically): several trips across the week due to power overdrive. Maximum voltage for cavity was reduced and RF expert had to rebalance load. |  |  |  |  |  |  |  |  |  |
| Plans | - LHC MD week (Monday to Friday morning) - SFTPRO physics - Intensity reduction on Wednesday - Parallel SPS MDs on Friday |  |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |  |
| No |  | Duration |  |  |  | Preferred date/time |  |  |  |  |
| Reason |  |  |  |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |  |  |  |