| PS East Area |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Facility Coordinator last week |  |  |  | N. Charitonidis |  |  |  |  |  |  |  |
| Facility Coordinator this week |  |  |  | B. Rae |  |  |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |  |  |
| T8 | Yes | T9 |  |  | Yes |  | T10 | Yes |  | T11 | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |  |  |
| Running T8 | 90.3% | T9 |  |  | 90.3% |  | T10 | 90.3% |  | T11 | 90.3% |
| Facility Status |  |  |  |  |  |  |  |  |  |  |  |
| Summary | T09: Good operation. Beam checked for HIKE and all files available for different momenta. T10: Good operation. T11: Good operation and end of run for P349. Experiment de-installed on Friday. |  |  |  |  |  |  |  |  |  |  |
| Issues | General: The manual beam request feature (required for muon beams with the beam stopper IN) has been fixed and tested for all beamlines. The functionality was migrated from SISv1 to SISv2. Many thanks to PS-OP and CESAR team for the help and very fast implementation. |  |  |  |  |  |  |  |  |  |  |
| Plans | T09: HIKE SAC → HETCAL. T10: HERD → ALICE ITS3. T11: P349 finishes 28 August 2025, then no user, preparing for CLOUD. |  |  |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |  |  |
|  | Duration |  | 4 h |  |  | Preferred date/time |  |  | 3 September 08:30 |  |  |
| Reason | Access to mixed area and T9-T10 zone: - Removal of signal cables T11.XBPF022 in the mixed zone; - Pump controller exchange in APRON barrack; in case of vacuum loss on any XSEC, a re-pumping intervention will be carried out, hence the need to have full access to all EA1 zones. - Install the new “vacuum control boxes” for the XCETs in the T9-T10 area. - Close T11 shutter. |  |  |  |  |  |  |  |  |  |  |
| Impact | No beam to East Area. |  |  |  |  |  |  |  |  |  |  |