| HL-LHC: Decision Management |  |  |  |
| --- | --- | --- | --- |
| Decision Description |  |  |  |
| WP13 | WP and PBS # when possible | Date of Issue | 2025-xx-xx |
| This document summarizes the outcome of the HL-LHC Beam Halo Monitoring Review held on December 18, 2024 [1], including the review panel's recommendations [2] and their planned implementation by the involved teams. |  |  |  |
| Facts |  |  |  |
| The HL-LHC Beam Halo Monitoring review panel recommendations can be summarized as in the two following tables. Table 1- Systems’ Status Overview System; Status. BSRH; Most promising instrument, recommended for baseline with additional developments. BGI; Baseline device for emittance measurements, promising backup for halo monitoring pending technical validation. BGC; Limited performance for halo monitoring but viable for profile measurements. Wire Scanner/Low Density Materials; Not viable for baseline but development continues for future applications. Table 2- Review Panel Recommendations Summary R#; System; Summary. R1; Cross-System; Continue MDs during Run 3 to study halo population dynamics throughout LHC cycle, particularly for higher bunch intensities. R2; Cross-System; Ensure selected measurement tool can measure halo throughout entire LHC cycle. R3; Cross-System; Prioritize collimation and MP specifications for Run 4 start over beam-beam use case features. R4; Cross-System; Maintain round beams constraint at e-lens location, ensuring compatibility with IR4 optics changes. R5; BSRH; Implement 2nd SR light source and extraction line for improved performance and maintenance. R6; BWS, Low Density Materials; Continue technology development through consolidation project despite not being viable for baseline HL-LHC. R7; BGC; Continue as option for profile measurements but not as baseline for halo measurements due to limited performance. R8; BGI; Make system design compliant with additional gas injection to maintain viability as backup halo measurement option. R9; BGI; Set mid-2026 hold point for LS3 deployment decision, pending validation in injectors and LHC prototype tests. R10; Alternative; Investigate Multi-Aperture interferometry with coronagraph, leveraging radio-astronomy concepts for 2D measurements. |
| Options |  |  |  |
| N/A |  |  |  |
| Rating of Options |  |  |  |
| N/A |  |  |  |
| Decision |  |  |  |
| Approved for implementation in the WP13 PSM and CSR XXXXXXXXXXXX |  |  |  |
| Implementation actions |  |  |  |
| The SY-BI group and parties convening in the BH WG reviewed and accepted the recommendations from the review panel. The BH WG will oversee all implementation and keep relevant HL-LHC entities informed of the progress. The BH WG will discuss and propose actions to address recommendations R1 through R4 (see Table 2). Here are the present implementation plans for the different systems: BSRH (addressing R5): - Implement additional synchrotron radiation extraction systems for beam halo measurements for B1 and B2 - Establish optical line monitoring in UA43 and UA47 - Complete documentation (ECRs) by September 2025 - Review resource planning within HL-LHC budget by October 2025 BGI (addressing R8, R9): - Update HL-LHC-WP13-BGI task (for beam profile monitoring) to include gas injection system capabilities - Review deliverables by mid-2026 based on SPS and LHC-prototype test results - Prepare required documentation (ECRs) BGC (addressing R7): - Continue development as profile monitor - Maintain design compatibility with future halo measurement capabilities - ANY NECESARY DETAIL ABOUT BUDGET AND IN-KIND CONTRIBUTION STATUS / CHANGE ? Wire Scanner (addressing R6): - Continue R&D activities outside HL-LHC scope Alternative Techniques (addressing R10): - While multi-aperture interferometry development isn't feasible for HL-LHC start-up, we'll monitor its potential through FCC R&D for future LHC applications |  |  |  |
| Approved by | O. Brüning (HL-LHC PL), XXXX | Date of Closure | 2025-XX-XX |

Shared for information:

REFERENCES:

[1] HL-LHC Beam Halo Monitor Review (18 December 2024): Overview · Indico

[2] Review Report  EDMS [PUT LINK WHEN AVAIALBLE]