Notes on WP8 Meeting #28 – 11/07/25

- Discussion on the “MuCol Cooling Cell Outcome of the 1st Integration Review”

- Recommendation 1: concerns on the use of LiH as absorber material (chemically reactive) and the use of unconventional materials for connections (SMA rings, pillow seals).

LiH is already our second option instead of pure hydrogen. Roberto proposes to assess the risk of LiH. In parallel, Chris will study the degradation of performance in case other materials are considered.

Experience with LiH is very limited, a thorough study of this material and the related risks is needed. Daniel reminded that at a later point, hydrogen should be considered for the 6D cooling section.

Consider LiH baseline and do a risk assessment of this material.

The reviewer asked for more conventional material for the connections. Risk assessments on SMA rings and pillow seals are needed.

Do a risk assessment on SMA rings and pillow seals (vacuum loss).

- Recommendation 2: concerns on the 10% maximum force unbalance hypothesis.

Reviewers asked to confirm this hypothesis. WP7 should take responsibility on this answer, this is one of the main priorities on the magnets, but we miss a tool to predict the force unbalance.

Consider the involvement of TE-MPE or subcontract this analysis to Little Beast Engineering.

Even a first evaluation will be relevant at this stage: if the 10% force unbalance hypothesis does not hold we need to re-think the magnets and the cell design.

A joint WP7-WP8 meeting is proposed next week. Participation of Bernardo and Luca is needed.

Although the force unbalance problem is not a concern for the single demonstrator cell design (and for the RFMFTF split coil design), it is important that we consider it, since the design of the RFMFTF and of the single demonstrator cell should have technical solutions consistent with the final integration.

- Recommendation 3: cooling cell cryogenics: gas-cooling or cryocoolers?

Marco proposes to study the He gas-cooling option, supporting this decision with simulations of pipe cooling (pipe dimensions, cryoplant considerations, etc.).

Study He gas-cooling with heat exchangers.

Do a rough thermal and mechanical estimate with cryo-pipe holes in the cold mass (a first order dimensioning useful for integration aspects).

- Recommendation 4: define the vacuum layout and sectorization.

Jose will help us with that.

- Recommendation 5/6: Investigate the accidental scenario of a loss of vacuum.

Make assessments on the loss of vacuum if using SMA couplers and if considering pillow seals.

- Recommendation 7: Review the cell support design.

Currently work in progress by Mattia.

- Recommendation 8: Consider the integration of axial stoppers between the cells.

Currently work in progress by Mattia.

- Recommendation 9: Assess the pressure on RF windows for transient op. and accidental scenarios.

Needed to confirm the feasibility of the proposed RF window design.

Dario will consider the impact of a dramatic loss of vacuum. He believes that a loss of vacuum cannot be withstand by the RF window: important to prevent any loss of vacuum.

study solutions to prevent any loss of vacuum. Challenges due to the tight space: not possible to install gate valves.

- Reviewer comments: Are there other more promising solutions worth exploring?

Girder concept: integrate the first 4-5 cells in the girder.  Work still needs to be done on the integration before evaluating this.

Concerns on the interference between the RF cavity and the cryostat: can we increase the magnet radius? Such that we could avoid interference. Marco is more concerned on the assembly of the waveguide. How to do it?

Dario is looking at the feasibility of the construction of the cavity, together with an external company. New dimensions of the waveguide showed by Dario, smaller than the previous design. Design with flange brazed to the cavity body (brazing + screw fixing). Water cooling channels will be within the body of the cavity.

Proceed with cavity design. Mattia will then integrate it with the present magnet configuration.

About the possibility of increasing the magnet radius but decreasing the magnetic field, Chris says that if we reduce the field, we will need to decrease the RF cavity frequency, so cavity will be bigger. This is not an option; costs will also increase.

According to Chris, the main issue is the IRIS radius: by reducing the magnetic field gradient, we will increase the beam emittance. If we could increase the IRIS radius and the beam pipe, we could afford this smaller gradient.

About the remote handling, this has been already developed for the LHC. However, we have space limits in the proposed tunnel. Difficulties in accessing it, considering it will be a high radiation area.

Bottom line (Lucio): the design needs to be fixed. A program that proves the feasibility of the solutions studied (SMA, pillow seals) is required. It will be important to guide the decision on whether to adopt SMA rings or pillow seals.

- Discussion on the executive summary of the IMCC Annual Meeting 2025

- Cooling cell aperture and mechanical integration

INFN is working on the 120 mm IRIS, Chris recommended the 160 mm IRIS aperture option. Are there updates on that?

Simulations by Dario and people of INFN Turin are ongoing. These are important to prove the vacuum issue discussed above.

It is a central point to dimension the IRIS size. Studies on the vibration modes of IRIS lamina will be presented by CERN people in July and by Dario in September.

Preliminary simulations by Chris on the 120 mm IRIS radius, showed a reduction in performance. Dario is currently considering 120 mm option, but he will consider also the 160 mm option.

Discuss the advancements from RF / physics at the next WP8 meeting in September.

- Al or Be RF windows?

Experimental work on Aluminum and Beryllium windows is needed.

Chris is considering Beryllium windows in simulations. Aluminum will soon be considered since it is the agreed baseline decided during the DESY meeting. He expects no significant loss of performance.

Fix Al as baseline material for the demonstrator: we will eventually consider Beryllium in the future, after passing all the regulation issues. Beryllium will be considered for the 6D cooling section

A similar choice has been made with the choice of LiH instead of hydrogen. We will consider an upgrade of hydrogen absorbers in the future. H H

Lucio suggests to confirm the design options and have a roadmap of the solutions that will be used for the demonstrator and for the actual 6D cooling section, as the options for the two may differ.

Alexey suggests the possibility of avoiding the use of RF windows for the demonstrator, since different solutions may be considered for the demonstrator / 6D cooling. Will this have an impact on the cost? Cost optimization is anyway needed.

- Evaluation of a continuous cryostat proposal by Alexey

Vacuum envelope proposal: instead of sealing the flange, seal the outer cryostat radius (à la LHC). Advantage is the assembly (no vacuum sealing in small spaces), and less risk of differential pressure on the RF windows. The cavity must not be leak tight: holes in the cavity can be made. Instead of a vacuum sealing connection, only the electrical connections are needed (beam screen).

Explore further this option.

Marco sees that one issue here is the connection of the waveguide. We need waveguide feedthroughs through this vacuum envelope. According to Alexey this is doable, it should not be an issue (no significant increase in complication compared to the previous option). A big disadvantage is that the volume to put inside vacuum is much bigger compared to this solution.   Jose will need to evaluate this possibility.

Fix a meeting with CERN cryostat section (V. Parma etc.), to discuss this solution.