MINUTES

# FCC-ee Layout and Optics Design Working Group 
MEETING No. 10

2025-09-04

Chair: G. Roy

Speaker: D. Lafarge, M. Boscolo, P. Raimondi

Participants in person: M. Boscolo, G. Broggi, J. P. Burnet, C. Carli, L. Deniau, M. Jebramcik, P. Hickersberger, K. Hanke, J. Keintzel, P. Krkotic, A. Lechner, M. Le Garrec, S. Kostoglou, D. Lafarge, D. Mergelkuhl, T. Mori, M. Morrone, K. Oide, N. Pinillos, P. Raimondi, G. Roy, R. Tomas, F. Van Der Veken, A. Vorozhtsov, B. Wicki

Participants in Zoom: K. André, H. Bartosik, P. Burrows, R. De Maria, D. Domange, A.  astrinellis, R. Kieffer, F. Palla, G. Perez, F. Poirier, P. Janot, L. Valle, C. Vollinger, G. Wilkinson, S. Yue, C. Zannini

# Comparison criteria for GHC and LCC: Transport (D. Lafarge)

D. Lafarge starts with an overview of the main challenges in transport: the geometric constraints due to limited space, which is the most critical aspect, and the logistics of installation which can impact installation times. For the geometric aspects, the key quantities are the cross sections, the transport zones and the dimensions of the objects to be transported.

Current status for GHC lattice:

- Arcs are well known and the main challenge are the supports (collider girders).

- Alcoves, where the main issue is the height of the transport zones in the arcs w.r.t to large elements such as transformers.

- Klystron galleries, RF tunnels are under study.

- Studies have started for point B as there is more information from the integration model.

- Limited information available for service and experimental caverns.

Transport strategy:

- The dipoles will be transported from the experimental points as they cannot fit in the shafts of the technical points.

- The girders will be transported from the technical points.

- The convoys will cross in the middle of the arc.

Preliminary comparison between GHC vs LCC:

- The key issue for the comparison of the two lattices is to identify the largest elements as increasing the element sizes directly impacts transport feasibility.

- The strategy for the transport is expected to be the same for both lattices.

- The overall schedule is expected to be similar between the two lattices.

- RF systems are well defined and can be included in the comparison.

- There is sufficient information for the small and big alcoves to include them in the comparison.

- Not enough information is available for the tunnel widening areas for both GHC and LCC.

Prerequisites:

- Dimensions, weights, quantities, installation location, transport constraints for all elements.

- The data that are currently available for GHC are the magnets in the arcs and the supports, the RF elements in points H and L, information for equipment alcoves, transport zones all over the machine.

- Ideally, comparable equipment data for point B for both LCC and GHC will be needed for the comparison.

## Discussion

- P. Raimondi asked about the maximum dipole length. D. Lafarge explained that there are 17 m-wide shafts in the technical points, so the dipole length should not exceed 16 m. G. Roy noted that the current target is to keep all elements below 12 m. D. Lafarge replied that studies of the 15 m dipoles for FCC-hh showed that transport is feasible, so a length up to 15 m should also be acceptable.

# Comparison criteria for GHC and LCC: MDI (M. Boscolo)

M. Boscolo presented an overview of aspects related to the GHC vs LCC comparison from the point of view of MDI. One important prerequisite for the comparison is the high-level IR optics and beam parameters such as L*, crossing angle, beam sizes at IP, drift between QC1 and QC2 and physical aperture at Final Focusing Quadrupoles (FFQs). In order to compare the same luminosity reach, these values should be the same between the two lattices.

QC1/2 aperture, collimators & SR masks

Based on GHC simulations by K. Oide, the QC2 aperture needs to be increased beyond 23 mm. The current baseline is 20 mm in QC2 and 15 mm in QC1. Increasing QC2 to 25–30 mm would allow for larger horizontal collimator gaps. Similarly, increasing QC1 to 20 mm radius would allow for larger vertical collimator gaps, which is beneficial since the TCP.V has the smallest half-gap and makes the largest contribution to impedance. For the LCC, the beam-stay-clear at the FFQs was previously computed by G. Broggi, but this will need to be updated with the latest lattice versions. Increasing the aperture also impacts the SR masks around QC1 and QC2, which intercept radiation upstream of the collision point. Their effectiveness must be re-assessed to make sure that they are compatible with the updated beam-stay-clear.

Main lattice prerequisites:

- Complete lattices to produce loss maps for beam-induced background simulations.

- Finalized IR layout for MDI design.

- Experimental insertions.

- Vacuum pressure maps through the whole ring to evaluate beam-gas loss maps

- Heat load estimates in the IR beam pipe to design active cooling systems and absorbers.

Input will be needed from various teams such as collimation, vacuum, shielding and radiation, beam instrumentation, magnets, alignment and tolerances.

IR beam losses and MDI collimators:

Preliminary studies comparing GHC and LCC loss maps indicate differences in beam losses at the IRs, mainly due to different collimator opening settings. Together with the collimator settings, the exact impact must be further evaluated by tracking losses into the detectors to estimate background levels. Information such as vacuum pressure profiles and SR background estimates will be needed.

Beam induced backgrounds, heat load in IR beam pipe & QC1 design

Beam-induced backgrounds that depend on optics are expected to be different between the two lattices for single beam effects, while those induced by colliding beams should be similar across the two optics for the same high-level parameters. Heat load in the IR beam pipe is also important, since it determines the requirements for active cooling. However, if the beam size, current and bunch spacing are identical, no major differences are expected between GHC and LCC. The design of QC1 is expected to be similar for both lattices

SR backgrounds

Due to the different dipole configurations, a different SR impact is expected between the two lattices in the IR, which translates into different SR collimator settings requirements and different power deposition on the beam pipe. A first analysis by K. André, with an older LCC optics version, showed that the collimator and mask closest to the IP receive higher SR with LCC by a factor of 10 w.r.t to GHC. It must be noted however that in both studies no SR reaches the detector. Further comparative studies with more recent optics versions will be required to assess this properly.

Solenoid compensation scheme

Two strategies under consideration: local compensation with strong anti-solenoid (-5T) in front of QC1 or non-local compensation, where anti-solenoid is outside of detector. The non-local compensation allows for increased detector magnetic field and offers a reduction of SR at the IR. The impact of the non-local compensation scheme on polarization needs to be estimated. The non-local scheme has been implemented in SAD and MADX for the LCC optics. Although this topic will not be part of the GHC vs. LCC comparison, it has important implications for MDI studies and a decision will need to be taken soon.

Overall, the choice of optics has a significant impact on MDI: if high-level parameters are the same, the IR mechanical layout and integration should be equivalent, but the background conditions will still be affected. Collimator settings should therefore be re-optimized, beam-loss maps determined, detector and FFQ backgrounds assessed together with shielding and radiation studies.

## Discussion

- A. Lechner mentioned that a full comparison, including tracking studies with full shower propagation, will require more time. He suggested using figures of merit (e.g.  for SR) to guide the comparison instead of full-scale simulations. G. Roy agreed that full detailed background studies cannot be completed within the current timeframe. He underlined that the performance of both lattices is expected to be comparable if the basic parameters are the same, but highlighted the strong interaction between MDI considerations and external systems such as collimation. Input from other groups will be essential.

- P. Raimondi noted that K. André could repeat the SR background studies with updated lattice files.

- P. Janot indicated that a factor of 10 more SR for the LCC optics is worrisome. He emphasized that minimizing SR must be in the design and selection criteria. P. Raimondi added that although photons may hit the masks, they do not necessarily reach the detector and that these numbers are not yet final, but the studies will be repeated. M. Boscolo pointed out the need to track SR from the masks to the detector.

- G. Roy mentioned that the comparison will be carried out using frozen optics. However, if simple fixes to problems are possible, they should be mentioned in the report, even if not immediately implemented. For example, SR is a critical issue but the fact that it can be intercepted by masks should also be clearly stated.

- P. Janot added that polarization must be part of the comparison. G. Roy confirmed that it is included and will be covered in a dedicated chapter.

# Update on LCC optics (P. Raimondi)

P. Raimondi presents an overview of the latest updates in the LCC optics. The latest version is v105, which is available for Z and ttbar. It includes a finalized design of the arc cells, the final focusing section, collimation, injection, with the RF section nearly complete.

Arc cells: All dipoles have the same length. There are 2 quadrupole and 4 sextupole families. The inter-magnet distance is currently set to 15 cm but it can be adjusted based on input from other groups. From the point of view of magnetic interference, P. Raimondi noted that studies have shown that, although we can have short distances between quadrupoles and sextupoles (~order of the vacuum pipe diameter), the spacing between dipoles and other magnets must be larger (~twice the order of the vacuum pipe diameter) to avoid magnetic interference.

Straight sections: Each straight section has been optimized individually. The generic straight section includes:

- Horizontal dogleg and vertical separation: the horizontal dogleg is implemented using 2 horizontal dipoles at a phase advance of 2 pi to close the dispersion bump. For the vertical separation, 4 vertical dipoles are needed. To suppress dispersion leakage, 2 quadrupole doublets are needed in between the vertical dipoles. Both dogleg and vertical separation can be switched on and off transparently. Some second-order dispersion leakage occurs that is mitigated with 4 sextupoles/side in the DS. The horizontal dogleg and vertical separation are not present for the RF section for ttbar.

- Dispersion Suppressor: length has been reduced to free up additional space in the straight sections especially for the RF section. 3 dipoles are used to build-up the dispersion resonantly to match the dispersion in the arcs.

- Dedicated optics based on insertion functionality.

Betatron collimation: Peaks of beta function at ~700m and 30 degrees phase advances between primary and secondary collimators as requested by collimation team. An identical sequence is then repeated with a 90-degree phase advance that can be used to collimate at two phases.

Diagnostic section (RF booster insertion): At the moment, identical to collimation section.

Injection: In the current design, closed dispersion bump is created with 4 dipoles. Some dispersion leakage from the arcs is used to enhance the dispersion at injection point. This bump is nearly second-order dispersion transparent and no sextupole adjustment is required. P. Raimondi also presented the alternative option of removing one of the doglegs and have a single and larger dispersion peak, together with a displaced crossing point, making the insertion assymetric.

RF: For the Z mode, the separation is increased to 96cm from 35cm. This is achieved by reducing the strength of 4 dipoles and increasing the strength of 3 DS dipoles. This creates a closed bump in terms of angle, position and dispersion, with minimal impact on second-order dispersion. The space between the quadrupoles is the one requested for the spacing between RF cryo-modules and can be changed. For the ttbar mode, the increase of strength in the incoming DS dipoles is the same as in Z mode to increase the separation by 61cm. Then, the outgoing DS dipoles have the same strength as the incoming to remove the additional separation. This insertion for ttbar will be further optimized.

Energy collimation: Can be placed at the second dispersion peak of the injection insertion. At this location, there are several dipoles (3mrad) followed by a field free distance of 150m that can be used for the polarimeter.  Similar conditions can be created in the diagnostics insertion.

Final focus optimization: final doublet of GHC is now used for LCC. Issue with DA reduction with quantum SR is under investigation.

## Discussion

- A. Lechner asked if this LCC version includes the splitting of dipoles. P. Raimondi replied that there is a version by M. Jebramcik.

- A. Vorozhtsov asked about the criteria for magnetic field interference and what is considered as the minimum permissible overlapping field between dipole field and field of other magnets. P. Raimondi replied that if the distance of the dipoles and the other magnets follows the scaling law from the studies, there should be no interference.

- C. Carli asked if the irregularity at the center of the arc (FODO cell in the middle that is shorter due to absence of sextupole between two central dipoles) creates any dispersion beating. P. Raimondi replies that this is indeed the case but the effect is small.

- A. Lechner mentioned that there are also requirements and constraints from the extraction lines in the injection insertion that need to be taken into account. G. Roy replied that this will also be addressed with a dedicated discussion.

- K. Oide mentioned that in GHC there is enough space for the polarimeter at each insertion. P. Raimondi replied that one polarimeter location was requested but this can be further improved. This will be clarified with EPOL WG.

The next meeting will take place on the 18th of September

Minutes reported by Sofia Kostoglou on 2025-09-10.