| ISOLDE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Supervisor last week |  |  | Alberto Rodriguez |  |  |  |  |  |  |
| Machine Supervisor this week |  |  | Erwin Siesling |  |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |
| GPS | Yes/No | HRS |  | Yes/No |  | HIE-ISO |  |  | Yes/No |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |
| GPS | % | HRS |  | 99 % |  | HIE-ISO |  | 83 % |  |
| Facility Status |  |  |  |  |  |  |  |  |  |
| Summary | Good week in HRS with the successful completion of experiment IS775. Very difficult week with several non-performing targets in the GPS / REX/HIE-ISOLDE side. HRS: - Continuation of experiment IS775 (98-100Cd+ beams to MIRACLS) until 27.08. - MEDICIS target (#865) irradiation (27-28.08). - New target (#901) installed (27.08) in preparation for the next physics experiment (IS686). GPS + REX/HIE-ISOLDE: - Preparation of a 226Fr57+ beam at 7.3 MeV/u from the GPS front-end to the XT03 line (IS581). Physics since 29.08. |  |  |  |  |  |  |  |  |
| Issues | - Limited production of 226Rn+ by the targets. Original physics goals not achievable. After discussion with the users, it was agreed to deliver a 226Fr56+ beam instead using a different target. This delayed the start of the experiment by ~ 22h. - Stability issues of the SRF16 cavity. Gradient had to be lowered, rest of the linac rephased and injection into the experiment redone. The beam energy decreased to 7.24 MeV/u (~2h). - Trips of the IH structure (30.08 and 01.09), the line heating (30.08) and the HT (01.09). - Issues with the buncher power level since 30.08 resulting in a poor transmission through the linac. Intervention by Luca Timeo on 01.09 restored the transmission. Follow-up intervention (02.09). |  |  |  |  |  |  |  |  |
| Plans | GPS + REX/HIE-ISOLDE: - Continuation of IS581 experiment until 02.09. HRS + REX/HIE-ISOLDE: - Preparation of IS686 experiment (108Sn to the ISS). Physics planned for 04.09). |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |
| Yes / No | Duration |  |  |  | Preferred date/time |  |  |  |  |
| Reason |  |  |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |  |  |