| ISOLDE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Supervisor last week |  |  | Emiliano Piselli / M. Lozano |  |  |  |  |  |  |
| Machine Supervisor this week |  |  | M. Lozano / E. Siesling |  |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |
| GPS | Yes | HRS |  | Yes |  | HIE-ISO |  |  | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |
| GPS | 99.0 % | HRS |  | 99.5 % |  | HIE-ISO |  | 100.0 % |  |
| Facility Status |  |  |  |  |  |  |  |  |  |
| Summary | Week #38 General: - Very good week at Isolde. GPS: - Target #783 (Cs external source) was installed on Wednesday afternoon. This is a prototype ion source consisting only of the line and a special system to extract the beam. With the support of the SY-STI target expert, we set up the ion source and were able to deliver beam to users by Thursday afternoon. - GLM users received 133Cs and 135Cs beams until Saturday morning for experiment IS721, without any issues. HRS: - On Tuesday, proton beam scan and yield measurements for the IS772 experiment to CRIS. - Beam to users from Tuesday evening (radioactive beam: ⁹⁹,¹⁰⁰In). - From Friday afternoon to Monday morning, we were authorised to temporarily increase the proton intensity limit from 2 µA to 2.5 µA (https://edms.cern.ch/document/3351732/1.0). REX/Hie-Isolde: - On Tuesday, we worked on preparing the 22Ne⁶⁺stable beam from REXEBIS to Miniball, and we delivered it to the Miniball users on Wednesday evening. The users continued with this beam until Thursday evening. |  |  |  |  |  |  |  |  |
| Issues | HRS: - We rebooted the HT FE computer twice, because the HT was tripping when protons hit the target and when we increased the proton intensity. REX/HIE-ISOLDE: - The REXEBIS cathode seems to be degrading, and F. Wenander is actively monitoring it. - 7GAP2 RF amplifier could not be restarted. After checking, L. Timeo (SY-RF) found that the low-pass filter for the filament current measurement was broken. It was replaced on Monday morning. Everything went fine and it is now working properly. |  |  |  |  |  |  |  |  |
| Plans | - GPS target change on Monday, after which we will work on the beam to be delivered to Miniball for IS702, foreseen for Friday. - IS772 will finish on Wednesday morning, followed by a target change on Wednesday. |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |
| Yes / No | Duration |  |  |  | Preferred date/time |  |  |  |  |
| Reason |  |  |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |  |  |