Minutes from the ISOLDE Technical Coordination meeting 02-12-2025

Link to ISOLDE Minutes https://edms.cern.ch/project/CERN-0000098288

ISOLDE Technical Meetings Indico: https://indico.cern.ch/category/11956/

ISOLDE Operation: https://isoldeop.web.cern.ch/

REX: https://isoldeop.web.cern.ch/rex-radioactive-beam-experiment
RILIS home page: https://isolde.cern/rilis

ISOLDE Web pages: https://isolde.cern/

ISOLDE Schedule: https://isolde.cern/isolde-schedule

MEDICIS web page: https://medicis.cern/

MEDICIS coordination: https://confluence.cern.ch/display/MEDICIS/MEDICIS+Home

Visits: https://publicoutreach.cern.ch/outreach/panel#agenda/today (use the drop-down menu to select the location ISOLDE).

Present: K. Chrysalidis, L. Di Giulio, A. Dorsival, C. Duchemin, L. Fadakis, O. Fjeld, L. Fraile, L. Lambert, M. Lozano, R. Martinez, S. Mataguez, E. Piselli, A. Rodriguez, S. Rothe, S. Stegemann, J. Vollaire.

- General announcement and follow-up of previous meeting

J. Vollaire welcomes the meeting participants and reviews the minutes from the previous ITC meeting. He highlights the tests carried out to validate the possibility of operating the QFO322 quadrupole magnet without water cooling by using a reduced duty cycle. He notes that the HIE-ISOLDE physics run has been terminated and that preparatory activities for the handover to the cryo team for the warm-up cycle are ongoing. Some important dates were recalled, including the end of the proton run at 08:00 on 08/12 (physics will stop after lunchtime, as long-lived isotopes will be delivered after the beam stop). He reiterates H. Heylen’s request to remove boxes and equipment from the material SAS.

J. Vollaire says that some pictures have been added to the Indico page by L. Lambert, who is requesting support for the identification of cables below the RILIS laboratory. The cables that are not properly placed inside cable trays will need to be disconnected and re-routed correctly for the RILIS LS3 upgrade (link to pictures).

J. Vollaire announces the presentation by Q. Vuillemin on the “Update on ISOLDE LE lines optics model”, which is scheduled just after the technical coordination meeting (link to the event).

The agenda of the present meeting is reviewed (link to the Indico page).

- Operation Report

L. Fadakis presents the operation report, noting the relatively low beam availability due to different technical issues (link):

GPS target and mass separator:

- COLLAPS users concluded the experiment (LOI171 – 17-19F at 30kV) on the 26/11 instead of the 27/11 due to different technical problems on the machine side.

- On Thursday 27/11, the target #567 Sn VD7 target (molten metal) was installed. The molten tin target must be operated with specific beam conditions and the temperature monitored closely.

- On Friday 28/11, tuning was done at 30 kV to the GLM collection chamber with the stable 136Xe beam.

- As radioactive collections handling can only be performed with HSE-RP support, the first collections were performed on Monday 01/12.

- For the issues, a line heating trip occurred on the 27/11 and a target heating trip on the 01/12 (a quick reset with restart at the middle of the ramp could be performed).

HRS target:

- The target change for the #916-UCx-LIST could not be completed as planned on Tuesday 25/11 due to a problem with the robot. It was finalized Wednesday 26/11 during the morning after an intervention in the target area to fix the robot.

- On Thursday 27/11, the OP team could perform the beam tuning with 58Ni to IDS(RC4) and the beam was given to the users. Users have been taking beam since then.

- Other issues encountered during the week:

  - A target trip occurred on the 26/11.

  - A power glitch (27/11) impacted the accelerator complex including many ISOLDE systems.

  - The ISCOOL RF Amplifiers needed reboot after the power glitch on Thursday 27/11.

  - One HT Trip is also reported.

  - Two Faraday cups were not working (YRC4.BFC0900 and YRC4.BFC5100) as well as the scanners YRC0.BSC0800 and YRC2.BSC0800. The scanners issue can be mitigated by selecting the “STOP_NOW” and then “STOP”. Although the fault re-appears after some time. For the Faraday Cup, M. Nieto and M. Duraffourg have proposed to intervene on site. (Added after the meeting, it is proposed to schedule the intervention after the 15/12 as stable beam tuning was already completed to the IDS chamber).

Power glitch on Sunday 30/11:

- Possible power glitch at ISOLDE: the transformers for both GPS and HRS went off. In addition, the High Tension (HT) power supplies were off, and several circuit breakers had tripped. Most devices could be restored, but the timing module for the HT would not recover (the timing crate was necessary even for stable HT). An intervention for Monday 01/12 morning was arranged, as this is a best-effort service. The efficient intervention from T. Gharsa is acknowledged.

The possibility of including support for HT-related issues in the SY-ABT piquet service after LS3 is discussed. J. Vollaire will follow up with the concerned group to determine whether such a service is already available for other facilities and whether it would have the necessary expertise to intervene on ISOLDE systems.

J. Vollaire provides additional details on the robot issue encountered during the HRS target change on Tuesday 25/11. After retrieving the target and before installing the LIST target, the robot operator noticed that the gripper was not closing properly. After verifying the proper functioning of the GPS robot to exclude an issue with the compressed-air distribution, an intervention was scheduled for the following day, Wednesday 03/12.

The pneumatic actuator from the MEDICIS robot, as well as the switch support—which was found to be damaged on the HRS robot—were used to repair the HRS robot. However, the issue was not resolved. Further investigation revealed a problem with the compressed-air piping upstream of the HRS robot. After this was repaired, the target change could proceed. The MEDICIS robot was put back into service on Friday 28/11.

L. Fadakis mentions that M. Fraser (SY-ABT) had proposed a rematch of the BTY line optics that would allow delivering beam to HRS without requiring the QFO322 magnet. This would remove the limitation on the number of cycles delivered to HRS due to the absence of cooling for QFO322. It has been agreed with the PS Booster team that the rematched optics will be tested at low intensity on Thursday 04/12, in the shadow of the change of scheme for RILIS.

(Added after the meeting: as the test required changing the polarity sign in a few converters, and given the limitations of the current power converters and electronics, it was decided not to take the risk of changing the configuration for the test.)

- RILIS Report

K. Chrysalidis reports that the RILIS team is currently running an in-source laser spectroscopy experiment with the LIST ion source. The nickel scheme was available from the previous HIE-ISOLDE run, and thanks to a collaboration with the CRIS team, light is being delivered to RILIS from their setup via an optical fibre. Hopefully, good statistics will be obtained for 69Ni; however, more exotic nuclei seem to be out of reach. It is noted that nickel is the lightest mass tested with the LIST/PI-LIST target, and the resolution is at the limit for laser spectroscopy. K. Chrysalidis adds that testing the target in offline2 was challenging due to the state of the lasers and the infrastructure limitations in the offline2 laboratory.

- MEDICIS (link) Report and Building 179 (Class A laboratory)

C. Duchemin reports on behalf of the MEDICIS team. The last 155Tb run of the year took place last week using the new target material developed by P. Kalnina (TaC–carbon nanotubes). This week, the ThO2 target from SCK-CEN has been re-installed on the Frontend as part of L. Gubbels’ PhD work on 225Ac production. In parallel, target #805M (vanadium foils) is being parasitically irradiated for a new 47Sc run. The collected 47Sc will be shipped to Latvia for the first time. It is noted that a target will be installed in the irradiation point (indirect irradiation) to benefit from the last “indirect” protons before the beam stop on 08/12. The objective is to build up the activity of long half-life radionuclides in preparation for operation in 2026.

Finally, C. Duchemin reports that an external source of 153Sm from POLATOM will be received on Friday 05/12. The sample will be used for a final mass separation of this radionuclide, which will then be shipped to Heidelberg, Germany for pre-clinical tests.

L. Lambert says that the gas point for Building 179 will not be accessible for a few days due to the IBDRS construction work. Access through the worksite can be arranged if required for operations.

- Physics, Schedule and Targets operation and production

H. Heylen reports on physics (information provided on the Indico page). On GPS, the goal was to measure the mean square charge radius of 17F via laser spectroscopy at COLLAPS. A development target was used, meaning that the yields and the level of contamination were uncertain beforehand. COLLAPS also employed a new experimental setup. In general, they faced several challenges, including higher-than-expected stable contamination and a number of interruptions due to issues at ISOLDE. After recording a first measurement of one 17F peak, they concluded that a full hyperfine spectrum would be out of reach and stopped the run one night before the planned end. Nevertheless, they did not leave dissatisfied, as they gained valuable knowledge that will help them tackle this case successfully after LS3. L. Fraile adds that the technique used is very promising and will be further optimized in the future.

L. Fraile reports that since Monday 01/12, a run with 111mCd from GPS has started for perturbed angular correlation spectroscopy with ions implanted in different solids. Several groups are interested in the rich measurement program planned during the week (collections performed during the day). A. Dorsival notes that the collected activity during the collections on Monday 01/12 was lower than expected. S. Stegemann explains that the yield measurements and tuning for the target performed on Friday 28/11 were not done with the tape station, as the central beam line was not available. It was retuned in the morning, and the activity of the second collection already appears higher. It is noted that the source temperature can be further increased, if needed, to improve the yield as an alternative to increasing the proton intensity.

S. Stegemann provides more information on the development target used for the COLLAPS experiment. This development was carried out in collaboration with colleagues from SCK-CEN to produce SiC material adapted for RIB production. Such material with the appropriate properties could be purchased in the past, but the supplier has discontinued its production. The required properties include temperature stability, particle size suitable for isotope release, and high purity—particularly a low level of aluminium contamination to prevent the formation of aluminium fluoride compounds. Pre-treatment of the material was required to reduce aluminium contamination, but further development will be necessary to achieve past performance levels.

On HRS, neutron-rich Ni beams are being studied via laser spectroscopy in the context of nuclear structure evolution around N = 40. A special LIST target was used to reduce the expected Ga contamination, although this also lowers the Ni yield. The PI-LIST mode allows for high-resolution in-source laser spectroscopy, which is important for medium-mass nuclei such as Ni. With yields orders of magnitude lower than expected and several operational issues, the full programme will be out of reach, but it appears that new physics results may still be obtained for 69Ni. S. Stegemann notes that the yields used to plan the experiment were taken from an old PhD thesis, where the line temperature was 2400 °C, which is known to be at the operational limit for the target. K. Chrysalidis confirms that with the upcoming bismuth run, the target needs to be operated in a conservative regime.

S. Stegemann concludes by saying that a review of the targets operated during the run will be performed. Statistics on the number of protons received will be facilitated by the virtual device that publishes the target number to Timber throughout the year. He also notes that the detector in the Tape Station will need to be calibrated and returned to its owners (n_TOF colleagues).

K. Chrysalidis enquires about the possibility of dismantling a LIST target in the hot cell. J. Vollaire explains that uranium-carbide targets are not yet included in the dismantling campaign, but if a tantalum LIST target was operated, it can be included for autopsy.

- Mechanical work and support to the users (link)

O. Fjeld reports on the activities performed during the previous week and on new requests received. During the previous week, rubber gaskets were produced for the solid-state physics team, and 3D printing of covers for the IDS germanium detectors began (15 pieces required). A new chemical cabinet was received and installed. For the coming days, a request has been made by the VITO team for the machining of very small parts. Support will also be provided to the Miniball team for the dismantling of their setup. J. Vollaire confirms that O. Fjeld will follow up on the production and installation of the “ISOLDE” signs on the buildings (Jura side).

- Safety, AOBs and visits

J Vollaire reminds the participants that scaffolding will be installed at the entrance of the hall (Jura side) as of 08/12. It is required for the cable installation for the building 197 extension. The passage will be blocked during the installation and removal of the scaffolding. The crane will not be available during the week.

L. Di Giulio summarizes the activities planned for the installation of the flammable-gas lines. As of 03/12, piping work will start in the HIE-ISOLDE part of the hall. The core drilling through the wall was successfully completed during the previous week, and cabling activities are well advanced. Close coordination has been implemented with the teams organizing visits.

- Safety, AOBs and visits

| ITC date | Description | Group(s) | Status |
| --- | --- | --- | --- |
| 07/11/2023 | Stable beam commissioning in 2024 done with the HT timing structure of proton runs (assess effect on target/line heating trips). | BE-OP, SY-ABT, SY-EPC | To be followed |
| 10/05/2022 | Automatic reset of the gas system PLC when the rack is off for a target change. Labelling of different cables (thermo-couple…) | BE-CEM SY-STI | Followed by BE-CEM |
| 02/08/2022 | Contact experts (RF and timing) to have an adjustable delay between the proton pulse and post-accelerated beam (TRAP/EBIS timing and synchronization with RF) | BE-OP-ISO | Deployed but not tested in all conditions |
| 23/08/2022 | Write a memorandum to include TT70 with other ISOLDE primary areas in IMPACT | SY-STI-RBS | J. Vollaire to follow |
| 22/11/2022 | Follow-up on the request by the cryo group to consolidate the gas recuperation line of the ISS magnet | SY-STI EP-SME | E. Siesling is following |
| 16/05/2023 | Produce an electronic card for the HRS beam gate system to replace the one loaned by ISOLTRAP colleagues | BE-OP-ISO / STI | To be included in general Beam Gate discussion 11/12 meeting. |
| 27/06/2023 | Organize a meeting with HSE-RP, TE-VSC and other concerned colleagues on the strategy for the recovery of the gases from the RFQ vacuum sector. | HSE-RP TE-VSC SY-STI, BE-OP | Decision to be taken after oil sampling campaign early 2024. |
| 27/06/2023 | Follow-up on the installation of a lift in the stair of building 508 (access for disabled persons) | SY-STI | J. Vollaire created a ticket. S. Freeman will follow with SCE management |
| 26/09/2023 | Installation of a sink at the entrance of building 170 (Jura side) | SY-STI | J. Vollaire to follow up on the request. Location to be clarified. |
| 13/02/2024 | Installation of a local demineralized water production unit in building 508 (location to be confirmed) | SY-STI EP-SME |  |
| 25/06/2024 | VAC interlock for collections chambers in GLM/GHM | SY-STI, BE-OP, TE-VSC |  |
| 25/06/2024 | Produce a spare water pipe for the 7GAP1 water cooling (supply) | BE-OP and EN-CV |  |
| 09/07/2024 | Validate HRS HT door safety function (locking mechanism) and implement systematic annual check | SY-STI | To be finalized |
| 09/07/2024 | Compressed air tubes consolidation (behind ISCOOL HT room) | SY-STI, TE-VSC | To be done |

Next meeting will be on the 09/12/2025 (last one of the year !!!!).

Minutes by J. Vollaire