| LHC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  | X. Buffat, M. Solfaroli |  |  |
| Machine Coordinator this week |  |  | H. Timko |  |  |
| Statistics |  |  |  |  |  |
| Availability | 67.4% |  |  | Stable Beam Ratio | 51.7% |
| Facility Status |  |  |  |  |  |
| Summary | Monday morning, ALICE lowered the solenoid to 12 kA. The LHCb vertical IP shift of -50 microns was applied. Before Tuesday morning, no sizable production, fills dumped by MSD of B1, QPS in S67, spurious collimators interlocks (TCTPV in point 2 - again !) and a corrector failure leading to a SIS dump on DOROS BPMs. After a second fault on the MSD B1, access for EPC and for SND emulsion exchange Wednesday morning. After a night of operation, failure of the point 2 cooling tower (water level sensor) leads to a cryo stop of around 24 hours. Operation restarted over Thursday night with two fills dumped by a QPS trip (RQT13.L2B1) and by the trip of S81, following an electrical glitch. Loss of a cryo turbine in point 8 on Friday afternoon, caused by a sudden drop of a temperature reading (suspected SEU). Quick recovery, but during the ramp-down an uncorrelated problem was noticed on XL8 PIC. All circuits appeared powered without the PC permit. It was traced to an issue in the supervision layer of the PIC and it required an intervention in P8 to exchange a board. Hectic weekend with repetitive short fills, interrupted by different dumps: QPS (RB.A78 & 3xRCBXH3.L1), RF quench, two PIC events (card to be exchanged in XL8 and dump without any circuit trip). |  |  |  |  |
| Issues | Power module to be exchanged on RCBV32.L1B2 |  |  |  |  |
| Plans | Continue physics production |  |  |  |  |
| Intervention Request |  |  |  |  |  |
| Yes / No | Duration |  |  | Preferred date/time |  |