| SPS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  |  |  | Panos Zisopoulos |  |  |  |  |  |
| Machine Coordinator this week |  |  |  |  | Giulia Papotti |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |  |
| LHC | Yes |  | NA |  | Yes |  | AWAKE | No | HiRadMat | No |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |  |
| LHC | 95.5% |  | NA |  | 91.1% |  | AWAKE | % | HiRadMat | % |
| Facility Status |  |  |  |  |  |  |  |  |  |  |
| Summary | It was a busy and challenging week at the SPS, with repeated transverse damper faults disrupting LHC fills and degrading NA beam quality, as well as an urgent EPC intervention on the BEF filter of the 18 kV network that caused additional downtime. Despite these issues, physics delivery continued to LHC and North Area, while providing beams for several MDs. W38 ended on a positive note with smooth LHC fillings over the weekend and uninterrupted FT physics. Weekly summary - Mon–Tue: - Repeated ADT H1 damper module trips which interrupted normal operation. Impact on beam delivery was reduced by adjusting chroma and BLM thresholds. An RF access allowed amplifier replacement (~1 h), but damper trips persisted. A further intervention traced the fault to a PLC contact causing voltage overshoot. - MKQH & MKQV tune kickers at fault; Issue resolved after expert restarted FEC - SPS was again in access for a couple of hours due to fire alarms in BA1, during which a water leak was discovered in the water distribution pipes near MBA.10590 - MSE6 septum water pump fault followed by a broken shaft, intervention to be scheduled next week. - Short parallel MDs on SHiP cycle optimization and a successful low-energy extraction test to HiRadMat. - Wednesday: - Stable operation with scheduled intensity adjustments (T4 raised to 55e11 ppp, later reverted to 39e11 ppp due to radiation alarms). - BDF target checks in preparation for the dedicated SHiP MD in W39. - Short parallel MDs on the new SPS chroma application and optics measurements with orbit bumps in extraction sextupoles. - Thu–Fri: - Long parallel MD on Thursday with LIU standard beams, followed by losses at BLM 42620 . Aperture measurements in Sextant 4 showed no restrictions. - Urgent EPC intervention on the BEF harmonic filter led to 2 h downtime. A later check identified a faulty card on the second compensator (BEQ2), which was resolved without downtime. - HiRadMat and ECN3 accesses. - Weekend: - Smooth operation with recovering availability and a very successful LHC fill with no lost injections. |  |  |  |  |  |  |  |  |  |
| Issues | - ADT H1 transverse damper module - BEF harmonic filter - BEQ2 compensator electronics card fault - MSE6 water pump shaft failure - MKQH & MKQV tune kicker FEC failure - Vacuum pump VPIA22501 fault - TT40 patrol search box communication loss |  |  |  |  |  |  |  |  |  |
| Plans | Next week: Mon-Tue-Fri: Short parallel MDs Wed: Dedicated MDs for SHiP target test & TECA controller Thu: Long parallel LIU MD Fri: Short parallel MDs |  |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |  |
| Yes / No |  | Duration |  |  |  | Preferred date/time |  |  |  |  |
| Reason |  |  |  |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |  |  |  |