| PS Booster |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  | Abdel AKROH |  |  |  |
| Machine Coordinator this week |  |  | Gian Piero Di Giovanni |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |
| ISOLDE | Yes |  |  | PS |  | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |
| ISOLDE | 98.4% |  |  | PS | 98.4% |  |
| Facility Status |  |  |  |  |  |  |
| Summary | Tuesday: - BI2.SMV10 tripped and has been rebooted (short beam stop) - BR2.XNO816L1 module need to be exchanged impact on others operational converters, so access request done for 30 min intervention without beam. (No machine access needed). - EPC expert cleaned-up the FGC HW on the BTY.BVT116 Wednesday: - BE3.DHZ11L1 status not ok in working set (dev not ready) did not have impact on the beam - BE.BSW14L4 which pulsing randomly at a wrong value causing radiation alarm. Issue solved by the expert Swap the CCU PXCKCCU01A-CR000013 with the spare available PXCKCCU01A-CR000012 - Officially started HL-LHC Reliability Run measurement Thursday: - PSB TFB Gain H function had strange behaviour on Ring 4 when sending the Function through Function Generator (seen by Foteini) need to check what could be the cause of that. TFB gain setting is a ‘func_list’. So, it can technically be shorter than 1.2s, but no one shorted the function which might point out a bug with on the HW side. - Very impressive performance obtained on MD16823 (High Intensity ISOLDE) beam >5e13 ppp extracted. |  |  |  |  |  |
| Issues | BE.BSW14L4 and BI2.SMV10 |  |  |  |  |  |
| Plans | Continue to deliver beam to downstream machines |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |
| Yes/No | Duration |  |  | Preferred date/time |  |  |
| Reason |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |