| PS Booster |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Machine Coordinator last week |  |  | G.P. Di Giovanni |  |  |  |
| Machine Coordinator this week |  |  | F. Asvesta |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |
| ISOLDE | Yes |  |  | PS |  | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |
| ISOLDE | 98.1% |  |  | PS | 98.6% |  |
| Facility Status |  |  |  |  |  |  |
| Summary | It was a good week for the PSB with good availability and several activities. ISOHRS completed the 2.5 uA run on Monday and the rest of the week was dedicated to serving the downstream accelerators and facilities and MD users. The OP team continued the HL-LHC reliability run in the CPS complex and started the preparation of the LHC MD Block #3. Extraction line to PS steering adjustments done to reduce the losses for the TOF beam extraction at PSB, as they had increased in the last month. Several problems with the LHC filling on Wednesday afternoon as the second injection was interlocked at the PSB every few shots randomly, delaying the LHC filling which took more than 1h. Eventually, the issue was traced back to a PS MD user which, tripping the 10 MHz cavities in the PS, interlocked the PSB extraction via external conditions. The PS team managed to reproduce the issue in the following days. |  |  |  |  |  |
| Issues | Downtime was mainly dominated by a few stops due to Linac4. On Tuesday in the shadow of the CCDTL2 intervention in Linac4, a few experts profited from some interventions: - BR2.XNO816L1 was replaced by the EPC piquet, as it has not been working for a few days (no impact on operation). - The EPC experts cleaned the configuration of a few FGCs. - Following the stop due to the BE.BSW14L4 last week, the ABT experts attempted a replacement of the crate converter, but had issue with the configuration of the spare, so they reverted to the old converter. - Once the Linac4 restarted with beam after the intervention the distributor, BI.DIS10 had to be reset 3 times [6 mins downtime] On Wednesday an issue with Beam Imminent Warning (BIW) timer unit prevented ISOLDE chain from switching to beam mode when ISOLDE was ready to take beam. The experts were called and replaced the unit [51 mins downtime only to ISOLDE]. |  |  |  |  |  |
| Plans | Provide beam to the downstream machines and facilities for physics and MDs. |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |
| No | Duration |  |  | Preferred date/time |  |  |
| Reason |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |