| SPS North Area |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Facility Coordinator last week |  |  |  |  | J. Bernhard |  |  |  |  |  |
| Facility Coordinator this week |  |  |  |  | N. Charitonidis |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |  |
| H2 | Yes |  |  | H6 |  |  | Yes | K12 |  | Yes |
| H4 | Yes |  |  | H8 |  |  | Yes | M2 |  | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |  |
| H2 | 93.9% |  |  | H6 |  |  | 93.9% | K12 |  | 93.9% |
| H4 | 93.9% |  |  | H8 |  |  | 93.9% | M2 |  | 93.9% |
| Planned Target Sharing (ASM) |  |  |  |  |  |  |  |  |  |  |
| T2 | 30 |  |  | T4 |  |  | 55 39 18:00, 8 October | T6 |  | 50 |
|  |  |  |  | T10 |  |  | 21 |  |  |  |
| Facility Status |  |  |  |  |  |  |  |  |  |  |
| Summary |  | H2/H4/H8/M2/P42/K12: Good operation. H6: Good operation. All diagnostics reinstalled after end of e+ beams including the CEDAR detector. A different CEDAR device is installed now to help identify hardware issues with parasitic beam tests. |  |  |  |  |  |  |  |  |
| Issues |  | H6: Users reported low efficiency in the XDWC.041.488 delay wire-chamber. The device was raised 15 mm to avoid the aged low-efficiency spot in the middle successfully improving data taking. H8: DRD6 report issues with the beamline delay wire chambers showing inefficiency in the centre of the detectors XDWC.042.475/476 and XDWC.042.543/544. Long-standing issue (not just in H8). BI team will check what they can do for replacement or repair. Small issue with movement of XSCI.042.509, not moving back into the beam (motor issue), still being worked on. |  |  |  |  |  |  |  |  |
| Plans |  | H2: CMS HGCAL continues as parasitic user. ALICE FoCAL. H4: SND → LHCf. RD25 CALET as parasitic user. H6: DRD3 CASSIA finishes. ATLAS HGTD and DRD3 continue. H8: DRD6 IDEA DRC → LHCb. M2: AMBER (main) and MUonE (parasitic) continue. P42/K12: NA62 continues. Low intensity run for NA62 tentatively scheduled for 15 October, test next Wednesday. |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |  |
|  |  | Duration |  |  |  | Preferred date/time |  |  |  |  |