| ISOLDE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Supervisor last week |  |  | Erwin Siesling |  |  |  |  |  |  |
| Machine Supervisor this week |  |  | Alberto Rodriquez |  |  |  |  |  |  |
| Beam Scheduled |  |  |  |  |  |  |  |  |  |
| GPS | Yes | HRS |  | Yes |  | HIE-ISO |  |  | Yes |
| Beam Availability by Destination (AFT) |  |  |  |  |  |  |  |  |  |
| GPS | 99.5% | HRS |  | 98.8 % |  | HIE-ISO |  | 99.5 % |  |
| Facility Status |  |  |  |  |  |  |  |  |  |
| Summary | Report WK40: Monday 06/10/2025 General: - From technical as well as physics point of view a busy but good week. GPS/HIE-ISOLDE: - Continuation of the molecular beam (134SnS) out of the target and molecule broken up in the EBIS to deliver 134Sn34+ at 4.4 MeVu to the IS702 Miniball experiment at XT01 with an energy of 4.4 MeV/u. - The run stopped Friday-morning 03/10. Despite the Te, Sb and Ba contaminants the users were very happy with the number of 134Sn counts being even higher than expected. - Target change to #911 UC foreseen for Tuesday 06/10. HIE-ISOLDE preparations: - Preparations for the upcoming IS690 experiment on 11Be4+ at 5.4 MeV/u started Friday-afternoon 03/10. - Instabilities for several SRF cavities were observed. These were solved by SRF specialist Daniel Valuch – many thanks. - Stripping foils were installed in the HEBT lines XT00 and XT03 in view of the upcoming 11Be4+ run by BI (Paulo Martin, Miguel Nieto) with support from RP and VSC. HRS: - Preparation of HRS for the IS765 VITO experiment on K beams at 50kV since Monday 29/09. - Proton Scan and Yield checks done on Friday-morning 03/10 and beam delivery to the IS765 VITO experiment started Friday-afternoon. |  |  |  |  |  |  |  |  |
| Issues | REX/HIE: - The performance of the Electron Gun at EBIS is still decreasing but believed stable and sufficient for the upcoming 11Be without an intervention (estimate 10 days downtime) HRS: - HRS.QP30-NEG, part of the first quad of the HRS Front-End triplet after the target, developed a severe low resistance to ground and is out of order. With access to it not being possible (high levels of radiation and risks of contamination) an optics workaround is now in place to run HRS until the end of the physics campaign. - During the weekend on several occasions the HRS MAG60 separator magnet cycling got stuck from the application. A workaround was found, and the issue will be addressed this week. |  |  |  |  |  |  |  |  |
| Plans | - HRS run with K beams to the IS765 VITO experiment until next Wednesday 08/10. - Preparations of a HIE ISOLDE run for IS690 at XT03 on 11Be4+ at 5.4MeV/u starting end of wk 41 |  |  |  |  |  |  |  |  |
| Intervention Request |  |  |  |  |  |  |  |  |  |
| Yes / No | Duration |  |  |  | Preferred date/time |  |  |  |  |
| Reason |  |  |  |  |  |  |  |  |  |
| Impact |  |  |  |  |  |  |  |  |  |