# Linac4 Beam Coordination Committee - Meeting 36 held on 2 October 2012

Present:	Giulia Bellodi; Caterina Bertone; Jan Blaha; Christian Carli; Julie Coupard; Jan Hansen; Lars Jensen; Ioan Kozsar; Jean-Michel Lacroix; Jean-Baptiste Lallement; Luz Anastasia Lopez-Hernandez; Cesare Maglioni; Antony Newborough; David Nisbet; Jocelyn Tan; Ivo Daniel Vicente Leitao; Joachim Vollaire; Maurizio Vretenar; Thomas Zickler.

## Minutes of the last meetings

The minutes of BCC-35 are approved.

## Follow-up of action items

No update.

## LBE-LBS beam dynamics (JB. Lallement).

The beam dynamics and proposed layout for LBE and LBS lines is presented.

- LBS line for energy and energy spread measurement Layout is presented. Starting from BHZ.40 magnet, it is composed of 2 L4T type bending magnets (23.3°) with one focusing L4T type quadrupole in between to generate a waist at the profile monitor and one defocusing after the second bending to vary the dispersion (layout in the presentation), on profile monitor,  a BCT and a dump. The dispersion at the profile monitor can be varied from 2.87 m to 5.28 m. The expected rms beam size in case of low dispersion and debuncher cavity ON, is 1 mm. A direct estimation of the beam energy spread in that case (neglecting the beta function) would lead to an error of 2 % (rms 104 keV instead of 102 keV). In case of energy (painting) measurement, the resolution at the profile monitor is approximately 100 keV/mm. Some simulations were performed with the Linac3 beam. The setting of the buncher cavity phase could be done with the proposed configuration of the LBS line. The minimum measurable rms energy spread is 0.675 MeV (nominal 0.224 MeV).

- LBE line layout is presented. It is composed of two bipolar EMQs, 3 profile monitors, a BCT and a dump. The emittance will be measured via the 3 monitor method (see presentation), generating a beam waist on the central monitor. Beam dynamics simulations results are presented and show that the transverse emittances can be reconstructed neglecting the space charge effects. The beam dynamics design is compatible with the Linac3 beam parameters.

The quadrupole settings used to adjust the beam dynamics for measurements are listed in the presentation.  The max gradient is 1.92 T/m (0.58 T integrated).

### Discussion.

D. Nisbet and A. Newborough remark that the upgrade to 2 L4T type quadrupole magnets was not foreseen for the LBE line. JB. Lallement answers that we might accommodate the beam dynamics with the present LBE quads, but the gradient limitation and integration issues should be taken into account. A meeting to clarify the needs for new magnet for LBE will be organized in the coming weeks. T. Zickler adds that the 2 extra quads (if needed) should be order with the present L4T serie in order to limit the extra-cost.

M. Vretenar asks if the profile monitors are compatible with ions from Linac3. A dedicated meeting for Linac3 measurement with diagnostic group is foreseen on 12 October.

J. Hansen remarks that there are, for the moment, no vacuum requirements, and that should be discussed.

M. Vretenar asks what should be the field stability for the measurement. JB. Lallement answers that the energy swing (+/- 1.2 MeV) corresponds to a magnetic field variation of +/- 0.4% or to a displacement on the profile monitor of about 12 mm. In order to measure a beam size of 1 mm, corresponding to a field variation of 3.10-4, the field should be stable within +/- 1.10-4.

## Civil engineering (N. Lopez-Herandez).

Natacha presents the 4 different integration options studied by the civil engineering group:

- Solution1: LBS on the left going down in a trench, LBE on the right

- Solution2: LBS going up (dump inside the ceiling), LBE on the right.

- Solution3: LBS going up (dump inside the ceiling), LBE on the left.

- Solution4: LBS going on the left (measurement in horizontal plane).

From the civil engineering point of view, solutions 2&3 are equivalent and the less expensive. Solution4 is the most expensive, requiring a 14 m deep shaft. Solution 1 is an intermediate solution with a limited civil engineering work (approx. 2 months).

### Discussion

JB. Lallement says that the solution 4 is excluded by the beam dynamics, which require the spectrometer line to be in the vertical plane (because of the remnant horizontal dispersion in the line). M. Vretenar adds that being the most expensive option, this solution is excluded.

C. Bertone and C. Maglioni are not in favor of solution 2 and 3 (having the dump inside the ceiling) for matters of handling, supporting and assembling the dump and its shielding.

## RP studies (J. Vollaire).

The results of the radio protection simulations are presented. The residual dose rate at the level of beam pipe is less than 1 µSv/h after 1 hour cooling time during operation and commissioning phases. It would not exceed few µSv/h in case of accidental scenario. From RP point of view, the solution 1 seems to be the more practical. The LBE shielding still needs to be specified.

### Discussion

M. Vretenar remarks that the operational scenario considering 2*8 h/week is pessimistic and should be revised. A. Lombardi says that even if the operation scenario is pessimistic, the RP studies are mainly based on the commissioning scenario.

.

## Transport (C. Bertone).

Caterina presents the results of the study on LBE/LBS area access by dismantling the shielding wall with PSB. The wall is made of 300 concrete blocks of 1250 kg each, 30*500 kg steel beams, 635*18kg concrete bricks and hundreds of steel bricks. Dismantling the wall will require studies and tooling (50 kCHF) and approximately 1 month handling work (50 kCHF). The wall should be evacuated from the PSB side (preferred to PS side) having benefits from PSB tremie, short path, and limited need for handling means. The re-assembly of the wall after work still needs to be decided. Concerning the different options for LBE and LBS integration, the solution 1 is the preferred choice.

### Discussion

The solution 1 is approved to be the baseline solution (keeping sol 2 as backup). The total time needed for transport and civil engineering is estimated to 3 months. The impact on the PSB planning during LS2 has to be discussed and considered. The need for the complete re-assembly of the 6 meters thick PSB separation wall after the work in the area remains an open question (PS to PSB separation wall is 2 m thick).

## AOB.

No AOB.

Jean-Baptiste Lallement

Next meeting: Thursday 4 October 2012.