| conceptual SPECIFICATION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INNER TRIPLET Q1 and Q3 cold mass MQXFA |  |  |  |  |  |  |  |
| Equipment/system description The inner triplet quadrupoles are the magnetic system used to squeeze the beta functions around the interaction point, allowing to reach values in the IP of the order of 10 cm. The triplet is made of three magnets Q1, Q2 and Q3. Here we deal with the Q1 and Q3 magnets, which have many features in common with Q2 described in a separate document |  |  |  |  |  |  |  |
| Layout Versions |  | LHC sectors concerned |  |  | CDD Drawings root names (drawing storage): |  |  |
| V X.X |  | S1-2, S4-5, S5-6, S8-1 |  |  | TBD |  |  |
| Traceability |  |  |  |  |  |  |  |
| Project Engineer in charge of the equipment G. Ambrosio |  |  |  | WP Leader in charge of the equipment E. Todesco, G. L. Sabbi |  |  |  |
| Committee/Verification Role |  |  |  |  |  | Decision | Date |
| PLC-HLTC/ Performance and technical parameters Configuration-Integration / Configuraration, installation and interface parameters TC / Cost and schedule |  |  |  |  |  | Rejected/Accepted Rejected/Accepted Rejected/Accepted | 20YY-MM-DD 20YY-MM-DD 20YY-MM-DD |
| Final decision by PL |  |  |  |  |  | Rejected/Accepted/Accepted pending (integration studies, …) | 20YY-MM-DD |
| Distribution: N. Surname (DEP/GRP) (in alphabetical order) can also include reference to committees |  |  |  |  |  |  |  |
| Rev. No. | Date |  | Description of Changes (major changes only, minor changes in EDMS) |  |  |  |  |
| X.0 | 20YY-MM-DD |  | Description of changes |  |  |  |  |

# Conceptual description

## Scope

The inner triplet quadrupoles are the magnetic system used to squeeze the beta functions around the interaction point, allowing to reach values in the IP of the order of 10 cm. The triplet is made of three magnets Q1, Q2 and Q3. Here we deal with the Q1 and Q3 magnets, which have many features in common with Q2 described in a separate document.

## Benefit or objective for the HL-LHC machine performance

The large aperture of these magnets (150 mm) allows reaching a beta function in the IP of the order of 10 cm.

## Equipment performance objectives

These magnets will provide 140 T/m in the transverse plane, with operational temperature of 1.9 K.

TECHNICAL ANNEXES

# preliminary technical parameters

## Equipment Technical parameters

The Q1 and Q3 magnets are made of two structures back to back in a single cold mass. The following parameters are for Q1 or Q3 cold masses. Each structure within a cold mass provides half magnetic length.

Table 1: Equipment parameters

| Characteristics | Units | Value |
| --- | --- | --- |
| Aperture | mm | 150 |
| Number of apertures |  | 1 |
| Distance between apertures |  | NA |
| Cold mass outer diameter | mm | 630 |
| Magnetic length | m | 4+4 |
| Approximated weight | t | 22.5 |
| Gradient | T/m | 140 |
| Peak field | T | 12.1 |
| Current | kA | 17.5 |
| Temperature | K | 1.9 |
| Loadline margin | (%) | 20 |
| Overall current density | A/mm2 | 490 |
| Stored energy per meter | MJ/m | 1.32 |
| Differential inductance per meter | mH/m | 8.2 |
| Stored energy | MJ | 10.6 |
| Differential inductance | mH | 66 |
| Superconductor |  | Nb3Sn |
| Strand diameter | mm | 0.85 |
| Cu/No Cu |  | 1.2 |
| RRR after nominal reaction |  | >150 |
| Superconductor current density at 15 T, 4.2 K | A/mm2 | 1400 |
| Number of strands per cable |  | 40 |
| Cable bare width before reaction | mm | 18.15 |
| Cable bare mid thickness before reaction | mm | 1.525 |
| Keystone angle | degrees | 0.55 |
| Insulation thickness per side at 5 MPa | mm | 0.15 |
| Number of layers |  | 2 |
| Number of turns inner layer |  | 17+5 |
| Number of turns outer layer |  | 16+12 |
| Cable unit length per coil | m | 430 |
| Coil physical length per structure | m | 4.3 |
| Magnet physical length ( 2 structures) | m | 9.5 |
| Cold mass weight | T | 14.6 |
| Heat exchanger hole diameter | mm | 77 |
| Heat exchanger angle | degrees | +/- 45 |
| Heat exchanger distance from centre | mm | 227.5 |
| Fx / Fy (per octant) | MN/m | +2.65 / -3.87 |
| Fz (whole magnet) | MN | 1.4 |
| Stress on the mid-plane | MPa | 160 |
| Cold bore thickness | mm | 4 |
| Gap cold bore to coil | mm | 1.5 |
| Fringe field on the cryostat outer surface | mT | 40 |

## Operational parameters and conditions

They will operate under a peak heat load due to the debris shower of 2.5 mW/cm3, withstand 25 MGy of radiation dose, and have a cooling power of 110/160 W on Q1/Q3 and 150/160 on the beam screen.

## Technical and Installation services required

Table 2: Technical services

| Domain | Requirement |
| --- | --- |
| Electricity & Power | 20 kA circuit |
| Cooling & Ventilation | 160+210 W cooling power for the magnet (110 W operational plus 50% margin) 210 W cooling power for the beam screen (150 W operational plus 50% margin) |
| Cryogenics | 1.9 K cooling through heat exchangers |
| Control and alarms |  |
| Vacuum |  |
| Instrumentation | Voltage taps for quench detection |

Table 3: Installation services

| Domain | Requirement |
| --- | --- |
| Civil Engineering |  |
| Handling |  |
| Alignment | Cold mass alignment in the cryostat within 0.1 mm (TBC) and 1 mrad (TBC) |

## P & I Diagrams

## Reliability, availability, maintainability

If the magnet is not providing the gradient, in any one of the magnets, the machine cannot operate. A lower than specified cooling power can limit the peak luminosity to values lower than nominal. A lower than specified radiation resistance can break the magnet before the gathering 3000 fb-1 in HL-LHC.

## Radiation resistance

The equipment is designed to withstand 25 MGy.

## List of units to be installed and spares policy

Four Q1 and four Q3 cold masses to be installed, one per side at IP1 and IP5.

Two spare cold masses (one Q1 and one Q3).

# preliminary CONFIGURATION and installation constraints

## Longitudinal range

Q1a: from 23.0 to 27.0 m from the IP.

Q1b: from 27.5 to 31.5 m from the IP.

Q3a: from 54.5 to 58.5 m from the IP.

Q3b: from 59.0 to 63.0 m from the IP.

## Volume

630 mm diameter and 9 m length: 2.8 m3.

## Installation/Dismantling

The present triplet, orbit correctors, non-linear correctors, and D1 need to be dismantled.

# preliminary INTErface parameters

## Interfaces with equipment

## Electrical interfaces

Table 4: Circuits to be generated

| New circuit description | Circuit LHC code name (if known) | Approx. current rating (if known) | Approx. voltage rating (if known) |
| --- | --- | --- | --- |
| RQXFA.L1 |  | 20 kA |  |
| RQXFA.R1 |  | 20 kA |  |
| RQXFA.L5 |  | 20 kA |  |
| RQXFA.R5 |  | 20 kA |  |

# Cost & Schedule

## Cost evaluation

Project code 91114

## Conceptual schedule

Table 6: Conceptual schedule

| Phase | 2012 |  | 2013 |  | 2014 |  | 2015 |  | 2016 |  | 2017 |  | 2018 | 2019 | 2020 | 2021 | 2022 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conceptual design |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engineering design (short) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Short model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Engineering design (long) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Prototype |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Production |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Installation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Note the following definitions:

- Conceptual design: choice of aperture, gradient, cable, cross-section and coil heads design

- Engineering design: CAD design of components and tooling

- Short model: first coil winding, assembly and test of short model

- Prototype: first coil winding, assembly and test

- Production: including test

# Technical reference documents

Future magnet report

# APPROVAL PROCESS comments FOR VERSION X.0 of the CONCEPTUAL SPECIFICATION

## PLC-HLTC / Performance and technical parameters Verification

Comments or references to approval notes. In case of rejection detailed reasoning

## Configuration-Integration / Configuraration, installation and interface parameters Verification

Comments or references to approval notes. In case of rejection detailed reasoning

## TC / Cost and schedule Verification

Comments or references to approval notes. In case of rejection detailed reasoning

## Final decision by PL

Comments or references to approval notes. In case of rejection detailed reasoning