Quench experiment proposals from TUT for the Roebel cable

Tiina Salmi, Antti Stenvall, and Erkki Härö, 12.5.2015

These quench tests can be done first for a single Roebel cable (excluding the transverse propagation in item 3.). After that, the tests can be repeated on a coil. The experience from the cable test will be useful in the coil test preparation. It will be also interesting to see the difference in the results.

- Detection time test with quench propagation velocity measurement

Test procedure: Operate the cable at constant current. Launch spot-heater and measure voltage evolution from the induced normal zone. Before launching the heater, the time delay to protection is pre-set to a maximum safe value. This value can be computed conservatively assuming quench immediately after heater activation. If the time seems to be too conservative, it can be increased during the test. Erkki Härö can compute this maximum safe delay time (needs Ic data of this cable).

This test should be repeated at various operation currents (temperature margins) and operation temperatures. Also the impact of different heat pulses can be evaluated.

Note: One should pay special attention to positioning the voltage taps (this is related to item 3 of this report, but the topics raised in item 3 should not distort this measurement). For example, if the heater covers half of the twist pitch, the one pair of voltage taps should be attached to the tape that is below the heater and other to the one which does not touch the heater at all. This way information about current and temperature diffusion within a cable can be attained qualitatively (whether this plays some role or not).

Motivation: The voltage evolution after quench onset can be used to estimate the detection time when using different voltage thresholds at different operation conditions. This test gives also the spot-heater time delay time to induce a quench, i.e., the time between heater activation and first voltage signal from the coil/cable. This can be used for validating / calibrating the simulation codes. This test also gives the propagation velocity.

- Presence of AC-loss test

Test procedure: This is a similar test than the Detection time test, but the heater is launched immediately after ramping up the magnet current. Different ramp-rates can be used. It can be assumed that the ramping up causes AC-losses in the cable. The presence of the heat by AC-losses can be seen in the spot-heater delay time and quench propagation velocity.

To do this test, one must have confidence that a natural quench during the ramp is safely protected.

- Current diffusion within the cable and transverse quench propagation

Test procedure: This test requires a spot-heater that is in contact to the wide side of only one strand. After the heater activation voltage is measured across the heated strand, and the strand that is in the opposite side of the cable. Also voltage can be measured across a strand in the adjacent cable to detect turn-to-turn propagation – if this kind of an arrangement is possible.

Motivation: Gives information how fast the quench spreads (or current and heat diffuse) between the strands in one cable, and across the coil turns.

- Maximum safe hotspot temperature test (destructive)

Test procedure: This is a destructive (or at least degradative) test and must be the last item in the test plan. The idea is to induce a quench with the spot-heater, and repeat this test with gradually increasing the protection delay (and hotspot temperature). For each quench, one computes the hotspot temperature based on the quench load (MIITs) – temperature relation. Between each spot-heater induced quench, a control quench is performed using regular delays. The control quench seeks for signs of degradation in the critical current. The shortening of the spot-heater delay time can indicate damage.

When damage is observed the test ends. The quench load associated with the previous quench may have resulted into a too high hotspot temperature. The highest hotspot temperature that did not cause degradation could be considered a maximum safe hotspot temperature. Of course, more tests can be done with regular delays to assess the level of degradation, or more high temperature quenches performed to assess the damage related to higher temperatures.

Motivation: The maximum hotspot temperature is the basis of the quench protection design. It is very important to know this value.