| MEETING MINUTES – IN WORK (V0.1) |  |
| --- | --- |
| Subject: | 16T Magnet Technology meeting #03 |
| Date and Time: | 2015-11-18, from 10:00 to 12:00 (~ 2.0 h) |
| Place: | 30-6-019 |
| Participants: | Present = , Excused = Bernhard Auchmann, Hugo Bajas, Marta Bajko, Amalia Ballarino, Michael Benedikt, Luca Bottura,Shlomo Caspi, Gijs De Rijk, Paolo Ferracin, Susana Izquierdo Bermudez, Friedrich Lackner, Rafal Ortwein, Juan Carlos Perez, Marco Prioli, Etienne Rochepault, Daniel Schoerling (Secretary), Ezio Todesco, Davide Tommasini (Chair), Arjan Verweij, Jeroen Van Nugteren, Felix Josef Wolf, Frank Zimmernann |
| WBS: | 1.6.1 |
| File Location/Link: | https://edms.cern.ch/document/1552201 |
| Indico link: | https://indico.cern.ch/event/453021 |
| Link to previous minutes: |  |

Agenda

1.	Introduction	1

2.	ERMC conductor properties and procurement strategy	2

3.	CCT – An option for 16 T dipoles?	2

4.	Technical challenges and solutions of FRESCA-2	3

5.	Review of mechanical tests on wires and cables in the different laboratories	3

6.	AOB	3

# Introduction

D. Schoerling opened the 3rd 16T Magnet Technology Meeting at 9:00.
The minutes of the last meeting are approved.

# ERMC conductor properties and procurement strategy

A. Ballarino presented in the first part of her talk the ERMC conductor properties and the procurement strategy. For ERMC wire with specifications similar to Fresca-2 (though with a reduced Cu/Sc ratio to 1:1) will be ordered (50 km PIT and 50 km RRP wire). The invitation to tender documentation can be prepared until December 2015, offers will be received until February 2016 and the conductor will be delivered until February 2017. This would however allow winding only by late Spring 2017, incompatible with the initial ERMC schedule.  Therefore, a twostep process for the winding of the ERMC coil was proposed.

First, Fresca-2 wire can be used to wind the first coils, due to the larger Cu content the field reached with this wire would be slightly lower if the coil size is unchanged. J.-C. Perez and S. Izquierdo Bermudez will explore the possibilities and the impact on the design for the two different conductors (Fresca-2 with Cu/non-Cu of 1.25 and ERMC with Cu/non-Cu of 1). A. Ballarino will contact the suppliers of the wire to check if they have short unit lengths of around ≥250 m available. This wire was maybe produced during the Fresca-2 production but at that time did not fulfil the requirements of the unit length and might be still available.

A. Ballarino presented in the second part of her talk the target values and the development program for Nb3Sn wire reaching the ultimate FCC targets. In the first stage of the development the programme will be focused in reaching the Non-Cu Jc of 1500 A/mm2 at 16 T and 4.5 K with effective filament diameter of ≤60 µm and a RRR of around 150. So far, Russia, Japan, Europe are included in the development and with Korea discussions have started.

A US contribution is highly desired and under discussion.

G. De Rijk asked whether the Bc20 can be raised as this would be highly beneficial. A. Ballarino remarked that this is under discussion but challenging. E. Todesco raised the question if not the specification for the Jc should be targeted towards the peak field value including the margin (i.e. in a range of 18T instead than 16T). D. Tommasini remarked that the values of margin are preliminary and the here presented values should give direction for development but are not final. L. Bottura stated that not many places (excluding also CERN) can measure at field values beyond 16 T. G. De Rijk commented that there is a need of upgrading CERN measurement possibilities at 163 by using a solenoid with up to 20 T field. A. Ballarino stated that an upgrade for measuring up to 20 T is indeed envisaged.

# CCT – An option for 16 T dipoles?

S. Caspi presented some considerations on design and cost aspects of a 16 T canted-cosine theta (CCT) for the FCC. The CCT is still an untested concept : so far two NbTi canted dipoles have been tested reaching 2.5 T and 4.6 T, and a 10T Nb3Sn is under construction at LBNL. The presented design reaches 16 T short sample bore field at 4.5 K and around 17.5 T at 1.9 K. D. Tommasini remarked that as a baseline –for allowing to compare different design types- a margin of 10% at 4.5 K and 20% at 1.9 K shall be envisaged. L. Bottura remarked that there is very little Cu in the outer layers yielding to an unacceptable increase in temperature in case of a quench. S. Caspi remarked that the Al-bronze spar may contribute to dump the stored energy, he will anyway update his design taking the margin and protection into account.

# Technical challenges and solutions of FRESCA-2

G. De Rijk’s talk was postponed to the next meeting due to time restrictions.

# Overview of mechanical test tools on wires and cables

F. Lackner presented the status of Task 2 of the FCC technology companion programme. The first task is the critical review of the existing tools. The following labs were visited: 163, UNIGE, FNAL, PSI, AIT, USTEM, AuM. Twente University and NHFML will be visited soon. He reported in the first part of his presentation the available measurement devices/facilities in the different laboratories for wires, in the second part for cables and concluded his talk by showing first results of ANSYS studies on cable wind-ability.

A. Ballarino encouraged having a discussion with B. Bernardo on the cable wind-ability study.

L. Bottura suggested to prepare a document describing what has to be measured (wires, cables, cables in coils, etc.) and which information has to be derived.

# AOB

No AOB.