CERN-ATS-YYYY-NNN

Crab Cavity SPS Test Program Day

Meeting Report

Editors: G. Arduini, R. Calaga, A. Castilla, R. Jones

Abstract

The 1st meeting to discuss the crab cavity test program foreseen during the run of 2018 was held on 11 January 2016 at CERN. During the half day meeting, the scope, potential RF & beam measurements, instrumentation, SPS-LSS6 operational constraints, vacuum and aspects related to transverse damper were presented and discussed. A summary of the meeting including the discussions is presented in this report. A draft specification of the crab cavity test program is also derived to launch further studies with the aim of defining concrete goals of the test program including the main accelerator and beam parameters, instrumentation needs.

Presented at: Crab Cavity SPS Test Day, CERN

# 1st Crab Cavity SPS Test Day

Workshop Summary, Edited by G. Arduini, R. Calaga, R. Jones

1	Introduction	4

2	Session Summaries	4

2.1	Crab Cavity SPS Test Scope	4

2.2	RF Commissioning & Measurements	5

2.3	Beam Measurements	6

2.4	SPS Instrumentation	7

2.5	SPS-LSS6 & Compatibility with SPS MD/Operation	8

2.6	Vacuum Aspects & COLDEX Compatibility	9

2.7	Impact of SPS Transverse Damper	9

3	Preliminary Specifications & General Discussions	10

3.1	Preliminary Specifications	10

3.2	General Comments & Discussions	12

3.3	Open Issues and Action Items	13

3.4	Potential MDs for 2016-17	13

4	Summary of Action Items	13

# Introduction

The 1st meeting to discuss the crab cavity test program foreseen during the run of 2018 was held on 11 January 2016 at CERN. During the half day meeting, the scope, potential RF & beam measurements, instrumentation, SPS-LSS6 operational constraints, vacuum and aspects related to transverse damper were presented and discussed. A summary of the meeting including the discussions is presented in this report. A draft specification of the crab cavity test program is also derived to launch further studies with the aim of defining concrete goals of the test program including the main accelerator and beam parameters, instrumentation needs.The detailed participation list, scientific program along with the associated contributions are available at:

https://indico.cern.ch/event/463435/

# Session Summaries

Notes and discussions from the session are summarized in this section

## Crab Cavity SPS Test Scope

G. Arduini, R. Calaga

This talk presented the scope of the crab cavity [CC] tests, its general specifications, and goals of the beam physics test program. As a reminder, it was stated that Point 6 in the SPS has been chosen since mid-2015, to accommodate one cryomodule for vertical crabbing, which is foreseen to be installed during YETS before 2018. The energy range between 26-450 GeV is feasible in the SPS with specific coast energies (55, 120, 270 GeV) already existing for MDs. Intensities from pilot to nominal per bunch can be used. The cavity parameters were presented (see Table 1) in which the cavity aperture of 84 mm poses one of the strictest constraints for SPS operation.

The installation of the movable table has been considered for EYETS in 2017, special care needs to be taken for the table’s displacement, since the movement is foreseen to be done with liquid He at 2K to minimize the preparation time during MD and the time for the movement is approximately 20 min and the design of the table has been done so to allow the movement in remote with no access required

Preparatory MDs performed on natural emittance growth since 2010-15 were summarized with lowest growth seen at 120 & 270 GeV coasts at very low intensities (1×1010 p/b) and single bunches.

It was mentioned that coasts with high intensity beams cannot be performed at 55 GeV because the beams cannot be safely damped in the range 37 to 100 GeV. Specific instrumentation needs to study crab cavity failures, or to allow the use of high intensity beams was raised. It was identified the need to study the beam response to three different cases of cavity trips: RF trips, beam induced failures, and system/operator faults. No high intensity coasts can be performed either at 120 and 270 GeV due to the cyclic dead time in the BLM system when the SPS is operated in coast. The limit maximum number of batches (less than 4?) from the BLM response time in coast with high intensity beams is seen as a limitation due to dead time. Therefore, the tests with very high intensity beams might be limited to cycling beams or coasts at injection energy, or a solution must be found for the BLM dead time when in coast.

The interlocking of the LHC extraction elements needs to be specified in order to prevent the activation of the LSS6 extraction bump when the crab cavities are in beam. Action: OP?

Action (Machine Development Coordination): specify the exact intensity limit for coasts at 55, 120 and 270 GeV

Action (BI): Investigate the issue with the dead time of the BLM when the machine is operated in coast.

## RF Commissioning & Measurements

P. Baudrenghien

The RF measurement program for both commissioning and the long term MDs were reviewed, including the RF instrumentation used for measurements. A comparison between the LHC and SPS setup for the control loops and distances show that loop delay time is approximately similar. The layouts for the LHC and the SPS were shown. The RF commissioning will be performed with 1-12 bunches and low intensities. Phasing of the two cavities w.r.t to the main RF system has to be performed with closed orbit measurements around the ring for each cavity and together. These have to be validated by the HT monitor in the crabbing phase. A phase pickup in SPS (APWL), should be considered to monitor the crabbing voltage signal for an independent source of measurement. This signal should be available to the CC LLRF. The counter-phasing of the two cavities will also be tested in the same fashion to establish the cavity transparency with closed orbits and HT monitors.

The electronics used in the SPS tests in 2 years’ time will likely be different for the HL-LHC due to improvements in technologies, however, the principle will remain the same. It is anticipated that the RF noise level in the SPS will be more than in the LHC.

The amplifier power as a function of loaded Q (Ql), for ±2 and 0 mm beam offset was shown (at 3 MV, 1.4×1011 p/bunch, and 2 ns long bunches). The amplifier power as a function of beam displacement will not only be used to centre the beam, but it will be useful to also validate the specifications for the LHC for transient beam loading compensation. Measurement of the HOM spectral amplitude due to beam displacement with high intensities will also be needed to study impedance related aspects. The estimation of transverse emittance growth as a function of the CC RF noise, along with its mitigation with signal from the HT monitor and the transverse damper was discussed as a function of the damper gain for different bunch lengths. The main noise contribution relevant to the beam is in the 100 kHz range. Measurement of the residual low frequency modulation & feedback via HT monitor could be vital. If no measurable emittance growth is seen, active noise shaping can be envisioned by injecting RF noise overlapping with the tails of the betatron spectrum w/o affecting the core. This can selectively excite the tails, which could be important for machine protection and halo related studies.

They are three identified fast failures to expect during operations: Transmitter trip (most likely), real quenches (not obvious to describe), and arcs in the couplers (this ones can only be seen in the cavity). For each of the failure modes, correlated HT monitor, BLM and closed orbit measurements should be taken. The machine protection aspects and cavity trips should also be considered as transient behaviour like MDs.

Other expected studies include the feedback mitigation of a failure in one of the two cavities. The LLRF electronics will include acquisition of cavity field (amplitude and phase) with a resolution of 80 Msamples/second. Concerning the phasing of the CC, with 2 ns long bunches a 50 ps resolution seems to be sufficient and this can be achieved with a phase pick up. It was also mentioned that the relative noise between crabs is relevant for the feedback controls. It is necessary to find a compatible set up to allow low and high intensity runs. One important aspect is to study beam induced transients with high intensities during injection and energy ramp and the mitigation with counter phasing or cavity detuning. The correct counter phasing of the CCs, will first be validated during the SM18 tests. Can the test of the full detuning scheme in the LHC be mimicked in the SPS with crabs for validation?

Action (ABP/RF): In general the expected impact of failures should be evaluated to define where losses could occur.

## Beam Measurements

R. Tomas

The beam measurement program with crab cavities for commissioning and long term MDs was discussed. Specific instrumentation needs for measurements were also detailed. Seven categories of measurements were described which included crabbing mode, crab cavity induced dispersion, longitudinal collimation, emittance growth, phase noise, RF multipoles, and dynamic aperture. Very low emittance (0.7 μm) and low intensities are preferred for most of the measurements. Bunch lengths of 2ns was considered with options to lowering the bunch lengths by scraping to test closer to LHC like bunch lengths. At 26 GeV the highest orbit distortion can be realized and with vertical crabbing, the limiting aperture is TIDV.11892. The equivalent kick at 26 GeV using 2 cavities operating at 3.4 MV each is approximately 260 μrad. However, the long bunch length at injection energy might be an issue as the Beam position Monitors installed in the machine measure the average position of the bunch. To get reasonable bunch lengths of 2ns or less, one needs to go to 120-270 GeV with low emittance (~0.35 eVs) or to prepare dedicated beams of very small longitudinal emittance (which likely will have small population). Two other options can be considered for shorter bunches at 26 GeV: Option of scraping at injection by RF voltage reduction but resulting in very low intensity and use of Q30-40 integer tunes. The gain w.r.t to the present optics is not evident but it opens a possibility. The scraping should be tested in MDs.

Horizontal crabbing was shown by simulations, to be preferable over vertical crabbing if larger aperture is needed (for example for b3 measurements). The use of a longitudinal collimation using the RF nonlinearity on the very long SPS bunch is proposed. The measurement of RF multipoles (b3) of the main RF field using crab cavities like an AC dipole was explored. The crab cavity is driven close to Qx,y and measuring the frequency response from the DOROS BPM or the BBQ to see the 3Qx+Qcc=N resonance or drive an instability and measure the rise time. To see measurable effect horizontal crabbing would be needed, since vertical crabbing is limited by the machine aperture. Some or all of the measurements can be enhanced by going to close to an integer working tune which needs validation.

Coasting beams at 120-270 GeV beam is considered feasible in the SPS test, while not at 26 GeV. The possibility of most of the measurements being performed with cycling beams was reiterated. The availability of additional quadrupoles (to be checked) in the LSS6 region to locally perturb the optics to enhance the effects near the crab region might be possible.

The possibility of operating the machine close to the integer was also mentioned as a mean to enhance the effect of crab cavities if needed. This should be possible a low intensity.

Action: identify the feasibility of beams with low longitudinal emittance for the crab cavity tests: Machine development Coordination

Action (BE-ABP): identify means to enhance the effect of crab cavities with a localized beta-beating bump.

## SPS Instrumentation

T. Lefevre

The existing and future instrumentation available for the crab cavity tests in the SPS were reviewed with the potential limitations to carry out the measurements. The upgrade of the MOPOS in BA6 is feasible before 2018 as the fibre optics infrastructure is available. This should be requested. New BPMs should be installed (Action: BI) at each side of the cryomodule if necessary to independently monitor the orbit in the crab cavities and correlate the beam position with the electric center of the cavity. There is an urgency to finalise the list of new instrumentation (before March 2016) to effectuate the work in EYETS and YETS so that it is included in the ECR and follow the approval process. “Off-the-shelf” available LHC like BPMs can be considered. The resolution of the HT monitor is in the order of 100 μm with long acquisition times of up to 1.6s available now. The possibility to use additional wide-band pick-ups (coonected to fast digitizers) installed in the SPS could be envisaged but would require extra cabling and new electronics. The phase of the available transverse wide-band pick-ups should be determined to assess whether it allows the observation of the effect of the crab cavity. Action: BE-BI/BE-ABP. MDs to be performed prior to 2018 to verify the measurable resolution and suitable attenuators for varying intensity operations.  The availability of synchrotron light monitors is expected at energies above 300 GeV and therefore difficult to foresee with coasting beams. There is uncertainty on the usage of Schottky monitors as they are presently not in functioning mode. They could be useful tool for transverse emittance blow-up studies at low intensities. Action: BE/RF to verify the feasibility of providing the Schottky signal. It is presently a resonant pickup under the RF responsibility and will need approximately 1-year of development for the motors movements and controls to be re-commissioned. The study should be launched in 2016. It was pointed out that the tune can be adjusted to vary the phase advance between the CCs and the respective instrumentation as one easy knob but with very limited range. This has to be verified in MDs. Among the ongoing developments: The multiband instability monitor that will be checked once the LHC comes back online this year [2016], the diode detectors and ADCs, the use of crystals in the Electro-Optical-BPM for faster time response. The phase advance of the new electro-optical wideband pick-up with respect to the crab cavities should be verified. Action: BE/ABP. The use of BGI for continuous emittance monitoring if re-commissioned in 2016 will be a necessary tool. A table with the intensity limits and measurement errors for the wire scanners (linear and rotational) were presented [Instrumentation, slide 10]. Finally there is no certainty about the need of extra BLMs in the machine, some BLMs near the crab region to quantify the beam losses to the equipment during SPS operation should be foreseen

Four BPMs equipped with LHC tyoe electronics exist in the SPS, which can perform measurements bunch-by-bunch and turn-by-turn. The phase advance with respect to the crab cavities should be verified and possible needs should be assessed. Action: BE/ABP-BE/BI, The work package responsibility on action items, such as BPMs, BLMs, etc. should be clarified, most likely a joint effort between WP2/WP4/WP13. Identification of the bottlenecks for the intensity levels is necessary. A time scale of ~2-3 months for finalising the BI equipment list was proposed.

## SPS-LSS6 & Compatibility with SPS MD/Operation

H. Bartosik

The LSS6 location from the viewpoint of SPS operations was reviewed, for the various cycles and defined the compatibility of the CCs with regular SPS operations, including optics options, aperture, orbit drifts, and other relevant aspects. Studies of an alternative location (QDA.614) for the CCs allowing to operate the CC in parallel to LHC operation showed that they would become the global horizontal aperture restriction, leaving the nominal location (QF.617), as the better option for FT operation. The nominal location (next to TPSG near the QDA.617) being compatible with slow extraction to the North Area is not compatible with the beam extraction to the LHC, since the cavity would limit the aperture of the extracted beam. Therefore, the cavities can only be operated when LHC doesn’t request beam. This could be envisioned during regular operation if the feasibility of moving in and out is shown without interrupting the LHC filling. It was pointed out that the usual 5% beam loss during the slow extraction should be verified such that the losses to the crab cavities are minimized. The need for additional BLMs and possibly fast BLMs using diamond were raised. Action: TE/ATB?

The possibility of reducing the time required for the movement of the table should be identified together with the compatibility of the hardware with frequent movements if the table would have to be moved at every LHC filling. Action: A. MacPherson.

For the closed orbit correction near the CC location, standard CODS with maximum ~38/20 µrad (H/V) at 270 GeV without using the extraction bumpers. The feasibility to correct the orbit with standard orbit correctors at energies of 120 and 270 GeV should be studied as extraction bumpers cannot be used as they will be interlocked when the crab cavities are inserted. The bump extent near the nominal crab location should be checked in both planes in 2016 MDs along with the measurements with BPMs. Action: BE/ABP. BPMs available around these regions are BPH.616, BPCE.617 / 618. The need for additional BPMs around the crab cavities was once again raised. Dedicated MDs on Wednesdays and/or parallel MDs, could be in conflict with LHC operations, if physical access to the moving CC table is required.

It was proposed to use the SPS cycles for MDs to check the transparency and robustness of the CCs’ control to validate its operation with FT beams. The beam interlocking will be discussed with OP to develop the hierarchy within the crab cavity powering equipment and the link to the beam interlock which offers enough flexibility to allow for movement of the cryomodule without requiring access. Special access conditions will be needed to the cryogenic and RF nature of the crab cavities which will be defined with safety.

## Vacuum Aspects & COLDEX Compatibility

V. Baglin

A summary of the studies performed on COLDEX, as well as the compatibility of the CCs operation with COLDEX, was presented. Sectorization of the module is already in place to avoid contaminations during module swaps. In this talk, it was pointed out that the aperture checks including the new pumping scheme should be performed. The removal of the fast valve in the vacuum chamber could be considered in case longitudinal space is an issue. The possibility to shift the assembly sector valve towards the right by ~5.5 – 6 m is also another solution.

The installation protocol has been planned in 2 phases for EYETS 2016-17 and YETS 2017-18, but definition and procurements are needed for: Vacuum sectorization and instrumentation, freezing Y-chambers’ specifications (by December 2015), main components such as sector valves (6+1), pumps, gauges, etc. Also, it was stated that there are no indications of issues for low intensities, but for high intensities, knowledge of the electron cloud effects is needed. It was also established that calculations of electron cloud levels in QDA-type chambers, could be beneficial for the CCs commissioning and operations. These simulations exist. It was reminded that COLDEX only uses filling schemes and LHC [HL-LHC] multi bunches @26 GeV and 450 GeV and is only operated during technical stops. Lastly, it was insisted that the SPS layouts are needed for mid-2016 to start procurements on time to meet proposed vacuum installation in BA6. The maximum number of cycles should be estimated for the Y-chamber specification.

A question about the possibility to use aC coating in the vacuum chambers to get rid of the electron cloud as a limitation for high intensity operations, was raised. There is a big concern about the chance of contamination of CCs due to bad vacuum from their neighbourhood and the NEG solution is therefore adopted as the present baseline to reach the specified 10-10 mbar. COLDEX requests about 5-6 blocks of 12 h separated in time for the MDs. MDs in the SPS post LS2 should also test the feasibility of crab cavity operation with aC coated chambers near by as in the case of the LHC P1 and P5.

The impedance of the Y-chambers and of the components installed around the crab cavities should be verified and approved by the impedance team. Action: B. Salvant.

## Impact of SPS Transverse Damper

G. Kotzian

This talk presented the operational modes of the transverse damper including the compatibility aspects with crab cavities. The potential of the transverse damper and the wideband feedback system as an additional tool for measurements was also outlined. The boundary conditions for the damper to counteract transverse emittance blow up include: prevention of non-linearities to cause filamentation, by sufficiently fast feedback action, and the need for a low instability detection threshold. In open loop, the damper equipment can be used for passive diagnostics of injection transients in the presence of crab cavities, and for monitoring the impact of the crab cavities on coupled bunch instabilities. In closed loop, it can independently damp horizontal and vertical motion. The capability of the transverse damper to handle large injection errors was also mentioned. The damper is used to cure coupled bunch instabilities  driven by resistive wall impedances. The new wideband stripline kickers with 700 MHz BW could be used to generate a pattern similar to that of the crab cavities. Two kickers are able to provide vertical kicks of ~51 nrad which could serve as check for instrumentation sensitivity prior to the installation of the crab cavities in 2018. The impact of the co-existence of the damper and the crabs can be tested during 2018. The transverse damper diagnostics have a potential to measure intra-bunch motion, with bunch-by-bunch and turn-by-turn data available from the ObsBox. RF synchronous sampling  at 4GSPS sampling rate/ 8 bit resolution is ready and available with the wideband transverse feedback system. The transverse BQM is available for monitoring and logging at 8 GSPS sampling rate / 10 bit resolution.

There is a work in progress to study the use of the dampers as beam diagnostic tool: Bunch motion visualisation (quiet bunches, bunches with transverse CoM motion, head-tail motions, or a correlated combination.

The use of the wideband stripline kicker together with specific chromaticities to induce crabbing-like motion to the bunches should be investigated in the coming MDs. The damper could also be used to induce quenches in the cavities [single bunch], as it has been done in the LHC to understand cavity behavior.

# Preliminary Specifications & General Discussions

## Preliminary Specifications

The first tests in the SPS are foreseen to use two DQW cavities in the LSS6 region (section 617) with deflection/crabbing in the vertical plane. Some relevant longitudinal, transverse and RF parameters are given in Tables 1, 2 and 3.

Table 11: Relevant Crab Cavity parameters

| Characteristics | Units | Value |
| --- | --- | --- |
| Resonance frequency | MHz | 400.79 |
| Number of Cavities |  | 2 |
| Nominal kick voltage/cavity | MV | 3.4 |
| Total cryomodule length | m | 2.9 |
| R/Q (assumed, linac convention) | Ω | 400 (update) |
| Q0 |  | ≥1 × 1010 |
| Qext (fixed coupling range) |  | 5 × 105 |
| Max RF power | kW | 40 (80) |
| LLRF loop delay | μs | ≈? |
| Cavity detuning (SPS Coasts) | kHz | ±100 (-60) |

Table 22: Relevant Longitudinal RF Parameters

| Characteristics | Units | Value |
| --- | --- | --- |
| Energy (Cycling – MD2) | GeV | 26-450 |
| Energy (Coasting Beam) | GeV | 55, 120, 270 |
| Bunch intensity | ±1011p/b | 0.05-1.3 |
| Resonance frequency | MHz | 200.39 |
| Voltage 200 MHz | MV | 3.0-7.0 MV |
| Voltage 4th Harmonic (800 MHz) | MV | 0.0 |
| Longitudinal Emittance | eVs | 0.35-0.5 |
| Bunch length | ns | < 2.0 (4 σ) |

Table 33: Relevant Transverse Machine Parameters

| Characteristics | Units | Value |
| --- | --- | --- |
| Betatron Tunes, Qx,y |  | 26.12, 26.18 |
| Beta-function, βx,y | m | 40, 80 |
| Dispersion, Dx | m | -0.5 |
| Total length including bypass | m | ~9.5 |
| Crab cavity aperture | mm | 84 |
| Orbit Stability | mm | < 0.5 |
| Mechanical Tolerance (x,y) | mm | < 0.5 |
| Cavity Tilt Tolerance (x,y) | deg | < 0.3 |

The present Q26 optics immediately before and after the crab cavity location in LSS6 are given in Table 4. The closest vertical BPMs to the crab cavity locations are the BPCE large aperture BPMs to accommodate for the LHC extraction. The orbit resolution with these BPMs should be checked to investigate if a special BPM is required on both sides of the cryomodule for slow orbit feedback during the MDs. To be compatible with the LHC extraction, the new BPMs will then have to be installed in the crab bypass.

Table 44: Optics functions near the Crab cavity locations.

| BPM Name | Location [km] | βx[m] | βy[m] | μx | μy |
| --- | --- | --- | --- | --- | --- |
| BPV.61508 | 6239.4571 | 22.37235074 | 99.79342646 | 23.57286234 | 23.63700432 |
| BPH.61608 | 6271.4548 | 103.7497594 | 20.42041406 | 23.69849911 | 23.75192576 |
| BPCE.61705 | 6302.8175 | 20.54565796 | 102.9200118 | 23.80772296 | 23.88244342 |
| ACFCA.A1.LSS6 | 6315.875 | 34.76464658 | 68.20838752 | 23.89629943 | 23.90537359 |
| ACFCA.B1.LSS6 | 6316.925 | 37.16618251 | 64.51659996 | 23.90094916 | 23.90789285 |
| BPCE.61805 | 6334.8152 | 99.83755966 | 22.50635143 | 23.9483914 | 23.98567406 |
| BPCE.61931 | 6371.7429 | 23.64597639 | 96.54662255 | 24.08734227 | 24.12681019 |
| BPH.62008 | 6399.4456 | 105.1088038 | 20.37030849 | 24.18110345 | 24.23420616 |
| BPV.62108 | 6431.4433 | 19.74187926 | 104.5238067 | 24.29480409 | 24.36703721 |

Table 5 list some of special instrumentation that could be used for the crab cavity tests along with their location w.r.t to the crab cavities and their respective optics functions.

Table 55: Locations and optics functions of special instrumentation that are foreseen to be used during the crab cavity beam tests.

| BPM Name | Location [km] | βx[m] | βy[m] | μx | μy |
| --- | --- | --- | --- | --- | --- |
| Head-Tail PU |  |  |  |  |  |
| Wideband PU |  |  |  |  |  |
| LHC-BPM1 |  |  |  |  |  |
| LHC-BPM2 |  |  |  |  |  |
| Wire Scan 416 |  |  |  |  |  |
| Wire Scan 517 |  |  |  |  |  |
| Wire Scan 519 |  |  |  |  |  |
| Electro-Opt PU |  |  |  |  |  |

## General Comments & Discussions

Comments from a pre-meeting at Evian workshop from R. Tomas

So far beam tests were mainly low intensity (2-4×1010p/b) were done so far to understand the natural emittance growth at 120 & 270 GeV. To see the effect of crab cavities, a head-tail measurement is necessary and possibly turn-by-turn. Due to the rather long bunches in the SPS, the banana-shape longitudinally is likely to be pronounced. So the effects of RF non-linearity in the longitudinal plane will also be interesting to study for the case of long bunches.

The resolution of the head-tail monitor is currently 20 points in 2ns long bunch. A fixed attenuator is present aimed at high intensities (1×1011p/b), but a remote change in attenuation could be foreseen to accommodate both low and high intensity beams. This should likely be in place in EYETS and validated. Action: BE/BI?

The new electronics for the BPMs will only be finished in LS2. However, in 2018 (since the BPM infrastructure is already there) electronics can be plugged in to be available for the crab cavity tests including the DOROS.

A question on centring the beam was raised and the use of BPMs around the crab region to ensure the proper orbit in the crab cavities. With an amplitude and phase scan of the crab cavity with low intensity beams, the “golden” orbit going through the electrical centre of the cavities should be established. This orbit should be maintained (possibly by slow feedback) and corrected for gradual drifts. Should a fast feedback based on RF power be in place and tested?

In LS2, fast BLMs will be installed, a halo cleaning could be envisioned for the protection of the CC. A collimation study should be launched to check the efficiency and impact of halo cleaning.

For emittance measurements, a new prototype will be available during the time of the crab cavity tests, the speed of the scan could be optimized. The existing 417 & 519 scanners were already successfully used in the past for emittance measurements. Sometimes, the reliability and reproducibility is somewhat of a concern. The exact gain, filter and other parameters should be clarified for different intensity beams to have a reference parameter list. Some of this can already be extracted from present measurements performed on low intensity measurements.

Beam losses on the cavity due to injection, ramp and extraction is a concern and its effect on quenching or cavity performance. Interlocks should be in place to restrict the extraction to the TI8, this could be masked if a dedicated MD is foreseen to study the impact of beam losses on the cavity. The TE-ABT team should be contacted for checking extraction.

## Open Issues and Action Items

- The crab cavity ECR including any major modifications required for the layout near the nominal location in LSS6 should be drafted by the March 2016. The responsibility of collecting the needs from all equipment and services groups is within WP4.

- Some immediate simulations and specifications to be decided within 2-3 months to understand the need of additional instrumentation. The responsibility of which can be undertaken by WP2 with joint support from WP4, WP13 and others as necessary.

- The request to carry out the LSS6 improvement of the MOPOS should be recommended before 2018.

- The local orbit and optics insertion favourable for the CC’s should be checked in early 2016 MDs. The use of the extraction bumpers and the full extent of the orbit manipulation near the CC region should be validated.

- HT monitor resolution MD.

- A follow up meeting in a few months to revisit the various aspects to further converge on experimental program and beam parameters is needed. In this meeting, the minimum subset for the input to the HL-LHC production modules should be defined.

## Potential MDs for 2016-17

In order of decreasing priority:

- Emittance growth studies with chromaticity scan and transverse damper

- Lowest emittance/shortest bunch at injection via scraping (Q20 and Q26 for scaling)

- Wideband feedback kicks in a crab like pattern to measure the sensitivity from the existing instrumentation

- Beta-bump with relocated independent quadrupoles

- SPS operation feasibility at fractional Q~0.04

- Q30-Q40 optics (optimum to be found from MADX/simulations) provided the options above are not feasible

# Summary of Action Items

- OP: The interlocking of the LHC extraction elements to prevent the activation of the LSS6 extraction bump when the crab cavities are in beam.

- MD Coordination:  Exact intensity limit for coasts at 55, 120 and 270 GeV

- MD Coordination/RF: Identify the feasibility of beams with low longitudinal emittance for the crab cavity tests

- RF/MME: Reduction of time for the movement of the table

- ABP/RF: Expected impact of failures to evaluate where losses could occur during FT-beam

- ABP/RF/MP: Beam losses on the cavity due to injection, ramp and extraction

- ABP: Evaluate impedance of the Y-chambers and of the components installed around the crab cavities

- ABP: Localized beta-beating bump & The local orbit and optics insertion favourable for the CC’s should be checked in early 2016

- ABP/BI: New BPMs on each side of the cryomodule if necessary including use of BPCEs

- BI: Investigate the need for additional BLMs / fast BLMs near the crab cavity region

- BI: Investigate the issue with the dead time of the BLM during coast.

- BI: Remote change in attenuation of HT for both low and high intensity beams before 2018 (OASIS).

- BI: Investigate the need for new electronics for LSS6 to be plugged in before 2018 including the possibility of DOROS depending on the MD requirement of bunch parameters.