KE, LL 3/3/2009

Notes from the meeting of 27/2/2009 on parameters for the CLIC01_SiD simulation model

http://indico.cern.ch/conferenceDisplay.py?confId=53018

Parameters decided on / discussed

Solenoid magnet

Solenoid field:		5 T

Rfreebore  		2.90 m  (CLIC000 was:  2.80 m)

Length of the coil  	option 1 	+- 3 m

option 2	+- 4 m

nor DID nor Anti-DID (would need more study, so current default is without any)

HCAL barrel

Option 1 		W 1 cm 	Scint  0.5cm 	G10  0.25cm

Option 2		Fe 2 cm	Scint  0.5cm	G10  0.25cm

(it was understood that option 1 would lead to an HCAL of a bit more than 8, while option 2 would lead to an HCAL of less than 7, each behind an sid02 ECAL of ~1)

Cell size (not discussed) remains at 1x1 cm2

HCAL endcap

For engineering purposes:

HCAL endcap		Fe 2 cm	Scint  0.5cm	G10  0.25cm		total  9 deep

behind an sid02 ECAL of ~1

In the simulation exercises, one shall look whether there is a real need for such a deep HCAL endcap calorimeter.

Electromagnetic  Calorimeter

Not much new input – unchanged with respect to the most recent SiD value (sid02)

Tracker

Tracker Router   not changed w.r.t. sid02 value

Vertex detector

Innermost layer of barrel, radius R = 30 mm

Total 5 short barrels, 4 disks, following the proposed geometry on slide 7 of Konrad

General input to the discussion

a) Sandro Palestini:

If particle flow analysis should be adopted, the relevant parameters at ILC energies are

- BR2 and R for tracking (charged particles)

- Calorimetry for neutral particles

b) Jean-Jaques Blaising

as a general feature, the energy of particles in the interesting physics channels scales with the CM energy of the accelerator -> factor 6 for ILC vs. CLIC

at 3 TeV, some events show particles up to e.g. 400 GeV

at 3 TeV, PFA in the forward region is not proven, but considering interesting physics, the forward region is much more relevant

at 3 TeV, particle flow will generally suffer from more “confusion” (comment by  Paul Lecoq: yes, more confusion laterally, but better separation longitudinally)

for 3 TeV, we would like to have a longer coil and a (much) longer Endcap_HCAL

for ECAL, we might simply need to add a few layers w.r.t. SiD/ILD

c) Norman Graf

to illustrate the effect of ECAL depth: SiD simulated bhabha’s. Even at 1 TeV centre-of-mass this worked out fine, because any leakage from ECAL was anyway detected in HCAL

d) Dieter Schlatter

keep in mind that this discussion session is on an SiD-like detector, i.e. the high field and maximum Si tracker version (well knowing that the HCal is a bit thin in SiD)... will have to hold similar sessions on ILD-like detector

since the PFA might not be o.k. for the 3 TeV case, it is much more important to reduce leakage in the HCAL, in particular for neutral particles; at about 400-500 GeV tracks, the energy information from tracker and HCAL becomes of similar precision;

PFA algorithm so far did ONLY use tracker info for charged particles, then HCAL info for neutrals – will need to change this for 3TeV case

leave the tailcatcher out of this discussion – it has been shown to give a minor improvement (the 2 of coil are the real problem in this context)

natural upper limit for coil is the 3m from CMS (but insist on 5T)

e) Alain Herve

a little sceptical concerning 5T

a longer coil: not a serious problem

serious warning about the real field shape towards the endcap (yet another reason for a longer coil)

these discussions should finally put the R&D for conductors “en route”

f) Daniel Schulte

in the old design (2004), the vertex innermost detector was at R=30 mm because this allowed it to be outside the direct line of sight from the main dump (this depends, of course, on the mask design etc.)

yes, it is well worthwhile studying the background hits from incoherent pairs in the SiD-type “discs” of the vertex detector (so far, only barrel assumed)

DiD would presently look desirable, few % of luminosity at stake for the case of 20mrad crossing and 5T detector field – antiDiD detrimental to luminousity (comment Iftach: DiD very bad for LumiCAL)

g) Lucie Linssen

remember that CLIC000 in Sept 2008 was a “shot”, to get started; now we are attempting to make more improved choices for a next detector version such as to start physics simulations

Things to be investigated in the near future

DiD, anti-DiD etc.: clarify what needs to be shielded against what for the incoming beams / luminosity performance (then check the damage DiD or nothing does on the physics performance of LumiCal)

Realistic B-field at 5T: check homogeneity of field over the full tracker volume (in particular towards endcap) for the large Rfreebore value

Vertex Detector: check why SiD has such a lot of free space between vertex and tracker (in the barrel); check whether there would be any problem to place the discs of the vertex detector at larger z

Mixed HCAL W+something: we need (and wait for) input from the engineers, to have some guidance before starting any simulations