Tiina Salmi, 17.3.17

Updated after the discussions at the meeting 16.3.17.

# Parameters for magnet measurement database

# Schema

Figure 1: Schematic of the database internal structure showing the references. Splices measurement might be removed.

# Detailed parameters for the part of Test facility / Test set-up

# Test set-up

Input parameters:

Table 1: Input parameters for test set-up tab.

| Parameter name | Description / comment |
| --- | --- |
| Test set-up ID | Must be unique |
| Date of edit | (automatic, but can be edited) |
| Author of edit | (automatic, but can be edited) |
| Name of test facility |  |
| Name of the cryostat |  |
| Name of the operator |  |
| Magnet thermal cycle |  |
| Possible other test facility and signal recording related parameters |  |
| Detection system ID | Information of the signals measured from the coil and the settings used for quench detection? |
| Heater circuit ID’s | This will include the ID’s of the heater circuits. Each heater circuit is connected to one HFU. |
| Comments |  |
| References |  |

# Detection system

Input parameters:

Table 2: Input parameters for the detection system tab.

| Detection system ID | Must be unique |
| --- | --- |
| Date of edit | (automatic, but can be edited) |
| Author of edit | (automatic, but can be edited) |
| Signal 1: Name | All the signals that can be used in the quench detection are listed and described here. |
| Signal 1: Description |  |
| Signal 1: type | (Diff./direct/splice) |
| Signal 1: Threshold (V) |  |
| Signal 1: Validation time (ms) |  |
| … Signals 2-10. |  |
| Comments |  |
| References |  |

# Heater circuits

Input parameters:

Table 3: Input parameters for the heater circuits.

| Heater circuit ID | Must be unique |
| --- | --- |
| Date of edit | (automatic, but can be edited) |
| Author of edit | (automatic, but can be edited) |
| Magnet ID | - Selecting here the magnet ID will allow limiting the selection of Coil ID’s and Heater ID’s to only those that are in the coil. - The magnet ID is defined here, and not in the quench ID. - If a protection quench, or a training quench does not have any heater circuits. They still have to define the Magnet ID in this section! |
| QH1: Coil ID | The heater circuit consists up to 10 QH’s. |
| QH1: Heater ID | The 10 QH’s are defined here. To identify the QH’s uniquely, the coil ID must be given. |
| QH2: Coil ID |  |
| QH2: Heater ID |  |
| QH3: Coil ID |  |
| QH3: Heater ID |  |
| QH4-QH10: Coil ID and heater ID |  |
| Rdummy,1 (Ω) | Dummy resistors that may be part of the heater wiring scheme |
| Rdummy,2 (Ω) |  |
| Rwires (Ω) | Resistance of the wires, maybe useful to add in series with the heater strip circuit. |
| Circuit wiring | This is a text describing the connections. Example: ((QH1+QH2)¦¦(QH3+QH4))+Rwires means that QH1 and QH2 are connected in series and they are in parallel with the series-connection of QH3 and QH4 and in series with all is the wires resistance. There will be a code that can interpret this line, and do calculations such as total resistance and current and voltage division in the system. |
| CHFU (mF) |  |
| HFU charging voltage (V) |  |
| Measured circuit resistance at cold (Ω) |  |
| Comments |  |
| References |  |

Computed quantities:

Table 5: Computed quantities for quench protection tests.

| Heater circuit total resistance |  |
| --- | --- |
| Current in each heater strip (A) |  |
| HFU time constant (ms) |  |
| Peak power in each heater strip (W/cm2) |  |
| Heater energy density (J/m3) |  |
| Total energy of the heater circuit (J) |  |

## Measurements

#### Heater delay

Input parameters:

Table 4: Input parameters for quench protection tests.

| Quench_hh ID | Must be unique |
| --- | --- |
| Date of edit | (automatic, but can be edited) |
| Author of edit | (automatic, but can be edited) |
| Test set-up ID | Reference to the test-facility and test-set up related parameters (include heater wiring scheme and detection system) |
| Top (K) | Operation temperature |
| Imag (A) | Magnet operation current |
| FiredQH (Circuit ID) | Quench heaters used to provoke quenches in heater delay measurements and QI tests. This is not a link, just the name! |
| tQH trigger (ms) | Time when QH is triggered |
| tQH powered (ms) | Time when QH peak current reached |
| tQonset (ms) | Time of quench onset in coil |
| Range for tQonset (ms) | Time range for the quench onset, if the exact time is uncertain (min and max possible times) |
| Quenching times in other coils |  |
| Signal used for defining quench onset | Name of the signal, or voltage tap |
| tdet (ms) | Time of quench detection |
| Detection signal | Name on the signal triggering quench detection. The signals are defined in the test set-up / detection system section. This is not a link, just the name. |
| VT of 1st Q | Location of first quench |
| First quench at neighboring layer | If layer-to-layer quench propagation is visible, this can be used to compute the propagation time (should be used only if the neighboring layer itself is not heated with heaters) |
| Quench propagation | Free text for observations |
| Fit for the magnet current decay | The piecewise polynomial fit |
| IQH (A) | Measured heater current (if not measured, leave empty) |
| VQH (V) | Measured heater voltage |
| Rdump (Ohm) | Dump resistor resistance |
| Dump delay (ms) | Delay between quench detection / protection trigger and dump resistor trigger |
| Ramprate (A/s) | Ramp rate to target magnet current |
| Time at plateau after reaching Imag/target | Time to wait at Imag before activating heaters or perform other tests. This waiting time ensures that heating from AC-loss from the ramp has cooled down. (Could be relevant for cables with no core, if ramprate is high.) |
| Comments |  |
| References |  |

Computed quantities

Table 5: Computed quantities for quench protection tests.

| Tcs (Imag, Bpeak) | Current sharing temperature in magnet peak field |
| --- | --- |
| Tcs (Imag, B_under heater) | Current sharing temperature for heater delay |
| ISSL (Top, Bpeak) | Short sample current |
| Imag / ISSL | Percentage of short sample current |
| Heater delay from QH trigger to Q onset | tQonset - tQH trigger |
| Heater delay from QH powered to Q onset | tQonset - tQH powered |
| Heater delay uncertainty based on the Q onset range | tQonset is replaced with min. and max. time of the range. (Delay to tQH powered and tQH trigger?) |
| Heater delay to detection | Tdet - tQH powered (Also from tQH trigger?) |
| MIIts from tQonset (MAAs) |  |
| MIITs from tQH powered (MAAs) |  |
| MIITs from tQH trigger (MAAs) |  |
| Hotspot temperature for MIITs from tQonset (K) |  |
| Times to reach different voltages in different signals | Details to be defined |
| Coil resistances at the end of the current decay (Ω) | Final resistances, or a fit? Final resistances, or a fit? |

Questions:

- These parameters should cover the heater delay quenches and QI-tests. Which parameters should be added to include also energy extraction and CLIQ tests?

#### Training

Input parameters:

Table 7: Input parameters for training tests.

| Quench_a ID | Must be unique |
| --- | --- |
| Date of edit | (automatic, but can be edited) |
| Author of edit | (automatic, but can be edited) |
| Test set-up ID | Reference to the test-facility and test-set up related parameters (include heater wiring scheme and detection system) |
| Top (K) | Operation temperature |
| Ramprate (A/s) | Ramp rate to target magnet current |
| Quench sequence for training |  |
| Iquench (A) |  |
| Coil ID | For quenching coil |
| tdet (ms) | Time of quench detection |
| Detection signal | Name on the signal triggering quench detection. The signals are defined in the test set-up / detection system section. This is not a link, just the name. |
| VT of 1st Q | Location of first quench |
| Initial quench in one turn, or many turns? |  |
| First quench at neighboring layer | If layer-to-layer quench propagation is visible, this can be used to compute the propagation time (should be used only if the neighboring layer is not heated with heaters) |
| Longitudinal quench propagation | Free text for observations. |
| Fit for the current decay | The piecewise polynomial fit |
| Rdump (Ω) | Dump resistor resistance |
| Dump delay (ms) | Delay between quench detection / protection trigger and dump resistor trigger |
| Comments |  |
| References |  |

Computed quantities

Table 8: Computed quantities for training quenches.

| Tcs (Imag, Bpeak) | Current sharing temperature in magnet peak field |
| --- | --- |
| ISSL (Top, Bpeak) | Short sample current |
| Imag / ISSL | Percentage of short sample current |
| MIITs from tQonset (MAAs) |  |
| MIITs from tQH powered (MAAs) |  |
| MIITs from tQH trigger (MAAs) |  |
| Times to reach different voltages in different signals | Details to be defined |
| Coil resistances at the end of the current decay (Ω)? | Final resistances, or a fit? |

#### RRR

Input parameters:

Table 9: Input parameters for RRR measurements.

| Quench_RRR ID | Must be unique |
| --- | --- |
| Date of edit | (automatic, but can be edited) |
| Author of edit | (automatic, but can be edited) |
| Magnet ID |  |
| Coil ID |  |
| Measured coil resistance at Thigh (Ω) |  |
| Measured coil resistance at Tlow (Ω) |  |
| Thigh (K) | Upper reference temperature |
| Tlow (K) | Lower reference temperature |
| Comments |  |
| References |  |

#### Inductance measurement

Needed parameters must be defined

#### Splices (away)

This section might be removed

#### AC-loss measurement

This section should be added