NOTES made after AWAKE synchronisation meeting (28th Nov 2017)

Figure 1 plots key AWAKE frequencies with respect to the master RF oscillator, the laser oscillator and the SPS. Values taken from http://accelconf.web.cern.ch/AccelConf/ipac2016/papers/thpmy039.pdf

10 MHz GPS

5,995.798136 MHz DRO Awake Master

÷ 2

LINAC 2,997.899068 MHz

88.1735 MHz

88.173502 MHz mode locked laser

Comparator

Sagnac Loop

× 25/11

2×SPS RF = 400,788,645 Hz

SPS

÷ 2

÷ 5082

÷ 870

Plasma triggering clock = 9.971355 Hz

÷ 2

17

fc = 1/(5SPS period) = 8,675.079 Hz

Trigger

Sync Box

Period

RF

Magnets

Extract

Figure 1

SPS frequency = 200.394322 MHz (adjustable +/- 500 Hz)

SPS fill = single bunch in 4620 buckets

AWAKE RF master oscillator (RFMaster) = 5995.798136 MHz (RF DRO)

The SPS must lock to an RF oscillator to provide long term stability.

The AWAKE experiment repeats every ~ 30 seconds.

The SPS accelerates from 26 GeV to 400 GeV using its own clock, this takes ~ 4.3 seconds.

The single bunch circulation frequency must be a rational fraction frequency of the RF master. This is determined as 5,995,798,136 × 5 /(2685082) = 43375.4 Hz

The SPS receives pulses at 1/5 of the circulation frequency= 8675 Hz. The SPS modifies the circulation time by changing the magnetic field for a period until the pulses at 8675 Hz are coincident with the bunch. This takes 500 ms. The circulation period is then locked to the AWAKE RF master.

What is the accuracy of synchronisation at 8675 Hz? My guess is worse than 300 ps.

The SPS then fixes the RF frequency at 200,394,323 Hz = (RFMaster) × 25 /(6811).

The Electron Gun and plasma seeding needs a Laser. Extraction, triggering of the plasma and triggering of the high power laser is done with respect to a clock running at 9.971 Hz.

The Laser Master is at (RFMaster)/68 = 88.173502 MHz this allows an integer PLL synthesizer to generate 200.394322 MHz as times 25/11. (The SPS uses double this frequency for synchronisation.) Consequently laser pulses only arrive in the middle of SPS buckets every 25 SPS RF buckets corresponding to every 11 mode locked laser pulses see figure 2. This is why synchronisation at 8675 Hz is necessary.

Figure 2

Before plasma acceleration can commence both the mode locked laser and the SPS must lock to the 6 GHz AWAKE RF Master.

I would have thought that

The mode locked laser can stay permanently locked to the AWAKE RF master frequency using the Sagnac loop. It is locked at 1/68 of the RF Master frequency. Synchronisation between the Laser and the 2.998 GHz Linac RF is then excellent.

The laser pulses will lock to a zero crossing of the 5.996 GHz, see figure 3. A phase shifter on the Gun RF is needed to set the correct phase with respect the laser pulses. The Awake RF Master (Red) has four zero crossings on each Linac RF cycle (Blue). Once the RF gun phase shift has been set, its value depends on which of the four zeros the Sagnac loop has found. The RF gun must be re-phased each time the Sagnac Loop resets.

The phase between the Gun RF and the Linac RF can be fixed and does not need to be re adjusted in the same way.

Figure 3

The phasing of the bunch in the SPS is performed with the pulses at 8675 Hz and whilst the 200.39 MHz is derived from the AWAKE RF Master, the minimum of the SPS RF bucket (black) determined by the measurement of the bunch arrival time could be anywhere over a span of several hundred pico-seconds which would encompass several potential locking points, which at 6 GHz are 83.4 ps apart.

On each fill of the SPS, synchronisation of a single bunch with the (RFMaster)/691152 = 8675 Hz leaves uncertainty on the location of the bunch. This should be resolved when the SPS finally receives the RF at 200.394 MHz derived from the AWAKE RF master as the centre of the bucket should align perfectly with a zero of the AWAKE RF Master. Unfortunately if the SPS bunch was not aligned to better than 83.4ps to the (RFMaster)/68 then the centre of the 200.394 MHz bucket does not necessarily align itself on the Awake RF Master zero coincident with the laser pulse.

My Understanding of the Current Problem,

To solve this problem the idea was to re-align the Mode Locked Laser pulses to the AWAKE Master RF after the bunch had been aligned to the SPS and has been locked to the 200.394 MHz derived from the AWAKE master. This done initially with the comparator in figure 1. Unfortunately alignment is not sufficient for the Sagnac loop to find the correct zero.

Possible solution

A new measurement of the phase between the 400.788 MHz reference to the SPS and the 88.174 MHz mode locked laser using an interferometric measurement system can resolve this. This is not straightforward as the 88 MHz is not a factor of the 400 MHz. It could be done if the detection system only records every 11 pulse. At the moment I will disregard this option as I am not sure on the hardware needed to do it.

The interferometer needs the RF as a sine wave. To maximise the range one would want a lower frequency that the 6GHZ RF Master. A good choice might be ¼ of the RF master ~ 1.5 GHz. This is easily created by dividing the linac RF by 2. It will also be an integer multiple of the laser pulse frequency. Depending on the system design it should be possible to reduce the RF power so that the locking range is half an RF cycle ~334 ps. Initial alignment at 8675 Hz hopefully would be in this range.

The measurement resolution is likely to be of the order of 1:1000 so if the range is initially half a cycle at ~1.5 GHz then the phase error can be reduced to about 400 fs. If this signal is used for locking then once the phase error has reached this value, RF power can be increased. A factor of 10 would bring the locking error down to 40 fs. At this point one would probably switch to a second interferometer that uses a higher RF frequency such as the one currently in use.

If a 1.5 GHz and a 6 GHz interferometer are both generating error signals at the same time, and if a digital control is employed then the output to the controller can start relying on the 6 GHz interferometer signal when it becomes valid.

STFC Daresbury/Lancaster Cockcroft Capability

STFC is currently developing electronics so that a commercial Max Zehnder optical fibre interferometer (modulator) can be used to lock a mode locked Laser to the 3 GHz RF at the Free Electron facility CLARA. The scheme we are currently trying to implement is shown in figure 4.

The bias signal is adjusted so that the optical phase advance on the anticlockwise path shown in figure 5 varies by exactly  on successive laser pulses. Consequently the phase difference is determined from the difference in height of sequential pulses as follows.

Let Horizontal output = Dh

Clockwise amplitude contrib. = Gch

Anti-clockwise amplitude contrib. = Gah

Centre laser pulse in centre of RF applicator at t=0

Electric field on phase modulator=

Optical phase shift of clockwise laser pulse

Optical phase shift on anticlockwise laser pulse

Hence

(1)

The scheme being implemented allows pulse heights to be determined and hence a calibration can be performed to determine absolute phase.

Equation 1 shows that the range can be extended up to half an RF cycle by reducing the power to the RF port.

CLARA Locking Scheme with reference to figure 4.

Adjust time delays so that laser pulses come at the flattest part of the square biasing field

With no applied RF, adjust biasing square wave for max difference in peak heights

Double biasing voltage watching peak height return to become identical

Start phase control feedback loop then gradually increase the gain on 3GHz RF amplifier until lock is achieved.

Increase RF gain until maximum power reached or lock is lost. If lock is lost run start again from zero gain and do not adjust quite so high.

Figure 4

Figure 5