| BGC Regular Meeting # 864-1-B04 (CERN) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ## Date: 2018-04-13 |  | ## Project/Activity: WP13 |  |  |  |
| ## Attendees: ## GSI: ## Peter Forck (PF), Serban Udrea (SU) ## The Cockcroft Institute: Hao Zhang (HZ), Amir Salehilashkajani (AS) ## CERN: ## Ray Veness (RV), Gerhard Schneider (GS), Stefano Mazzoni (SM), Johanna Glutting (JG), Marton Ady (MA), Tom Dodington (TD) |  |  |  |  |  |
| Agenda: - Update from Cockcroft, HZ - GSI Optics, SU - Status of the LHC Tests, SM - Actions from the Collaboration Meeting, RV - GSI Collaboration Meeting minutes, TD Full agenda and presentations on https://indico.cern.ch/event/720875/ |  |  |  |  |  |
| Prepared by : T. Dodington G. Schneider R. Veness | Checked by : |  | Approved by : |  |  |
| ### Update from Cockcroft Institute, H. Zhang The damaged chamber has been repaired and is back at Cockcroft. HZ presents pressure in chambers, about 3x10-8 mbar after 12 hours, except interaction chamber which is 6x10-8 mbar. This is possibly due because the gauge is on a tee, HZ will follow this up with GS. The second nozzle, produced at RAL, has not yet been delivered to Cockcroft. GS will follow this up with SciTech and RAL. HZ presented the new design for the Faraday cup on a DN40 feedthrough. GS asks if it is possible to visualise the electron beam. PF suggests an additional phosphor screen on a moveable stage directly in front of the Faraday cup. PF emphasises that the scintillator screen in the interaction chamber is very fragile. HZ will discuss with Carsten and make a proposal in the next meeting. The new electron gun has arrived. HZ is making progress getting it ready for use. HZ confirms that the gauge next to the electron gun does not have a magnetic field, hot cathode type. Amir has been using the v1 system. No success in finding the gas-jet since it has been taken apart. HZ confirms the laser shines through the nozzle and skimmers. SU asks if there is any sign of the electron beam impacting the blackened insert in the interaction chamber. This has not been checked, but HZ will investigate. ### GSI Optics – Integration time calculations, S. Udrea Serban presented updated calculations on the expected integration times. There are many unknown variables to be considered, and previous calculations (presented at the Collaboration meeting 19.03.2018) were very conservative. A more positive outcome has been considered here. A ‘worst-case’ was presented. The density was 10 times lower than first estimation, and an increased depth of field by changing optical solid angle. The MCP of the camera will have at least 70% efficiency, rather than 50%. With Neon, detection times are 0.1s for electrons and 15s for protons. A new ‘best-case’ scenario – doubled gas density estimation, increased solid angle of optics, correction using Scheimpflug geometry will be necessary. Transmittance of optical system, optical filters and efficiency of MCP all at 90%. Excellent electron (1.4x10-5s) and photon (2x10-3s) detection rates for Neon. RV asked about feasibility of these optimistic numbers: - Solid angle increase is complicated but doable with Scheimpflug geometry. SU will make a proposed geometry for the next meeting. - Gas density is hardest to improve, taken as 5x10-10 cm-3. - 80-90% transmission for optical lens, up from 70%, is feasible and simple. - Narrow band filters could improve transmittance 3x. SU will look at costs and bring proposal for new narrow band filters for N2, Ne and Ar for the next meeting. - We don’t know the efficiency of the MCP as this is hard to calibrate and measure. ### Beam Time in Munich, S. Udrea SU talked about beam time at the Tandem accelerator facility in Munich with 14MeV protons. The beam had a current of 8 nA and diameter of 1mm. This offered the opportunity to test the emCCD camera, and fluorescence measurements were made with N2, Ne and Ar. Filters at different wavelengths were used, studies also carried out on the beam profile, intensity, integration time and pressure effect. SU will present more results in the next meeting. SM asked what wavelength was used for Ne. Both 585nm and UV band - in both cases a profile was detected, similar intensity. More analysis will be done. The UV line has a very short lifetime. ### LHC Test Status, S. Mazzoni SM confirmed he has received the new camera. All components necessary are now available at CERN. The image intensifier could be gated, but initially will not be used like this. Only a lens will be used for the yellow line – a teleconverter will increase magnification but decrease transmission. The system length is 300mm in total. Installation planned for TS in June 2018. Magnification tests for resolution will begin at CERN next week, nominal resolution is 57 line pairs/mm. Additional calculations should be done for the expected dark current. The permission for gas injection during operation should be negotiated by OP. RV will organise this. Connect the PMT to time the digital converter with 50ps resolution, allowing to see the bunch structure. Access to the machine is necessary to manually change the channel. The cooling on/off is automatic, but access is required to set the temperature. It is necessary to calibrate the gauges. ### Neon as a Working Gas, R. Veness RV presented a summary of the advantages and disadvantages of using neutral Neon. Strengths are its insensitivity to magnetic and electric fields, short excitation time, it is not pumped by the NEG and has a low condensation temperature, less prone to gas jet clustering. However longer integration times due to low cross-section, and low pumping efficiency for cryogenic and entrapment pumps. A list of how this impacts the BGC work plans was shown. Steps should be taken to maximise the signal by improving the curtain gas density and improving the optical system efficiency. Solutions to vacuum issues should be investigated, and further understanding could be sought by working with other gas-jet groups, e.g. CERN’s ‘Physics Beyond Colliders’ programme. Agreed that CERN, Cockcroft and GSI will look for other groups working on gas jets in their areas and check for potentially interesting collaborations or information. We will all come back in about a month and report from the 3 sides on potentially interesting other groups working on gas jets. ### Minutes from BGC Collaboration Meeting, T. Dodington Minutes from GSI are available here: https://edms.cern.ch/document/1964074/1 Individuals should check the action list to be sure they agree with tasks assigned to them, and provide a reporting date to Tom who will recompile the list. Add action: Simulation of Accumulation of electrons and ions in the interaction chamber from the e-lens process (Adriana) NEXT MEETING: 26 April at 14h CET. |  |  |  |  |  |
| ACTIONS |  |  |  |  |  |
| Investigate pressure difference in interaction chamber. |  |  |  | Hao & Gerhard |  |
| Proposal to install simple phosphor screen in front of Faraday Cup. |  |  |  | Hao |  |
| Contact RAL to ask about expected delivery of the nozzle. |  |  |  | Gerhard |  |
| Investigate impact of electron beam on the blackened chamber insert. |  |  |  | Hao |  |
| Propose filters (narrow band for N2, Ne and Ar) for optical improvements |  |  |  | Serban |  |
| Develop Scheimpflug compensation geometry to increase optical solid angle. |  |  |  | Serban |  |
| Prepare results from Proton beam time in Munich with emCCD camera. |  |  |  | Serban |  |
| Discuss with OP to arrange gas injection during operation. |  |  |  | Ray |  |
| Look and contact other groups working on Gas Jets for collaborations. |  |  |  | All |  |
| Check the Action list, confirm and provide reporting date to Tom |  |  |  | All |  |
| Update GSI meeting action list and minutes once feedback has been received |  |  |  | Tom |  |