The RD53A large scale pixel demonstrator chip has been developed in 65 nm CMOS technology by the RD53 collaboration, in order to face the unprecedented design requirements of the pixel 2 phase upgrades of the CMS and ATLAS experiments at CERN. This prototype chip is designed to demonstrate that a set of challenging specifications can be met, such as: high granularity (small pixels of 50x50 or 25x100 µm2) and large pixel chip size (~2x2 cm2), high hit rate (3 GHz/cm2), high readout speed, very high radiation levels (500 Mrad - 1 Grad) and operation with serial powering. Furthermore, coping with the long latency of the trigger signal (~12.5 µs), used to select only events of interest in order to achieve sustainable output data rates, requires increased buffering resources in the limited pixel area.

The RD53A chip has been fabricated in a wafer engineer run. It integrates a matrix of 400x192 pixels and features various design variations in the analog and digital pixel matrix for testing purposes. This paper presents an overview of the chip architecture and of the methodologies used for efficient design of large complex mixed signal chips for harsh radiation environments. Experimental results obtained from the characterization of the RD53A chip are reported to demonstrate that design objectives have been achieved. Moreover, design improvements and new features being developed in the RD53B framework for final ATLAS and CMS production chips are discussed.

~1500 characters