# LIU-PM-MIN-0462	EDMS 2052804

# LIU beam performance meeting

# Notes from the meeting held on 29 November 2018

Present: S. Albright, H. Bartosik, G. Bellodi, H. Damerau, G.P. Di Giovanni, K. Hanke, A. Huschauer, A. Lasheen, B. Mikulec, G. Rumolo, R. Scrivens

Excused: B. Goddard, M. Meddahi

- Matters arising (G. Rumolo)

- Last LIU Beam Performance meeting was held on 1 November and was dedicated to the review of the high intensity in the SPS by Hannes and Elena, and a brief summary by Richard of the outcome of the Linac4 source review, which took place on 22-23 October 2018.

- Today we review the progress on the understanding of the emittance growth at PS injection as well as the related item of the emittance measurements in the PSB.

- PSB studies on emittance measurements (F. Antoniou)

- Slides are available here.

- Before the end of the run, data in the PSB have been collected to progress on the dispersion measurements, the brightness curve and the optics model.

- Dispersion measurements have been carried out for the four Booster rings and BTM (with two different optics). Both BPMs and WS have been used. The dispersion measurements with BPMs in Rings 2-3-4 match the model, there is a discrepancy for Ring 1. At the WS location, the measured value differs by ~10% from the model value. The calibration of the instruments could be the reason for the discrepancies.

- Comparisons between emittance measurements (SEM vs. WS, Full Deconvolution vs. Standard Gaussian) have been conducted as a function of the momentum spread. The emittances have been recalculated using the measured values of dispersion. The difference between SEM and WS can be as high as 10% and there is no clear correlation with the momentum spread nor with the beam intensity. Looking at the measurements with the SEM, the difference between FD and SG increases linearly for larger momentum spread, but it decreases for larger intensity (where the relative betatronic contribution increases).

- Brightness curves have been measured in both H and V plane in many operating conditions (see slide 5). Also data with the prototype WS both in the PSB and PS have been acquired using two different transfer line optics (standard and rematched).

- Preliminary results for the brightness curves (slopes for each ring, emittances at 60e10 p for each ring) for nominal BCMS beam and standard TL optics have been obtained in various different cases (model, measured dispersion, different devices, PSB and PS). A few main points:

  - There is a spread between different measurements.

  - It seems to be important to have measured optics functions.

  - There seems to be an emittance growth in the horizontal plane between PSB extraction and PS injection (see next talk).

  - Almost no emittance growth is seen and the spread between different measurements is less important in the vertical plane.

Many more data are left to analyse in different configurations (1.5 eVs, rematched optics)

- Lots of data for optics measurements have been taken to establish the beta-beat at 160 MeV and 1.4 GeV using the nominal Q4Q4 optics – and in addition the Q3Q5 for 160 MeV for the calibration of the BPMs. There are still very large error bars at 1.4 GeV, while the beta-beat at 160 MeV is well below 10%.

- Next, the error analysis will be carried on and an important work of simulation is needed to better understand and interpret the measurements taken over 2018.

- PS studies on emittance growth at injection (M. Fraser)

- Slides are available here.

- The KFA14 waveform has been probed with 10 ns long bunches (INDIV rotated before extraction). This requires a huge number of measurements.

- For the four rings, the KFA14 results of these measurements show

  - First part of the flat top (~205 ns to extract LIU beams) exhibits +/- 1% ripple, as specified.

  - Second part of the flat top exhibits +/-2% ripple, sufficient for fixed target beams, as specified.

- No important impact of KFA14 on the beam size has been measured with bunches of 40, 140 and 160 ns length. A variation of 2% mainly in the second part of the flat top is seen, consistently with expectations.

- The emittance evolution at the PS injection has been studied with the turn-by-turn SEM grid during dedicated MD sessions (3h in total).

  - With standard optics, the horizontal beam size oscillations observed in first (14) turns after injection, periodic with the tune, are dominated by dispersion mismatch. Their amplitude is expected to lead to 15-20% emittance growth. This is roughly consistent with the results shown by Fanouria, but a detailed analysis is ongoing to quantify.

  - With re-matched optics, the horizontal beam size oscillation due to dispersion mismatch is strongly reduced.

  - Introducing voluntarily a betatronic mismatch in the horizontal plane, the observed beam size oscillation has also a component with double frequency (twice the tune), as expected from the model.

  - Basically negligible mismatch is observed in the vertical plane both with standard and re-matched optics.

- The fast SEM measurement system has proved to be very useful and will be critical for commissioning of the upgraded PSB-to-PS transfer line and injection system at 2 GeV after LS2

- The dominant contribution to emittance growth in the present situation has been found to be coming from dispersive mismatch and will be removed with new TL after LS2.

Giovanni thanked Fanouria and Matthew for the excellent progress in the general understanding and for continuing work with clear outlooks for the next steps.

- AOB

  - The next LIU Project Team meeting will take place on 6 December, 2018 at 10:00, preceded by a brainstorming discussion on the LIU structure in 2019-20.

  - The next LIU Beam Performance meeting is scheduled for Thursday, 13 December, 2018, and will cover a wrap up of the general experience with Pb ions in 2018.

Minutes by G. Rumolo