# LSWG minutes (29th November 2018)

Present: A. Calia, M. Crouch, M. D’Andrea, S. Fartoukh, M. Hostettler, G. Iadarola, E. Maclean, D. Mirarchi, T. Persson, B. Petersen, A. Poyet, A. Rossi, M. Solfaroli, G. Sterbini, C. Schwick, H. Timko, R. Tomas, J. Uythoven.

https://indico.cern.ch/event/772189/

MD3343 & MD3344 : automatic collimator alignment – cross talk of aligning two beams in parallel (G. Azzopardi)

The results on automatic collimator alignment were presented. A cross-talk on BLM signal is present when aligning two beams in parallel. Analysis from 2018 alignment (not done in parallel) allowed to identify the BLM with highest cross-talk (TCLAs). From this model an algorithm alignment in parallel of both beams was used to align all collimators in about 15 minutes.

Results on automatic angular alignment were also presented. After an update of the FESA class, the new algorithm was successful. TCTPV.4R5.B2 that failed in 2017 was aligned, even though the results shown by some BPMs are still to be clarified. The cross-talk model has still to be added to the software.

MD3324 counteracting coupling decay at injection (T. Persson)

Investigations were done on the coupling decay at injection, identifying a correlation with the powering of b3 spool pieces. Misalignment and/or miss-powering of these circuits seems indeed to be the reason for the observed decay. The analysis was done on B2 were the static component of b3 correction is already not ideally local, as the contribution from S78 is distributed on the other sectors (RCS.A78B2 has a short). A new knob that powers b3 unevenly around the ring was demonstrated to take care of the b3 decay without introducing any change in coupling. This knob could be used by Fidel to cancel the dynamic b3 decay maintaining zero coupling. Promising results for Run3.

MD3263 LRBB (G. Sterbini)

36 hours of MD were assigned in 2018 to study the effect of the compensation wires installed in IP1 and IP5. In MD#3 asymmetric beams were used and compensation was activated at different crossing angle levels (down to 130 urad). In MD#4 a validation fill was needed as the configuration of the wires was changed to quadrupolar connection. Then a fill with three trains was carried out. Scans of wires, crossing angle and octupoles were performed. The compensation effect was clearly visible, resulting in a gain of about 10 urad with the wires powered.

As end of MD, a test of main bend active filters was performed, switching off all eight sectors singularly. A non-trivial impact of the spectra of the two beams was observed. The studies to model the observed response matrix are ongoing.

The possibility to change the configuration of the wires to have them on B1  is being investigated.

MD3270 Round ATS optics & MD4023 TCDQ levelling (S. Fartoukh)

An intensity ramp-up was done on the combined ramp and double squeeze cycle. The capability to run the full cycle with negative octupole polarity was demonstrated. No instabilities were observed when collapsing the separations.

In MD#4 the cycle was used with 8b4e. At 800 bunch, high losses (~70%) at TCT at end of ramp were observed, due to non-optimized working point. The ~1600 bunches fill was then performed on the nominal cycle. BBLR studies were carried out with 1.5-1.6e11 p/b, with 120 urad half crossing angle.

The results of the MD performed at injection to demonstrate the possibility of moving the orbit and one jaw of the TCSP were presented. The TCDQ levelling was demonstrated.

MD3165 RF power limitations & MD3166 RF injection oscillations (H. Timko)

The MD was performed both in MD#3 and MD#4, with similar conditions. Cavity tune and loaded Q were optimized to measure the maximum RF voltage maintainable with beam. In MD#4 higher beam intensity was injected with klystrons at 58 kV and the voltage was halved on the weakest line. In MD#4 1.3 MV/cavity was achieved on 15 out of 16 cavities. More precise analysis is needed to define an operational margin.

MD3166 RF injection oscillations throughout the ramp. Two tests were performed to identify the origin of the injection oscillation that are observed to continue until high energy. Twenty injections of 48b were done with 6 MV, then the same amount of injections was performed with single bunches into 4 MV. The second conditions showed better damping, but the “hole” in the particle distribution was observed again. Similar conditions are observed at flattop. When opening the phase loop, the area fills up in about 10/15 min.

MD4484 was cancelled due to lack of availability of the machine.

MD3298 & MD3300 & MD4203 & MD2484 & MD3295 Heat load & e-cloud (G. Iadarola)

The results of several MDs carried out on heat load studies were presented. The heat load dependence on bunch intensity was characterized. Low limit is found at 3e10 p/b. The high intensity limit was scanned with 12 bunches as this beam can be injected with up to 1.9e11 p/b. The results match well with simulation expectations. Results indicate  that heat load in LHC is dominated by dipoles.

Studies were also done with 8b4e beam (1.6e11 p/b). As expected, the heat load is drastically reduced with this type of beam.

Characterization of heat load vs bunch length was also done. Length of bunches was varied between 0.85 ns and 1.6 ns, observing a decrease of heat load in dipoles, but increase in quadrupoles for the longer bunch length, while the total contribution goes down. This behavior confirms the fact that heat load is dominated by dipoles.

Finally, the distribution of heat load on the two magnets aperture was checked, showing some asymmetries between B1 and B2. The difference points to the fact that heat load depends on beam screen.

MD3263 beam loss or emittance growth from MO settings (E. Maclean)

The MD aimed at checking whether blow-up observed in the LHC ramp could be related to stable islands, with particles that move out, as octupole strength varies during the ramp. The MD was carried out at injection trimming the octupoles on the ramp time scale. Some influence of MO ramp-down on emittance growth was observed, specially with chromaticity of 15 units, but a more detailed analysis is needed, specially to diagnose if the mechanism involves the formation of islands or it is a different one.

MD3312 Replicating the LH-LHC DA (E. Maclean)

The MD was carried out on flat optics cycle. The first part was dedicated to try to explain the 5% beta beating difference in optics measurements observed in MD#4 wrt MD#2. Tune corrections done with the QFB (thus distributed over all arcs including  the ATS sectors) were identified, explaining the observed variation. Beta* was also re-measured after new corrections, obtaining a value of 17 cm (against the theoretical 15 cm and a previous value before correction of 18cm). The second half of the MD aimed at replicating the HL-LHC DA, by artificially introducing non-linear errors. Severe challenges even to excite the AC dipole for normal optics measurements were identified. A partial mitigation of moving the tunes to avoid resonances was identified experimentally.

S. Fartoukh commented that the collinearity knobs could the reason we do not measure a beta* of 15 cm.

MD3349 start collisions at 1 m beta* (A. Calia)

Mechanics of collide and squeeze demonstrated. Collisions were set at 1 m, then squeeze was performed to 30 cm moving collimators and observing orbit excursions. Collisions optimized at every step then incorporated. The second attempt showed full reproducibility. Despite some minor improvements the MD was a great success.

MD3327 & MD4167 Crystal collimation with protons (M. D'Andrea)

Two MDs were performed on crystal collimation with p.  the first one was dedicated to angular and linear scans and loss maps. During the second MD the first ramp with all four crystals was also performed. Loss maps were done with crystals as primaries and different TCLA settings. Results seem promising, but some deeper analysis is needed.

Channeling during the ramp (angular and linear position functions) was kept until the trip of a corrector that happened slightly above 3 TeV.

MD3328 Injection with collimators at 3 sigma (M. Crouch)

The MD aimed at probing the tighter aperture foreseen for FCC and HE-LHC, by reducing the collimator gaps from nominal settings down to 3 sigma in the TCP and TCDI in steps. A second part was foreseen when applying an orbit bump across IR7 to investigate the cold losses downstream of the betatron cleaning section, but it was not performed due to lack of time (injector availability issue). Measurements were done on b2 while another MD was carried out in parallel. Results show that operating with TCDI at 4.5 sigma is challenging but possible whereas HW modifications are needed for tighter conditions. The limit on the TCP mostly depends on how well we can control the satellites, but preliminary observations indicate  that a primary collimator cut at 5 beam sigma is within reach.