# Run 2 Meeting 29-11-18

Present: Edda Gschwendtner, Patric Muggli, Erik Adli, Spencer Gessner, Livio Verra, Marlene Turner, Brennan Goddard, Steffen Doebert, Allen Caldwell, Alexey Petrenko.

# AWAKE Run 2 Discussion (Edda)

Edda summarizes the goals of AWAKE Run 2: high electron witness energy and bunch quality. This means: accelerating gradients on the order of GeV/m; percent-level electron bunch energy spread at GeV energies; normalized electron bunch emittance (of the accelerated bunch) on the order of 10 mm-mrad and the amount of accelerated charge >100 pC.

AWAKE Run 2 experiments are supposed to be finished at the end of CERN long-shutdown 3 (LS3) (2025). After LS 3 (2026) the AWAKE concept should be ready for first applications. Given this tight timeline, the baseline design concept needs to be decided in the next months.

Two scenarios for Run 2 exists:

- Start Run 2 with the existing 20 MeV electron injector in 2021, and upgrade the injector while running.

- Upgrade injector immediately, which could last then until 2021 and start Run 2 in 2022.

The following questions need to be answered along the coming months.

Open technical questions include:

- choice of the electron injector energy: two options are proposed:

  - continue with the existing 20 MeV electron injector and beam transport line (with small modifications possible) and upgrade to 100-200 MeV during Run 2 experimental time.

  - upgrade the electron injector to 100-200 MeV right after LS2 (2021) and start experiments in 2022.

- additional space to the experimental facility: emptying CNGS takes ~1.5 years.

- development time delays :

  - new plasma design is expected to take ~2 years;

  - new electron source ~3 years;

  - electron line ~2 years;

  - equipment integration and installation > 2 years.

Open scientific questions include:

- Is there a consistent (realistic) set of parameters (and parameter range) with which the physics goals stated in the first paragraph could be achieved?

- Which external electron injection mechanism should we use for Run 2? Will we inject on-axis or on-axis after transversely crossing wakefields?

- What further information do we need? Can further measurements with the current system answer open questions for Run 2?

Edda explains that while Run 1 is fully funded, there is no budget for Run 2 at the moment. Currently even Run 2 studies need to be paid from Run 1 budget. AWAKE Run 2 is not yet in the LS2 planning, which means that available person-power is very limited. To ensure continuous funding and the be included in LS2 we need to present a good proposal for Run 2 with convincing arguments for the required resources.

Thus, Edda proposes to:

- Prepare a breakdown of required resource for the studies for Run 2 so that funding is secured in 2019 and 2020.

- Propose a complete Run 2 design once the studies are finished (end 2019, beginning 2020) and get full funding once approved.

Edda stresses to really concentrate in the coming months on the general strategy and not to focus too much on technical details.

# Possible measurement program with existing 20 MeV and high energy 100-200 MeV electron beam (Patric)

Patric reminded us that while AWAKE Run 1 was overall successful, we found many challenges in the experimental concept and design. Difficulties included:

- electron beam: the 3D geometry of the transport line makes it very sensitive to momentum jitter;  the earth-magnetic field bends the beam trajectory over several milli-meters over meter scale distances.

- oblique injection: there is no diagnostics to ensure overlap; the electrons bunch drives wakefields itself; the plasma edge or geometry effects are important as one transversely crosses wakefield.

The success of AWAKE Run 2 will depend on achieving certain specified parameters, which makes the experiments more demanding. To meet the goals specified for Run 2, Patric proposes to:

- separate the seeded self-modulation of the proton bunch from the acceleration stage,

- use a short, dense electron witness bunch (much shorter than the plasma electron wavelength) to beam load the wakefields,

- inject a matched (to the plasma) electron bunch on-axis in a non-linear plasma wave. As it is well known, the linear focusing forces of the plasma electron bubble then preserve emittance.

This poses challenging requirements:

- short gap between the SM and acceleration stage to avoid transverse evolution of the proton bunch,

- a 100 fs long, ~5 micron in transverse size electron bunch to be able to beam load the wakefields,

- diagnostics:

'standard':

  - screen based proton and electron bunch measurements.

  - SSM-OTR streak camera and halo measurement on each event.

  - electron imaging spectrometer.

'advanced':

  - beam overlap diagnostics in and close to the plasma.

  - precise timing measurement of electrons and protons (e.g. electro-optical sampling) and 	  electron and wakefields.

  - wakefield measurements (e.g. Schlieren, interferometry, photon acceleration...).

  - single event emittance measurement (e.g. spectrometer, betatron radiation, optical pepper- pot).

Edda notes that a new Coordination Package in AWAKE lead by Stefano Mazzoni has been setup in order to study the beam diagnostics requirements and equipments and to coordinate the contributions from the different institutes involved.

Patric proposes a consistent set of proton, electron and plasma parameters to achieve the Run 2 goals: plasma electron density of 7x1014/cm3; electron bunch charge 100 pC;  electron bunch length 60 um; radial bunch size: 9.5 um; normalized emittance: 2 mm mrad.  Action: Patric to provide a consistent set of parameters for a plasma electron density of 2x1014/cm3.

For first Run 2 experiments, the plasma will likely be a laser ionized rubidium vapor source. Eventually, the accelerating section should be of a scalable technology (e.g. helicon or discharge plasma). Additionally the charge and emittance of the outcoming bunch must match the requirements from the applications.

Patric also summarizes the experiments that he thinks could be done with the current 20MeV bunch (in 2021):

- electron propagation in plasma (no protons) to study the wakefields that the electron bunch drives and its energy depletion with a low energy spectrometer on IS 1.

- studies to understand the plasma radius and plasma decay times (ns-scale).

- seeding the self-modulation process with electrons.

The following steps are concluded from the discussions:

- to evaluate the electron injection schemes. Should the electron injection be done in plasma or vacuum?  Action: simulation team (Alexey)

- determine consistent sets of electron bunch parameters with different energies, charges, emittances and beta functions.  Action: Patric, Erik.

- study the beam transport with the proposed sets of parameter: bunch compression, dog-leg for on-axis injection and micron size focusing.  Action: Steffen for e-source, Brennan/TE-ABT for e-line.

- study beam alignment for beams with very short beta functions (~ 1mm).  Action: Brennan/TE-ABT.

# Run 2 Plasma Source (Allen)

The idea is to split the self-modulator (~8m) stage from the accelerator (~10m) to be able to preserve electron bunch quality. Allen wants to review what we have learned from data and simulations to discuss plasma source design options and electron bunch parameters.

What we learned from simulations:

- It was shown in simulations (A. Petrenko, K. Lotov) that a longer gap between the self-modulator and the accelerator decreases more the accelerating gradient in the accelerator. Thus, it is important to keep the gap short (< 1m). Additionally a density step of a few percent in the self-modulator can prevent the decrease of the accelerating fields after self-modulation saturates.Studies also showed that the density in the self-modulator and accelerator should agree within >0.2% for electron injection after ~60 microbunches (i.e. ~7 cm for 7x1014/cm3 and ~10 cm for 2x1014/cm3).

- K. Lotov performed 2D-cylindrical simulations of on-axis injection: a) of test electrons (electrons do not interact with the wakefields) and b) of electrons with different bunch charges into wakefields at different delays with respect to the seed point. The results show that electron beam loading of the wakefields favours emittance preservation (we note that the wakefield potential well in these simulations is sensitive to 3D imperfections in the drive bunch).

- Simulations from Veronica et al. (using a single proton bunch to drive wakefields and on-axis electron injection; electron bunch energy is 217 MeV to match the relativistic gamma factor of the protons (gamma = 427)) show that the emittance of the back part of the witness electron bunch can be preserved. In these simulations, the head of the electron bunch contributes to driving the wakefields such that the back of the bunch experiences the linear focusing force from the then created blow-out region. Additionally, percent level energy spread for the bulk of the electrons is shown.

What we learned from experiments:

- Plasma electron density ramps at the plasma entrance and exit degrade the quality of the injected electron bunch. The same is true for experiments in which the electron bunch has to traverse transverse wakefields. Ideally the Run 2 vapour source(s) would have no density ramps (e.g. thin foils at the entrance and exit) and on-axis electron injection would occur in vacuum.

- Run 1 experiments show that the proton bunch self-modulates even without laser pulse seeding (even with ~2 larger proton bunch radius at the plasma entrance).  Consequently, the head of the proton bunch could self-modulate in a preformed plasma and disturb phase stability of wakefields driven by a train of micro-bunches. One solution could be to seed with the wakefields driven by a short electron bunch. Preliminary experimental results (A. Bachmann, P.Muggli) show that an electron bunch can seed the self-modulation process.

Allen proposes two different electron injection concepts (and thus plasma designs) for Run 2:

- electron injection in a region of detuned plasma electron density

- electron injection in vacuum.

First studies show that for scenario 1) the injection gap could be short (~few cm), but the bunch has to cross the detuned wakefields driven by the proton bunch (wakefield amplitudes on the order of 10 MV/m). Action: Alexey to simulate the emittance that is possible by using that injection scheme. In this concept, the vapour source design needs to accommodate the last bending dipole. Density ramps are present at both ends of the detuned injection region.

For scenario 2) electrons are injected on-axis into the accelerator in vacuum.  Two thin foils separate the vacuum from the plasma sections (preventing density ramps). The second plasma could be ionized by a laser propagating from the end of the accelerator (preformed plasma). This injection scheme also allows the second plasma to be of a different technology (e.g. helicon source) and is thus advantageous for first applications.

Livio Verra performed first studies on electron bunch emittance growth in thin foils (as a function of electron bunch energy and foil thickness and material) using the Monte-Carlo simulation code FLUKA. His results show that a 1 mm-thick Aluminum foil increases the emittance of a 100 MeV, 10 um pencil (emittance = 0 mm mrad) electron beam to ~8 mm mrad. To compare, a 100 um thick foil increases the emittance to ~2.5 mm mrad.

Spencer comments that a laser pulse hitting a thin foil could create a plasma. Experimental results from plasma electron density decay studies (S. Gessner) show that the plasma electron density does not change over ~100 ns (within the accuracy of the measurement).

Allen summarizes his talk with the decisions to be made on:

- what should be the length of the self-modulator (also experiments at 2x1014/cm3 should be possible) ?  Action: Patric

- what is the space needed for electron bunch injection?  Action: Brennan/TE-ABT

- what should be the electron bunch energy?

- what should be the design-concept of the two plasma sections?

# Run 2 simulations of a pre-ionized second plasma stage (Alexey)

In order to understand the effect of the proton beam in the plasma source design as in scenario 2) proposed by Allen, Alexey performed LCODE simulations at a plasma electron density of  7x1014/cm3  and a rms proton bunch length of ~6 cm. He compares the maximum amplitude (the longitudinal location along the bunch at which those fields occur can vary) of the longitudinal wakefield along a pre-ionized plasma section for when the unmodulated head of the bunch is present (scenario a) or not (scenario b).

Simulation results show that if the head of the proton bunch (initially unmodulated) is present (scenario a) the field amplitudes agree with scenario b over the first ~2-3 m. Then the wakefield amplitudes start to grow (unlike in scenario b) for about ~4m before they start decreasing (factor 2 over 5 m). Wakefields increase as the head of the bunch that starts self-modulating. They then decrease as wakefields driven by the head of the bunch cause the defocusing of micro-bunches in the modulated bunch train. He shows that the effect is less important when the seed point is shifted forward (less protons in the unmodulated head). In this case the optimum plasma density step in the self-modulator can be different.

In any case, the phase of the wakefield shifts when the head is modulating but Alexey thinks that the effect is negligible. ==> Action: Alexey to give more details on the simulation numbers. Additionally Alexey claims that simulation results overestimate the effect of the modulation of the bunch head as the noise level is usually overestimated in simulations.

# Electron injector studies for AWAKE Run 2 (Steffen)

Steffen explains that as a result of the experimental and simulation results, the electron bunch parameters of  Run 2 were updated to: energy > 50 MeV, energy spread < 1%, energy stability 10-3, rms bunch length ~100 fs, bunch charge 100 pC, normalized emittance 1-4 mm-mrad, transverse beam size at the injection location ~5-10 um.

Electron injector designs were studied with the ASTRA code. Final electron energies of 85MeV and 164 MeV were simulated. In both cases all required Run 2 bunch parameters could be achieved. In general the results show that: the higher the energy, the more costly and spacious the injector, but the easier the beam transport (i.e. because space charge effects decrease). Bunch lengths of ~100 fs can be achieved for energies of ~50 MeV and higher.

Steffen also studied what parameters could be achieved using the 20 MeV electron injector of  Run 1. He found that the shortest bunch length at 20 MeV is around ~400 um. If one uses the booster structure for bunching, one could get down to ~100um at an energy of ~7 MeV and an energy spread of 3-4% and a normalized emittance of 4-6 um. The maximum energy possible with the current injector is ~25 MeV.

He also summarized his experience and impressions:

- higher electron bunch energies facilitate beam transport and injection into plasma.

- ultra small emittances might not be extremely helpful (based on experimental Run 1 results).

- shorter bunches are difficult to produce and transport but important for beam loading.

- the electron injector might be housed in the proton tunnel, the transport line should comprise as few bends as possible.

- the Run 1 injector will likely not achieve any of the target parameters for Run 2.

# Electron beamline performance reach (Brennan)

Brennan summarizes the status of the current beamline. The transport optics is fully understood as measurements agree completely with the optics model. Position prediction tools give robust results and will be developed for post analysis. If the current electron injector will be used for Run 2, momentum jitter, momentum spread, dispersion control and overall stability would need to be improved to reach ~10um beam sizes.

Lessons learned for Run 2 (from operation in 2018) include:

- parasitic dispersion from corrector magnets can easily reach ~3-4cm and is thus not negligible.

- realistic assumptions for the transverse emittance, momentum spread and jitter sources are needed to understand the transport line.

- a focusing triplets with even stronger magnets will amplify errors and instabilities and cause effects such as e.g. chromatic aberration and non-linearities.

His understanding for the Run 2 electron bunch requirements are (additionally to those parameters already discussed in these minutes): a beta function at the focal point of 3.08 mm at 200 MeV (or 1.78mm at 20 MeV) and a pointing stability smaller that the rms spot size.

Brennan needs further input concerning: adjusment range of the focal depth in plasma and the adjustment range of the transverse position and angle  Action: Patric to give those numbers.

Brennan explains that the beta function at the focal point can be estimated from linear optics theory and depends on the aperture as well as the focal length of the triplet. For a maximum aperture of 60 mm and a beta function at the focal point of 1.8mm the focal length can be a maximum of 0.9 m for a beam energy of 20 MeV or 3m for 200 MeV. Linear estimates of achievable transverse beam sizes show that the dispersion at the focal point must be on the milli-meter scale for energy spreads on the 0.1% level.

The stated Run 2 parameters are difficult to achieve even with a 200 MeV electron bunch, however, once the parameter list is given to them  Brennan/TE-ABT to study the feasibility of the injection area into plasma.

# Infrastructure constraints in the AWAKE area (Edda for Ans)

AWAKE could add ~100m of experimental area to its experimental facility by emptying the CNGS area. First estimates show that such an intervention would take around 18 months and cost on the order of 2 MCHF (mainly transport costs included here). Alternatively, one could add around 5m to the experimental area by dismantling only the structure upstream the target (+ 5m, ~3 months, 0.2 MCHF). Brennan remarks that we should also investigate how much space could be gained upstream the current plasma entrance.

Edda (and Ans) remind us that the CNGS area has an overhead crane, but nothing else. If we need equipment such as lights, cooling, network, cable trays... we need to define, request and install it. Since certain equipment e.g. plasma, electron source need special infrastructure it would be important to define their location soon.

Integration options of a new 100-200 MeV electron source need to be studied. Given the funds and the time, tunnel enlargment is possible (cost can easily be 5-10 MCHF, time ~6 months).

# AOB

The next meeting will be organized by Edda in January 2019. The invitation will be sent out in due time.

Marlene Turner, 6 December 2018