# 62nd Meeting of the HL-LHC Technical Coordination Committee – 29/11/2018

Participants:, A. Apollonio, V. Baglin, C. Bahamonde, E. Belli, L. Bottura, R. Bruce, O. Brüning (chair), M. Calviani, F. Cerutti, S. Claudet, R. De Maria, M. Freitas Mendes, R. Garcia Alia, A. Lechner, G. Lerner, M. Martino, R. Martins, F. Menendez Camara, E. Metral, D. Mirarchi, M. Modena, F. Oliveira, Y. Papaphilippou, S. Redaelli, M. Rodriguez, L. Rossi, B. Salvachua, D. Schoerling, G. Sterbini, R. Tomas Garcia, R. Van Weelderen, A. Waets, D. Wollmann, S. Yammine, M. Zerlauth.

Excused: C. Adorisio, G. Arduini, P. Fessia, S. Gilardoni.

The slides of all presentations can be found on the website and Indico pages of the TCC.

After a review of today’s agenda, the minutes of the previous meeting were approved without further comments. Two actions were noted. The first refers to the need for an assessment by WP2, WP3 and WP12 regarding a beam screen vibration test on a long magnet outside the IT STRING.  The second action refers to the confirmation from WP3 and WP12 on the proposed split in responsibilities concerning the BPM. R. Jones confirmed off-line that there is some follow-up work on this and he will come back for a final confirmation in a future TCC.

# Analysis of proton lifetime studies –  B. Salvachua Ferrando- slides

B. Salvachua gave an overview of the 2018 statistics of losses along the LHC cycle, as done for previous years in several reviews, meetings and Evian workshops. A typical fill is being presented, where the loss patterns are typically similar for both B1 and B2, with B1 generally showing higher losses than B2, similar to previous years. The overview of the 2108 runs includes all fills with high current (higher then 4e13p) and stored energy just below 300 MJ. The intensity per bunch was reduced to around 1.1e11 due to 16L2 events.

For the ramp and flat top, around 240 fills were analyzed, and 13 dumps occurred, mostly due to 16L2. The transmission was very good with only 0.5% lost for both beams. For the squeeze, around 220 fills were analyzed from which 7 were dumped. The transmission was again better than 99 % and stabilized towards the end of the year in particular for B1. The same is true also for adjust. The beam lifetime is estimated through the IR7 collimator BLMs. For ramp and flat-top, the overall average lifetime is around or above 200-250 h, whereas the average minimum lifetime is 3.4 h, always well above the critical lifetime limit of 0.2 h. Following a question of L. Rossi, B. Salvachua clarifies that the statistics contains high intensity fills through the year (with total beam currents of ~0.5 A). Following the questions of R. Tomas, B. Salvachua points out that the high intensity MD fills are also included in the statistics.

During the squeeze, the average (minimum) lifetime is very large. The maximum power losses are 100 kW, whereas the average is 10 kW for B1 and 2 kW for B2. During adjust, there are just a few cases with lifetime dips below 1 h, with the minimum typically occurring when beams are collapsing. The cumulated duration of the cases with lifetime lower than 1 h is only a few seconds for all parts of the fill.

Discussion

M. Zerlauth points out that this excellent machine performance is also due to the experience gained after several years of operation, and that we might observe again worse lifetimes especially in the beginning of a new run period. L. Rossi agrees that HL-LHC corresponds to a step in both, energy and intensity. Y. Papaphilippou stresses that Run III will be the period where the gap between present LHC and HL-LHC will be filled and running experience will be gained under high intensity conditions at 7 TeV. R. Tomas reminds that there were indeed a few occasions with high intensity bunches during 2018 MDs and could be also used as a first indication. L. Rossi concludes that from these observations, there is some confidence to install 11 T to the chosen position. He further asks for a clarification about the duration of the lifetime drops below 1 h and B. Salvachua shows that there is only one case in adjust that lasted 15 s, all others are typically lasting a few seconds maximum. S. Redaelli further clarifies that the corresponding losses in beam power were very low. Following a question of D. Wollmann, B. Salvachua answers that the resolution of the lifetime measurements is indeed 1 s. R. Garcia Alia asks if there are some statistics for stable beams, and B. Salvachua answered that they can be indeed evaluated. Y. Papaphilippou adds that it could be interesting to have the lifetime at the first part of the fill at collision, where both beams are suffering from additional losses beyond standard burn-off. O. Brüning reiterates that in summary moving the 11 T dipole from cell 8 to cell 9 is a good approach, mitigating the losses for ions and considering that the increased proton losses in cell 9 are unlikely to be high enough to reach the quench limit.

R2E levels for installation of 11T in cell 9, C. Bahamonde Castro -  slides

C. Bahamonde explains the motivation from R2E to investigate the need to relocate racks, which are placed mostly underneath the cryostat, for both TCLD locations. FLUKA simulations are undertaken to estimate dose and high-energy fluence and compare these to the limit of 200 Gy assumed over all the HL-LHC period.

The considerations for the simulation studies are briefly reviewed. Only protons are considered which are the dominating case for R2E. The current (but not the new) rack locations are included in the simulations. A factor of 3 is considered in all plots due to the consistently observed underestimation of FLUKA simulations as compared to actual BLM measurements in the DS. The dose is estimated for 1e17 protons lost during the whole HL-LHC period, whereas the fluence is estimated for 1e16p/y lost in the collimation system.

L. Rossi asks if the factor of 3 discrepancy is observed only for the DS area or if it is similar to the IPs close to the tertiaries. F. Cerrutti answers that indeed this discrepancy is found only at the collimator DS, whereas the TCTs deal with experimental background which can be well represented by simulations. Following a comment of L. Rossi that the factor of 3 is quite big, S. Redaelli mentions that tracking simulations do not include imperfections and that a factor of 2 could be accounted just for this, or due to a tilt in the primaries. A. Lechner stresses that this should not be considered as a safety margin but like a correction factor. L. Rossi points out that it is indeed remarkable that the studies of N. Mokhov at the design phase of the LHC predicted quite accurately the quench limit of the magnets.

The dose below the cryostats left of IP7 with TCLD in cell 8 are presented. Three hot spots are found, around cell 8, 9 and 11. For the TCLD located in cell 9, the hot spot is indeed the TCLD itself. Overall, moving the TCLD to cell 9 just moves the exclusion zone and eliminates exclusion zones in cell 8 and 11. The situation is almost symmetric for R7 and L7. The diodes are located in the left side of the collimator, and for both cases for the TCLD location, the dose is quite small (< 1kGy) and the fluence is well within margin.

Fluence plots for both cases show that it is comfortably below 1e13 HEH/cm2 per year, outside exclusion zones. Regarding the peak power density in the SC coils, the cell 9 position is better for ions but there is higher density for the 11T coils, with 49 mW/cm3 for 0.2 h lifetime. TCLD gap opening does not reduce the load to the most exposed 11T but only worsens the situation at cell 11. O. Brüning concludes that hybrid schemes are indeed not a good option. L. Bottura wonders why the 50 mW/cm3 are almost unchanged whatever the collimator gap is, while with no TCLD the dose on the dipole is only 20 mW/cm3. C. Bahamonde believes that this is due to the difference between curved over straight dipoles. F. Cerruti stresses that the number of proton losses is increasing. L. Bottura stresses that the sagitta is only 2 mm and it is strange that this may affect that much the deposition. O. Brüning wonders if there are two distinct dose peaks and D. Wollmann answers that this is true for cell 11. S. Redaelli stresses that one should take into account as well that the aperture cannot be controlled to infinite precision. C. Bahamonde clarifies that the peak power density of the most exposed dipole is quoted in the table. L. Bottura reiterates that a sensitivity to the level of a few mm within a 56mm beam screen is quite strange. F. Cerruti points out that this sensitivity was already shown in the past. R. De Maria explains that the dipole is installed in a way that could affect the energy deposition. He adds that, as discussed during the alignment working group, the loss of aperture is not symmetric, i.e. aperture is lost on the outside and gained on the inside side. The debris with lower momentum are bent by the dipole field and are directed towards the inside for the aperture. This is of course not the case for neutrons. R. Bruce adds that for the 11 T, the losses are concentrated at the end of the magnet while for the curved standard dipole are distributed along the length.

# WP9 statement on overall heat load – R. Van Weelderen -  slides

R. Van Weelderen gives a brief update on heat load on the TCLD-11 T assembly. Improved analytical treatment of heat transfer data was used and the time dependence on luminosity was included. In addition, the impact on installation location (MBB.8 or MBA.9) was evaluated. The total deposited power on the 11 T coils for the two positions is presented for the two cases of 0.2 and 1 h lifetime. The heat can be extracted through the Kapton outer layer of Nb3Sn coils. Heat loads corresponding to 1 h lifetime can be thus extracted but also for 0.2 h, the load can be handled based on SM18 ramp-rate data. Summing up all loads, the global heat extraction will improve when the TCLD is located in cell 9.  Regarding the total cryogenic HW behavior, and based on heat deposition as a function of time, the temperature excursion can be contained below Tλ in all cases. In summary, the local heat extraction of 11 T is not considered an issue and the global heat extraction is actually improved to the new location.

Based on an CFD assessment, SM18 ramp-rate data and Cryolab measurements, we are confident that the 11T dipole coil can sustain power densities in excess of 100 mW/cm3, compatible with either location. O. Brüning and L. Rossi thank all the speakers and colleagues who made all this work and propose to endorse the move of the TCLD to cell 9. The probability to have 0.2 h lifetime is really low and there is no risk of quench. S. Redaelli adds that in that case a dump via the BLM signals will be triggered.

DECISION: The TCC endorses the recommendation to change the location for the DS collimators (TCLD) and 11 T from cell 8 (MBB.8) to cell 9 (MBA.9).

# Update of 11 T ECR with input for cell 9 installation, including considerations on corrector package and implications on cryostat (release valves) – D. Schoerling -  ECR, comments/answers

D. Schoerling presents the comments, answers and recent changed for the 11T ECR. Regarding the comment of J. Wenninger on the potential risk on operational availability during LHC run 3 operation due to the 11 T installation, an answer was given providing information about the current study and the change of the location, thereby mitigating potential risks and problems. S. Chemli rejected the ECR due to the absence of drawings. Thereby, missing drawings have been added and updated. Following a question by S. Redaelli, L. Rossi answered that the TCLD ECRs which have been already circulated have to be updated and replaced by a new version reflecting the new position. D. Schoerling proceeded to the description of the reason of change and modification in order to reflect the hybrid solution studies. The document has to be further checked by rMPP and MP3 before being presented to LMC for approval. All comments are being discussed off-line. Regarding integration, two to three months will be needed to get all the necessary information. M. Modena clarified that indeed work has started with respect to the information provided by the ECR. For checking the proper installation of TCLD in the new slot and location with respect to electronics and cabling, scans in the tunnel are necessary and are expected for the end of the year. Then, time is needed to re-do the integration study. For these reasons, the integration study is expected to be finalised on February. L. Rossi stresses that the ECR has to be presented to the LMC asap while the formal approval can come when all studies are finished. A statement from the LMC is needed soon. L. Bottura adds that the ECR should be recirculated for comments.

Regarding the cryostat, L. Bottura mentions that care should be taken, as works have already started and holes for safety valves have already being drilled as was required for the initial installation slot in cell 8 (by chance cell 9 requires less release valves). Regarding the flow-meters, R. Van Weelderen confirms that they are now included in the additional cryo instrumentation. The rack positions have to be changed and checked. D. Wollmann asks that the parts which are under update are clearly marked. With this information and the TCC approval, O. Brüning will inform the LMC of the endorsed change of installation location during its next meeting.

DECISION: The TCC endorses the circulation of the updated ECR on the “Installation of the 11 T Dipole Full Assembly in LHC P7” reflecting the new position of the 11 T for comments in EDMS. A final version including the integration details will be issues by February 2019 and circulated for approval in EDMS.

Results of the BBLR compensation experiments in the LHC – G. Sterbini, slides

G. Sterbini gives an overview of the 2018 BBLR wire compensation experiments in the LHC, including simulation results for the HL-LHC and some future plans. The timeline of the four MDs is presented, corresponding to measurements with round optics at 30 cm, reduced crossing angle down to 130 μrad and finally tests with trains using a new wire cabling. He recalled that four wires are installed in B2, around IP1 and IP5, embedded in collimator jaws, with an effective length of 1 m. The wire has a diameter of 2.5 mm and can carry up to 350 A. For 1.15e11 p/bunch, 103 A are needed, for the ideal configuration (i.e. an installation location at ~159 m and a transverse distance of the wire to the beam of 5.7 mm). These optimal conditions cannot be met by the present hardware. Answering a question by L. Rossi, G. Sterbini clarifies that in the present LHC, the optimal location is exactly on D2, but for HL-LHC it is between Q4 and Q5.

The minimal distance of the TCTs at their nominal position put the wire far, as compared to the 5.7 mm optimal distance. In order to position the wire so close to the beam (with jaws at 5.5 σcol), set-up beam conditions should be employed. In this sub-optimal configuration, we can only address the compensation of the most important resonances, which are the fourth order ones (leading order amplitude tune-spread), that cannot be compensated by the H/V crossing.

The best observable during the experiment are the losses normalized to the luminosity (a kind of “effective” cross section). The first tests in MD1 were jeopardized by beam instabilities of the strong beam (beam 1) and only successful to check alignment and feedforwards on orbit and tune. During MD2, the compensation of the BBLR with 4 wires was proven. Clear reduction of losses was observed while switching on the wires, while losses increased when the wires were switched off for the bunch suffering long-range encounters, while the bunch with only head-on beam-beam was mostly unaffected. It was also proven that the two wires in IR5 give already a very good compensation, while the ones in IR1 are underperforming due to their sub-optimum positioning with respect to optics. During MD3, the compensation of the BBLRs was successfully demonstrated, while reducing the crossing angle down to 130 μrad. A typical modulation of lifetime was always observed while switching on and off the wires for all crossing angle steps.

During MD4, the wires were tested in a configuration compatible with present operation of the LHC, i.e. the TCTs in their operational position. In that case, only the wires at the right side are efficient (the ones on the left are too far away from the beam). In order to enhance the compensation, it was proposed and accepted by the LMC to modify the cabling of the wires by powering the ones located on both jaws in series. This enables doubling the even multipoles and cancelling the odd components, thereby enabling the compensation of 4th order resonances. The experimental results proved once again the compensation of the BBLR effect even in that configuration, as shown by the decrease of the losses from the BLM signals and also for reduced crossing angles.

Simulation results for HL-LHC proved the wire compensation efficiency in flat optics. Indeed, the wires enable the increase of the DA from 5 to 6 σ for an optimized working point area. Several simulations were also performed for round optics. The gain in the DA can be as much as 2 σ, for a large range of currents and transverse distances. Y. Papaphilippou comments that this numerical analysis confirms the obtained experimental results, for which even in a sub-optimal configuration with respect to the transverse distance (i.e. the wire at a location beyond the TCTs), the DA can be increased and thereby the crossing angle can be further reduced, enhancing performance. Regarding the working point scan, the area of 6 σ DA is largely increased, allowing to keep the same working point throughout the levelling process. First iterations on possible hardware for operational use in HL-LHC has been undertaken by A. Bertarelli, with a vacuum chamber equipped with a molybdenum or copper wire, isolated with a ceramic insulator. An attempt to integrate the wire collimators to LHC operation during Run III, compatible with the technical constraints is currently being followed up. Thereby the possibility to move two wire collimators (the left ones of IP1 and 5) to B1 is being proposed.

L. Rossi asks if there is any show-stopper for this. Y. Papaphilippou mentions that A. Rossi had discussions with the responsible equipment groups and no major show-stopper was yet identified for this change. S. Redaelli mentioned that this is also true from collimation side. Care should be only taken from RP, as one of these points is quite hot. O. Brüning and L. Rossi congratulate G. Sterbini and the wire compensation team for these excellent results and the clear experimental and numerical demonstration of the potential gain with the wires. The TCC also endorses the proposal of moving the two wire collimators to beam 1, during LS2, to be further approved by the LMC.

DECISION: The TCC endorses the proposal to move two wire collimators from beam 2 to beam 1, during LS2.

AOB

L. Rossi wanted to share the good news that Russia is about to make a commitment for in kind contribution to the project which opens the way to finance certain options such as the hollow e-lens, crystal or the wires.

The next TCC meeting will take place on the 13th of December 2018.