=0r
0 = 4∙10-7 Vs/(Am)
=0r
0 = 8.854∙10-12 As/(Vm)
c0 = 2.998∙108 m/s

JUAS 2020 – Tutorial 1

F. Caspers, M. Bozzolan, M. Wendt

# 1.) Transmission-lines

Given is a coaxial transmission-line with an inner diameter of the outer conductor of 100 mm, the dielectric is air (so-called “air-line”).

# Questions:

- What is the outer diameter of the inner conductor to achieve a characteristic impedance of 50 ?

Z0=μrεr60ΩlnRr=60ΩlnDd

d=Dexp5060=D2.3=43.46 mm

see script page 10

- With which velocity is a wave travelling in this line?

v=c=2.998∙108 ms

- Specify the capacitance and inductance per meter length of this transmission-line?

C'=1vZ0=66.71 pFm

L'=Z0v=166.78 nHm

see script page 10

- Instead of an air dielectric this transmission line is now homogeneously filled with Teflon 
(εr=2). Determine the phase velocity, characteristic impedance, as well as capacitance and inductance per meter length?

v=cεrμr=2.12∙108 ms

Z0=60ΩεrlnDd=35.36 Ω

C'=1vZ0=133.43 pFm

L'=Z0v=166.78 nHm

# 2.) Impedances in the complex plane (2)

The impedance of a resonant circuit is a function of frequency. For a given resonator the impedance was measured at 7 different frequencies, f1…f7. The result is shown in the complex Z-plane:

|  | f1 | f2 | f3 | f4 | f5 | f6 | f7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f / MHz | 105.11 | 105.05 | 104.94 | 105.29 | 105.35 | 105.46 | 105.20 |
| Z / kΩ | 200.0 ej30° | 162.6 ej45° | 115.0 ej60° | 200.0 e-j30° | 162.6 e-j45° | 115.0 e-j60° | 230.0 ej0° |

# Questions:

- Determine the resonant frequency.

fres=f7=105.20 MHz

- Determine the 3-dB bandwidth (BW) of this resonator.
(Hint: The bandwidth of a resonator is defined as the frequency difference 
between the upper and lower 3-dB frequency points.)

BW=f5-f2=300 kHz

In order to evaluate the properties of a resonator, 
it is common to model it as equivalent circuit with lumped RLC elements.

- Sketch the equivalent circuit for the measured resonator.

C                 R            L

- Determine R.

R=230 kΩ (at fres=f7)

- Draw the locus of admittance of this circuit in the Y-plane, 
and indicate lower and upper 3-dB points.

Straight line parallel to the Im{Y} axes, crossing Re{Y} at 1/230 mS. The 3-dB points are located on the locus as points crossing with lines from the origin under 450.

see script page 58

- Determine the Q-value, as well as L and C for this circuit.

Q=fresBW=350.667

L=R2πfresQ=992 nH

C=12πfres2L=2.31 pF

# 3.) Multiple choice questions

1. How will the resonant frequency fres of the E010 (TM010) mode of a pill box cavity change if height of the cavity is doubled? (check 1)

o The fres  decreases by a factor 2.

o The fres decreases by a factor 2.

o The fres increases by a factor 2.

o The fres increases by a factor 2.

o The fres will not change.

2. A critically coupled aluminum pill-box cavity is driven by an RF generator. The same pill-box cavity is now made out of copper, again with the generator operating at critical coupling, such that the gap voltage remains the same. σAl=3.8∙107Sm, σCu=5.8∙107Sm. What happens with the dissipated power in the cavity? (check 1)

o The power dissipation decreases

o The power dissipation increases

o The power dissipation will not change

3. Calculate the thickness of a copper wall of 5 times the penetrations depth for 50 Hz signals. σCu=5.8∙107Sm, μ=μ0μr with μ0=4π∙10-7 VsAm (check 1)

o 46.7 mm

o 4.67 mm

o 0.46 mm

o 0.046 mm

4. A rectangular waveguide has a width (long side!) of a = 10 cm. (check 2)

o The mode TE10 or H10 has a cutoff frequency of 3 GHz.

o The mode TE10 or H10 has a cutoff frequency of 1.5 GHz.

o The electric field is parallel to the side with the larger dimension.

o The electric field is orthogonal to the side with the larger dimension.

5. Which mode is the fundamental mode (lowest cut-off frequency) in a cylindrical waveguide of circular cross-section without inner conductor? (check 1)

o TE

o TEM

o TM

6. Adding capacitive loading to a cavity (check 1)

o lowers the resonance frequency

o does not affect the resonance frequency

o increases the resonance frequency

7. When you cover the antenna of your mobile with your hand, the attenuation caused is in the order of 20 dB. Human tissue is a rather good absorber, so you can neglect reflections for this calculation. How many percent of the mobile’s output power stay in the hand? (check 1)

o 9 %

o 99 %

o 99.9 %

o 99.99 %

# 4.) Impedances in the complex plane

# Questions:

- Plot the following impedances in the Z-plane, use the plot axes on the next page:

| Z = (3 + 4 j) Ω | ¦Z¦ = 2, arg(Z) = π/4 | Z = short circuit |
| --- | --- | --- |
| Z = 2 Ω | ¦Z¦ = 1, arg(Z) = -π/2 | Y = Z-1 = (0.16 + 0.12j) Ω-1 |
| Z = (1 – 4 j ) Ω | ¦Z¦ = 5, arg(Z) = 53° |  |

- Qualitatively, how would an inductor look like, plotted from DC to some arbitrary frequency, in the Z-plane? 					Hint:  ZL=jωL

- How would a capacitor look like?		Hint:  ZC=1/(jωC)

- The input impedance of a RLC circuit has been plotted in the Z-plane (blue circle). 
Mark the points in the diagram describing:

  - Impedance at the resonant frequency

  - DC impedance

  - 3-dB bandwidth

  - Impedance at f → ∞

# 5.) Waves of a transmission line Z = 50 Ω

Problem: Convert the circuit-based formats, voltage V and current I into the
                   equivalent wave-based formats, forward wave a and backward wave b
                   and vice versa using the relations:

| a= V+I Z2 | V= a+b |
| --- | --- |
| b= V-I Z2 | I Z= a-b |

# Questions:

- In a 50 Ω system, a directional coupler measured forward and reflected waves a and b at a certain plane as: a = 100 0° and b = 60 45°.

  - Calculate the corresponding voltage V and current I

V=a+b=100+42.43+j42.43 V=142.43+j42.43V=148.61 Vej16.59°

I=a-bZ=100-42.43-j42.43V50 Ω=1.15-j0.849 A=1.43 Ae-j36.39°

  - Sketch the “phasors” of V, I Z, a and b.

- At some plane in the 50 Ω system, a voltage of V = 100 0° V and a current of I = 1.0 -45° A are measured.

  - Calculate the corresponding forward and backward waves a and b.

a=V+IZ2=100 V+0.7-j0.7A 50 Ω2=67.68-j17.68 =69.95 Ve-j14.6°

b=V-IZ2=100 V-0.7-j0.7A 50 Ω2=32.32-j17.68 =36.84 Vej28.68°

  - Sketch the “phasors” of V, I Z, a and b.

# 6.) Scaling laws

A cavity shall be scaled from existing designs for a frequency fx = 318.32 MHz and Cx = 10 pF.
There are three test designs, with the following parameters:

| Cavity | fres / MHz | C / pF | Q | Diameter / mm |
| --- | --- | --- | --- | --- |
| A | 100 | 7.957 | 10000 | 600 |
| B | 500 | 3.18 | 5000 | 200 |
| C | 3000 | 1.061 | 2000 | 25 |

# Questions:

- Which cavity is suitable as reference design?

For the optimal candidate design, the parameter r/Q has to agree with the one calculated for the actual cavity. Using:

rQ=1ωC

we find r/Q=50 for the scaled cavity, 
and r/Q=200cavity A, 100 cavity B, 50 (cavityC) for the three design candidates. Therefore, cavity C is chosen for scaling.

- Calculate the diameter of the new design.

All physical dimensions of the cavity scale proportional to λ∝1/f.

Dx=Dffx=0.236 m

- Calculate the expected Q factor for the new design, provided it will be built out of the same material as the reference design.

Using the 3rd scaling law:

Qδλ=const.

we find Q∝λ/δ, with the skin depth δ decreasing linear with the frequency f:

δ=1πfμσ∝1f∝λ

This results in:

Q∝λ∝1f   ⇒   Qx=Qffx=6140

# 7.) Thermal expansion and scaling laws

An accelerator cavity heats up under high RF power load.
The cavity used is constructed from a material having a:

| thermal expansion coefficient: | l/l = 20e-6/0C (per degree Centigrade) |
| --- | --- |
| thermal resistivity coefficient: | / = 4e-3/0C (per degree Centigrade) |

At room temperature the cavities resonance frequency is f1 = 100 MHz and has a 3-dB bandwidth of BW1 = 100 kHz.
Under RF power the cavity temperature increases by 100 0C (subscripts 2 apply).

# Questions:

Determine

- the ratio 2/1

The wavelength λ scales proportional with the cavity dimension d, because λ is inverse proportional to f. Therefore:

λ2λ1=d2d1=d11+ΔllΔTd1=1+ΔllΔT=1.002

- the ratio L2/L1

The characteristic impedance L/C=R/Q stays constant, therefore:

L2L1=R/Qω2=ω1R/Q=ω1ω2=λ2λ1=1.002

- the ratio C2/C1

Similar to the previous we get:

C2C1=1R/Qω2=R/Qω11=ω1ω2=λ2λ1=1.002

- the ratio Q2/Q1 (hint: the skin depth  is proportional to ρ/f

The quality factor Q∝λ0δ, and the skin depth follows δ∝ρf, therefore:

Q2Q1=const λ2δ2=δ1const λ1=λ2λ1δ1δ2=λ2λ1ρ1ρ2f2f1=λ2λ1ρ1ρ2λ1λ2=ρ1ρ2λ2λ1=1.1844

with: ρ1ρ2=1+ΔρρΔT=1.4

- the resonance frequency f2 under load

f2f1=λ1λ2   ⇒   f2=f1λ2λ1=99.8 MHz

- and the 3-dB bandwidth BW2 of the resonance under load

Δf1=f1Q1, Δf2=f2Q2   ⇒   Δf2Δf1=f1Q2Q1f1

Δf2=f2f11Q2Q1Δf1=84.26 kHz