KM3NeT Pressure Qualification Procedure for the Hydrophone

KM3NeT_QUAL_2021_001_v1

Dorothea Samtleben, Miles Lindsay Clarck, Miquel Ardiz Ramirez, Vincent Bertin, Alin Ilioni, Marco Anghinolfi, Laurence Caillat + S. Henry, G. Androulakis

CPPM

Abstract This document describes the qualification tests procedure to be applied to the ORCA and ARCA Hydrophone for the KM3NeT project. Recipients The KM3NeT PSC, The KM3Net Calibration working group

Document Status

| Revision | Date | Comment | Reviewed by | Approved by |
| --- | --- | --- | --- | --- |
|  |  |  | XX | YY |

Revision History

| Revision | Date | Description |
| --- | --- | --- |
| V1 | 19/01/2021 | Distributed to the working group |

Table of Contents

2	Documentation	3

2.1	Abbreviations	3

2.2	Reference Documents	3

3	Introduction	3

4	Quality and organisational issues	3

4.1.1	Status of NCR-DCR-Waiver of the item under test	3

4.1.2	List of people committed	4

4.1.3	Test report	4

5	Definition of the qualification tests	4

5.1	Pressure test	4

5.1.1	Objectives and expected results	4

5.1.2	Material configuration	4

5.1.3	Test and monitoring of the material	6

5.1.4	Pass/fail criteria	6

5.1.5	Test procedure	6

5.1.6	Tests organization	7

5.2	Vibration and shock tests	7

5.2.1	Vibrations	7

5.2.2	Shocks	8

5.3	Climatic tests	9

5.4	Thermal shock	9

# Documentation

## Abbreviations

| Abbreviation | Description |
| --- | --- |
| UUT | Unit Under Test |
| LBL | Long Base Line |

## Reference Documents

| Abbreviation | Title | Reference |
| --- | --- | --- |
|  | KM3NeT TDR | KM3NeT_DS_TDR |
|  | KM3NeT CRD | KM3NeT_DS_CDR |
|  | KM3NeT Qualification Plan | KM3NeT_QUAL_2018_001 |
|  | KM3NeT Qualification Procedures | KM3NeT_QUAL_2018_002 |
|  | MAB100-T Acoustic Beacons Technical Description | MSM doc |
|  | Template of qualification_test_report | KM3NeT_QUAL_2020_001 |

# Introduction

This document describes the qualification stress tests foreseen on the ORCA LBL emitter from the company MSM that will be used both in the Calibration Base and in the DUs.  It will be the support for the Test Readiness Review (TRR) that should be held before to start the qualification processes.

The qualification is divided in 2 parts: the pressure test performed in August 2020 and the stress tests performed in beginning 2021 (date to be confirmed)

Remark: the ORCA and ARCA product are the same except for the connector which is a Seacon for ORCA and a Gisma for ARCA

The UPI of the UUT is:

# Quality and organisational issues

### Status of NCR-DCR-Waiver of the item under test

To be filled by the quality group

### List of people committed

Vincent Bertin  as responsible of the product

Laurence Caillat as LQS of CPPM

Sylvain Henry as responsible of the qualification

### Test report

After the test described in this documentation, a test report will be elaborated using the template in RD6

# Definition of the qualification tests

## Pressure test

### Objectives and expected results

The goal is to check the service pressure of a container from the point of view of the mechanical resistance and sealing.  The expected result is no degradation of the functional parameter, no visual degradation of the UUT and no alteration of the safety

### Material configuration

#### Unit under test (UUT)

- The UUT test is the hydro s/n 105

#### Mechanical support equipment

The UUT doesn’t need any mechanical support for this test. It will be put directly in the chamber

The only precaution to be taken is the mount the connector with a blind connector to stand the pressure

### Test and monitoring of the material

Investigation: visual and meter measurement of the envelope diameter (as requested),

Result: no change

### Pass/fail criteria

### Test procedure

Pt : Test pressure

PS : Service pressure (350 bars for the ORCA case)

|  | Ramping speed | Test pressure | Temperature | Duration | Number of cycles |
| --- | --- | --- | --- | --- | --- |
| Qualification tests | 12 bar/min | Pt = PS x 1.14 Pt = 400bars | Room Temperature | 1h | 1 |
|  | Visual inspection in between |  |  |  |  |
|  | 12 bar/min | Pt = PS Pt = 350 bars | Room Temperature | 48h | 1 |

### Tests organization

The test has been dine at COLMAR in August 2020

## Vibration and shock tests

### Vibrations

The goal is to check the capability of the material to stand a vibratory environment (due to transportation or handling) without suffering unacceptable of performance or structure degradations. If possible, the vibrations tests must be performed before any others stress tests.

The devices should be mounted as in normal service including shock or vibrations dampers if any.

#### Procedure

The Table 1gives the test parameters:

|  | Frequency range | Amplitude and acceleration | Sweep speed | Duration | Number of cycles |
| --- | --- | --- | --- | --- | --- |
| Critical frequencies search(a) | 5 Hz to 25 Hz 25 Hz to 150 Hz | amp. = ±1 mm acc.= 25 m/s2 | 1 oct/min |  | 1 |
| Frequency sweep (b) | 5 Hz to 25 Hz 25 Hz to 150 Hz | amp. = ±1 mm acc.= 25 m/s2 | 1 oct/min | 1h |  |
| Fixed frequency tests (c) | 5 Hz to 25 Hz 25 Hz to 150 Hz | amp. = ±1 mm acc.= 25 m/s2 |  | 1h |  |

Table 1: vibration test procedure

- Step (a): search for possible resonance frequencies

- Step (b): If no resonance frequency detected

- Step (c): On the 3 main resonance frequencies (from 5 to 25Hz → amp.= ±1mm and from 25 to 150Hz → acc. = 2.5G)

- Remark: a resonance can be defined by having an acceleration response twice the excitation

#### Mechanical support equipment

As following the life cycle of the material (transportation in the dedicated package and mounting on the anchor at the last moment), the material will be tested in its transport package (as delivered by the provider)

#### Test and monitoring of the material

The UUT could not be tested in a simple way, a test bench with a DAQ system is needed

So it is proposed to test the UUT at CPPM before the set of stress tests and tested again at CPPM after the set of stress tests

#### Pass/fail criteria

TBD

Moreover the visual inspection should no reveal any damage in the container or the connector

### Shocks

Shock tests are performed to provide a degree of confidence that materiel can physically and functionally withstand the relatively infrequent, non-repetitive shocks encountered in handling, transportation, and service environments. In addition, shock test could help to determine the materiel's fragility level, in order that packaging may be designed to protect the materiel's physical and functional integrity

The goal is to check the capability of the material to stand mechanical shocks of 2 types:

- Hard shocks: strong but rare

- Shake: light but repeated

#### Procedure

The proposed procedure is described in the following table. The shape of the signal is half-sine and the type of the shocks are pulsed (initial speed is zero). The shocks must be applied in all three axes and in both the positive and negative directions.

|  | Number of shocks | Number of axis | Description |
| --- | --- | --- | --- |
| “Hard” shocks | 3 | 6 (±X, ±Y, ±Z) | acc.= 10 G duration=11ms Wave form : half sine |
| “Soft” shocks | 100 | 6 (±X, ±Y, ±Z) | acc.= 5G duration=30ms Wave form : half sine |

Table 2: mechanical shocks test procedure

#### Mechanical support equipment

As following the life cycle of the material (transportation in the dedicated package and mounting on the anchor at the last moment), the material will be tested in its transport package (as delivered by the provider)

#### Test and monitoring of the material

The UUT could not be tested in a simple way, a test bench with a DAQ system is needed

So it is proposed to test the UUT at CPPM before the set of stress tests and tested again at CPPM after the set of stress tests

#### Pass/fail criteria

TBD

Moreover the visual inspection should no reveal any damage in the container or the connector

## Climatic tests

The goal is to test the capability of the material to work after an exposure to high temperature and humidity levels during the storage or transit phases (it is considered for the KM3NeT situation that the stress induce by high levels temperature and humidity is not critical during the exploitation of the detector devices).

#### Procedure

|  | Temperature & HR | Duration |
| --- | --- | --- |
| Ramping up | 20°C to 70°C HR not specified | 2h |
| Stand | 70°C 93% HR | 96h |
| Ramping down | 70°C to 20°C HR not specified | 2h |

Table 3: climatic test procedure

#### Mechanical support equipment

The UUT doesn’t need any mechanical support for this test. It will be put directly in the chamber

The only precaution to be taken is the mount the connector with a blind connector to protect the UUT

#### Test and monitoring of the material

The UUT could not be tested in a simple way, a test bench with a DAQ system is needed

So it is proposed to test the UUT at CPPM before the set of stress tests and tested again at CPPM after the set of stress tests

#### Pass/fail criteria

TBC

Moreover the visual inspection should no reveal any damage in the container or the connector

## Thermal shock

The purpose of this test is to determine the resistance of the part to sudden exposure to extreme changes in temperature. For KM3NeT the case is to detect the possible effects of immersion in cold water after long sun exposure (especially of a metallic vessel deck). It must make it possible, in particular, to detect a loss of tightness related to a design problem, mounting or different expansion phenomena between materials constituting the equipment.

The immersion depth should be around 20cm

#### Procedure

|  | Temperature | Duration |
| --- | --- | --- |
| Warm phase | 50°C | 2h |
| Cold phase | 10°C | 15min |

Table 10: thermal shock procedure

#### Mechanical support equipment

The UUT doesn’t need any mechanical support for this test. It will be put directly in the chamber

The only precaution to be taken is the mount the connector with a blind connector to protect the UUT

#### Test and monitoring of the material

The UUT could not be tested in a simple way, a test bench with a DAQ system is needed

So it is proposed to test the UUT at CPPM before the set of stress tests and tested again at CPPM after the set of stress tests

#### Pass/fail criteria

TBD

Moreover the visual inspection should no reveal any damage in the container or the connector

## Power cycling (TBC)

In order to simulate long term field service (20 years for KM3NeT sub-sea equipment), the material should be submitted to a certain number of power cycles.

This number has to be calculated for each specific case with the necessity to be equal or superior to the number of power cycle expected during the life time of the equipment.

## Network disturbance (TBC)

In order to simulate a possible disturbance of the power network, the material must be specified and subsequently tested at the nominal voltage ± 15%

11